3GPP TSG-RAN4 Meeting #113	R4-2419751

# Orlando, USA, 18th – 22nd November, 2024

Source: 	Qualcomm Incorporated

Title: 	Logistical Issues of Two-Sided Model for AI/ML CSF

Agenda item:	7.17.2.2

Document for:	Discussion

# Introduction

In this paper, we are discussing logistical issues of two-sided model for AI/ML CSF.

# Discussion

## Prior Agreements

RAN4 agreed to the following to align simulation parameters among companies [1].

Configuration for alignment: - For the results with quantization, to use a sigmoid function in the encoder for normalization Sigmoid in the decoder is optional. Quantization of encoder output Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization

RAN4 agreed to the following during the last meeting [2].

## Training Data file format Single file for fixed scalar quantization(only dataset), TBD whether additional file is needed for variable/trainable quantization

## Clarification Regarding Quantization Codebook:

The agreement of [2] suggests that all companies should have a clear understanding of the fixed scalar quantization codebook. That’s why, RAN4 decided not to introduce any additional file for fixed scalar quantization.

The configuration alignment parameters of [1] suggest UE to use a sigmoid function in the encoder for normalization before quantization. The output of sigmoid function ranges from 0 to 1. It also suggests UE to use fixed uniform quantization with equal ranges. This implies that UE should use fixed scalar quantization codebook of [1/8 3/8 5/8 7/8]. However, the levels of 2 bit fixed scalar quantization codebook have not been explicitly captured yet. For clarity, it would be better for RAN4 to capture this 2 bit fixed scalar quantization codebook levels as an agreement.

Observation 1: RAN4 has previously agreed to use a sigmoid function in the encoder for normalization before quantization.

Observation 2: RAN4 has also agreed to fixed uniform quantization with equal ranges.

Observation 3: Observation 1 and 2 imply that RAN4 wants companies to use [1/8 3/8 5/8 7/8] as levels of fixed scalar quantization. The specific levels have not been captured in the spec.

Proposal 1: RAN4 explicitly specifies the following levels of 2-bit fixed scalar quantization codebook in the AI-ML model for studying feasibility of option 3: [1/8 3/8 5/8 7/8]

# Conclusions

Observation 1: RAN4 has previously agreed to use a sigmoid function in the encoder for normalization before quantization.

Observation 2: RAN4 has also agreed to fixed uniform quantization with equal ranges.

Observation 3: Observation 1 and 2 imply that RAN4 wants companies to use [1/8 3/8 5/8 7/8] as levels of fixed scalar quantization. The specific levels have not been captured in the spec.

Proposal 1: RAN4 captures the specific levels of 2-bit fixed scalar quantization codebook as: [1/8 3/8 5/8 7/8]

# References

[1] R4-2414447, “Alignment of model training and data set parameters for AI/ML based CSI compression”, 3GPP RAN WG4 #112.

[2] R4-2417201, “WF on Data Sharing Framework”, 3GPP RAN WG4 #112bis.