3GPP TSG-RAN4 Meeting #113	R4-2419753

# Orlando USA, 18th – 22nd November, 2024

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI/ML Beam Management

Agenda item:	7.17.4.1

Document for:	Discussion

# Introduction

This paper focuses on RRM core requirements for AI/ML based beam management.

RAN4 made the following agreements during the last meeting [7][8\

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed - Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point.

RAN1 made the following agreements in [5][6]:

- RAN1#117: Working Assumption - Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). - RAN1#118 agreement: For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list are associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

# Discussion

## 2.1 Selection of Beam Prediction Metrics

3GPP focused on the following prediction metrics in [3]

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics/KPIs are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The overhead/latency reduction should be considered for the requirements as the side condition.

RAN1 discussed this issue during the last meeting and made the following agreement [9]:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

RAN1’s agreed Alt 1 (without margin) simply translates to RAN4’s option 2 (beam prediction accuracy). On the other hand, RAN1’s agreed Alt 1 (with margin) simply translates to RAN4’s option 3 (beam prediction accuracy with X dB margin between measured RSRP of predicted top K beams and actual strongest beam).

RAN1 has not yet agreed to support RAN4’s option 1 (“L1-RSRP accuracy”) yet. Hence, RAN4 should postpone defining requirement for L1-RSRP accuracy until RAN1 reaches a conclusion.

Observation 1: RAN1’s agreed beam prediction metric without or with margin simply translates to option 2 and option 3 of RAN4’s beam prediction metric.

Observation 2: Support for option 1 of beam prediction metric is still TBD in RAN1.

Technically, RAN4 can define performance requirements for both option 2 and option 3. However, option 3 provides more flexibility than option 2 for RAN4 to define a metric. Option 2’s drawback lies in the fact that option 2 (top-1 (%) version of option 2) does not distinguish between following two scenarios:

- UE’s top-1 predicted beam is adjacent to, but not same as, the top-1 strongest beam

- UE’s top-1 predicted beam is far from the top-1 strongest beam.

This drawback of option 2 is addressed in option 3. UE can pass option 3 if the maximum RSRP among UE’s top-K predicted beams lies within X dB of the RSRP of the strongest beam. Hence, option 3 allows UE to pass a test if UE’s top-1 predicted beam is adjacent to the top-1 strongest beam.

RAN1 and RAN2 have not yet decided the maximum number of beams that UE can convey in its inferred reporting. The maximum number of measured beams that TE can report is also upper bounded by legacy spec. If TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of topK beams of setA lies within the range of UE’s measurement error, UE will not be able to pass option 2 focused test.

Observation 3: Option 2’s drawback lies in the fact that its pass/fail criteria don’t depend on whether the top predicted beams are adjacent to the strongest beams or not.

- If TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of more than topK beams of setA lies within the range of UE’s measurement error, UE will not be able to pass option 2 focused test.

- Note: The maximum value of K depends the maximum number of beams that UE can report for measurement and inference. The max value of K is decided by RAN1/RAN2 and RAN4 cannot control it.

Option 3 is a more refined version of option 2. It does not suffer from the constraints of option 2 and it retains the same usefulness of option 2. Even if more than topK beams of setA lies within the range of UE’s measurement error, UE will be able to pass option 3 tests as long as the measured RSRP of its predicted beam lies within X dB of the strongest beam. RAN4 can control the value of X dB and ensure that UE does not fail a test due to RAN4 allowed measurement error.

Observation 4: Option 3 does not suffer from the constraints of option 2 that got mentioned in observation 4. Even if more than topK beams of setA lies within the range of UE’s measurement error, UE will be able to pass option 3 tests as long as the measured RSRP of its predicted beam lies within X dB of the strongest beam.

- RAN4 can control the value of X dB and ensure that UE does not fail a test due to RAN4 allowed measurement error.

Proposal 1: Regarding down-selection of prediction metrics, RAN4 takes following steps:

- Option 3: RAN4 defines requirement for this option now.

- Option 2: RAN4 waits to investigate simulation results of different companies in RAN4 before defining requirements.

- Option 1: RAN4 waits until RAN1 agrees to support option 1.

## 2.2 TCI State Status of Predicted Tx Beam

During the last meeting, RAN4 discussed whether the TCI state of predicted Tx beam is known or unknown. RAN4 also discussed whether UE knows the RX beam paired with the predicted TX beam.

3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. For both temporal and spatial beam prediction, UE must know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam. If UE does not know its RX beam corresponding to the gNB Tx beam, UE’s predicted L1-RSRP RSRP will be drastically different from the actual L1-RSRP.

Observation 5: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

3GPP is also considering top-1 (%) beam or top-K (%) beam as a prediction metric. UE has to predict the top beams of setA based on what UE would see at its baseband. To compare between gNB beams, UE needs to have some idea of the Rx gain associated with them. Hence, UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 6: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Some companies mentioned that the AI-ML model considered in RAN1 uses (gNB Tx beam, measured RSRP) as inputs and the model does not explicitly indicate UE’s Rx beam. RAN1 assumed that UE knows its corresponding RX beam for the output of the AI-ML model, including predicted gNB Tx beam. That’s why, RAN1 did not explicitly include UE’s RX beam in the AI-ML model.

Observation 7: Some companies mentioned that the AI-ML model considered in RAN1 uses (gNB Tx beam, measured RSRP) as inputs and the model does not explicitly indicate UE’s Rx beam. RAN1 assumed that UE knows its corresponding RX beam for the output of the AI-ML model, including predicted gNB Tx beam.

Figure 1: Spatial Beam Prediction

The agreements in the introduction section shows that RAN1 has not yet fully clarified if UE can assume two sets of beams to be QCLed if they have the same association ID. However, RAN4 should enforce UE to “know” RX beam of predicted TX beam from setA only if it has measured the same setA beams during training. UE will not be able to “know” RX beam of predicted TX beam from setA if it has not measured the same setA beams during training.

Observation 8:  RAN1 has not yet fully clarified if UE can assume two sets of beams to be QCLed if they have the same association ID. RAN4 should enforce UE to “know” RX beam of predicted TX beam from setA only if it has measured the same setA beams during training.

RAN4 needs to further discuss whether the TCI state corresponding to predicted TX beam is known or unknown.

RAN4 spec differentiates between known and unknown condition for TCI state in the following way [7]:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

As above text shows, the SNR of the SSB associated with the TCI state needs to be >= -3 dB for TCI state to be known. These types of side conditions need to revisited to determine if the TCI state corresponding to the strongest beam is known or unknown.

Proposal 2: UE knows the RX beam paired with the predicted Tx beam from setA under following conditions:

  - UE has measured setB and setA beams during training.

  - UE has measured setB, but not setA, beams during inference.

  - FFS: Whether TCI state should be considered known.

  - FFS: Whether side conditions for known TCI state should be modified (relaxed) for this case.

## 2.3 Measurement Error and Beam Prediction Accuracy

### 2.3.1 Beam Prediction Accuracy without Measurement Error

We first show our system simulation results for 8 beam based setB and 64 beam based setA without any kind of measurement error. We will show simulation results for 32 beam based setA by the next meeting. We also displayed these results in [10]

#### Simulation Assumptions

we predict the best gNB beam ID in Set A based on the measured RSRPs of Set B. In simulations, we focus on the UMa scenario, and follow the agreed SLS simulation assumptions so far as summarized in Table 1. We assume there is no UE mobility. The dataset is generated from random UE location drops in a cell with three sectors. Spatial consistency procedure is performed based on 38.901 [2], to ensure the channel characteristics between neighbouring UEs are appropriately correlated. Key assumptions for the simulations are summarized in Table 1. The datasets used for training and testing correspond to different sets of UE locations. The gNB array has 32 antennas (64 per sector), 8 on azimuth and 4 on elevation. We assume 192 DFT beams in Set A in the cell, and 24 beams are down-selected for Set B (8 per sector). In Figure 2, we illustrate the beam pointing angles of Set A and Set B beams in a sector. We select the Set B in a way that the measured beams cover as much space in both elevation and azimuth directions as possible.

Table 1 Simulation assumptions

| Parameters | Value |
| --- | --- |
| Scenario | Uma, outdoor |
| Carrier frequency | 30 GHz |
| ISD | 200 m |
| BS antenna cfg | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| gNB codebook (Set A) | 16 (azimuth) by 4 (elevation) beams per sector, in total Set A has 192 beams per cell |
| gNB codebook (Set B) | 8 beams down-selected from Set A per sector, as shown in Figure 42 (or in Figure 43 for Set B1), in total set B has 24 beams per cell |
| gNB antenna gain | 8 dBi |
| BS Tx power | 30 dBm for 20 RB bandwidth |
| SCS | 120KHz |
| Car penetration loss | Included |
| UE mobility and rotation | Not assumed, only consider a single time shot |

For the AI/ML based approach, we apply a fully connected (FC) layer based NN to predict the best beam in Set A. We also consider the following 2 baseline approaches to predict the best beam ID, in addition to a NN-based AI/ML model:

- Baseline-1 (linear interpolation-based approach): For the non-measured beams in Set A, we estimate their RSRPs by performing a 2D linear interpolation (and extrapolation, when applicable) based on the measured RSRPs and the pointing angle of the beams. We select the top K predicted beams based on the interpolated RSRPs.

- Baseline-2 (empirical PMF-based approach): We derive an empirical probability distribution of the best beam ID in Set A, given the best beam ID in Set B, based on the training dataset. For testing, we select the top K predicted beams based on the derived empirical distribution.

Figure 2 Illustration of pointing angles of Set A and Set B beams in a sector of the cell.

#### Evaluation Results

We present the beam prediction performance for use case1 in Table 2. For Top-K beam selection accuracy, we follow the KPIs for beam prediction accuracy. Compared with the baseline approach, our results show that NN based AI/ML model provide a significant performance gain, in terms of both beam selection accuracy and L1 RSRP difference. The linear interpolation-based approach (Baseline-1) provides the worst performance, as Set B beams are sparsely sampled from Set A, and the RSRPs in general cannot be approximated by a linear fit.

Table 2 Evaluation results for BM-Case1 (Use case 1) without model generalization for DL Tx beam prediction at UE side (Results for Set B, Set B subset of Set A)

| Qualcomm |  |  |  |  |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams [beams/beam pairs] in Set A | 192 | 192 |
|  |  | Number of beams [beams/beam pairs] in Set B | 24 | 24 |
|  |  | Baseline scheme | Empirical PMF-based approach | linear interpolation-based approach |
| AI/ML model input/output |  | Model input | RSRPs of Set B beams | RSRPs of Set B beams |
|  |  | Model output | Best Tx beam ID | Best Tx beam ID |
| Data size |  | Training | 4000 | 4000 |
|  |  | Testing | 600 | 600 |
| AI/ML model |  | Model description | FC layer based NN | FC layer based NN |
|  |  | Model complexity | 4.6K parameters | 4.6K parameters |
|  |  | Computational complexity | 9.8K FLOPs | 9.8K FLOPs |
| Evaluation results [with AI/ML/baseline (Baseline-2)] | Beam prediction accuracy (%) | Top-1 (%) | 63.5 / 28.3 | 63.5 / 10.7 |
|  |  | Top-2/1 (%) | 80.0 / 46.1 | 80.0 / 16.3 |
|  |  | Top-5/1 (%) | 92.5 / 79.2 | 92.5 / 31.8 |
|  |  | 1-dB marginal accuracy (%) | 90.4 / 59.0 | 90.4 / 32.8 |
|  | L1-RSRP diff (otion 3 with K = 1) | Avg. L1-RSRP difference in dB | 0.36 / 1.27 | 0.36 / 4.22 |
|  | System performance | RS overhead Reduction (%) | 87.5 | 87.5 |

Observation 9: The evaluation results for 8 beam based setB and 64 beam based setA without any measurement error are shown in Table 2.

### 2.3.2 Baseband Error Modelling

We have generated baseband error results at -3 dB with both AWGN and the TDL-C channel assumptions of [11]. We show the distribution of baseband error results in Figure [3] and [4].

Figure 3: Baseband error distribution of SSB L1-RSRP at AWGN (-3 dB SNR, curve fitted with zero mean normal distribution and sigma = 0.6 dB)

Figure 4: Baseband error distribution of SSB L1-RSRP (TDL-C, -3 dB SNR, curve fitted with normal distribution and mean = -0.4 dB and sigma = 0.85 dB)

Observation 10: At -3 dB SNR, the distribution of measurement error can be fitted with following Gaussian distributions:

- AWGN: Gaussian with zero mean and sigma = 0.6 dB

- TDL-CD: Gaussian with mean = -0.4 dB and sigma = 0.6 dB

### 2.3.3 RF Error Modelling

[11] mentioned the following regarding RF error.

In addition, the impact of RF errors should also be considered. ±4.5dB RF error is considered. Following options can be used to generate RF errors. - Option 1: Uniform distribution - Option 2: Gaussian distribution - Other options are not precluded - Companies are requested to provide details of other options. Companies are encouraged to provide simulation results with and without considering RF errors.

RF error of a UE remains stems from the calibration error that UE experiences to generate its RX beamforming gain. The RF error is expected to be same within a module, but it varies across modules. A UE can have multiple modules.

During spatial beam prediction, UE predicts a beam of setA based on its measurement of setB. If UE is not experiencing rotation or mobility, during most practical scenarios, UE is expected to receive a set of measured beams in setB and their corresponding predicted beams in setA though the same module.

On the other hand, during temporal beam prediction, UE is expected to experience rotation and mobility between the time instances where it measures setB beams and the time instances where it predicts setA beams. UE cannot assume to receive setB and setA beams across different time instances with the same module.

Hence, RAN4 should discuss the scenario where UE can assume same RF error between time instances of setB measurement and those of setA prediction.

Observation 11: RF error of a UE is expected to be same within a module, but it varies across modules. A UE can have multiple modules.

Proposal 3: RAN4 should discuss the scenarios where UE can assume same RF error between time instances of setB measurement and those of setA prediction.

# Conclusions

Observation 1: RAN1’s agreed beam prediction metric without or with margin simply translates to option 2 and option 3 of RAN4’s beam prediction metric.

Observation 2: Support for option 1 of beam prediction metric is still TBD in RAN1.

Observation 3: Option 2’s drawback lies in the fact that its pass/fail criteria don’t depend on whether the top predicted beams are adjacent to the strongest beams or not.

- If TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of more than topK beams of setA lies within the range of UE’s measurement error, UE will not be able to pass option 2 focused test.

- Note: The maximum value of K depends the maximum number of beams that UE can report for measurement and inference. The max value of K is decided by RAN1/RAN2 and RAN4 cannot control it.

Observation 4: Option 3 does not suffer from the constraints of option 2 that got mentioned in observation 4. Even if more than topK beams of setA lies within the range of UE’s measurement error, UE will be able to pass option 3 tests as long as the measured RSRP of its predicted beam lies within X dB of the strongest beam.

- RAN4 can control the value of X dB and ensure that UE does not fail a test due to RAN4 allowed measurement error.

Observation 5: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

Observation 6: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 7: Some companies mentioned that the AI-ML model considered in RAN1 uses (gNB Tx beam, measured RSRP) as inputs and the model does not explicitly indicate UE’s Rx beam. RAN1 assumed that UE knows its corresponding RX beam for the output of the AI-ML model, including predicted gNB Tx beam.

Observation 8:  RAN1 has not yet fully clarified if UE can assume two sets of beams to be QCLed if they have the same association ID. RAN4 should enforce UE to “know” RX beam of predicted TX beam from setA only if it has measured the same setA beams during training.

Observation 9: The evaluation results for 8 beam based setB and 64 beam based setA without any measurement error are shown in Table 2.

Table 2 Evaluation results for BM-Case1 (Use case 1) without model generalization for DL Tx beam prediction at UE side (Results for Set B, Set B subset of Set A)

| Qualcomm |  |  |  |  |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams [beams/beam pairs] in Set A | 192 | 192 |
|  |  | Number of beams [beams/beam pairs] in Set B | 24 | 24 |
|  |  | Baseline scheme | Empirical PMF-based approach | linear interpolation-based approach |
| AI/ML model input/output |  | Model input | RSRPs of Set B beams | RSRPs of Set B beams |
|  |  | Model output | Best Tx beam ID | Best Tx beam ID |
| Data size |  | Training | 4000 | 4000 |
|  |  | Testing | 600 | 600 |
| AI/ML model |  | Model description | FC layer based NN | FC layer based NN |
|  |  | Model complexity | 4.6K parameters | 4.6K parameters |
|  |  | Computational complexity | 9.8K FLOPs | 9.8K FLOPs |
| Evaluation results [with AI/ML/baseline (Baseline-2)] | Beam prediction accuracy (%) | Top-1 (%) | 63.5 / 28.3 | 63.5 / 10.7 |
|  |  | Top-2/1 (%) | 80.0 / 46.1 | 80.0 / 16.3 |
|  |  | Top-5/1 (%) | 92.5 / 79.2 | 92.5 / 31.8 |
|  |  | 1-dB marginal accuracy (%) | 90.4 / 59.0 | 90.4 / 32.8 |
|  | L1-RSRP diff (otion 3 with K = 1) | Avg. L1-RSRP difference in dB | 0.36 / 1.27 | 0.36 / 4.22 |
|  | System performance | RS overhead Reduction (%) | 87.5 | 87.5 |

Observation 10: At -3 dB SNR, the distribution of measurement error can be fitted with following Gaussian distributions:

- AWGN: Gaussian with zero mean and sigma = 0.6 dB

- TDL-CD: Gaussian with mean = -0.4 dB and sigma = 0.6 dB

Observation 11: RF error of a UE is expected to be same within a module, but it varies across modules. A UE can have multiple modules.

Proposal 1: Regarding down-selection of prediction metrics, RAN4 takes following steps:

- Option 3: RAN4 defines requirement for this option now.

- Option 2: RAN4 waits to investigate simulation results of different companies in RAN4 before defining requirements.

- Option 1: RAN4 waits until RAN1 agrees to support option 1.

Proposal 2: UE knows the RX beam paired with the predicted Tx beam from setA under following conditions:

  - UE has measured setB and setA beams during training.

  - UE has measured setB, but not setA, beams during inference.

  - FFS: Whether TCI state should be considered known.

  - FFS: Whether side conditions for known TCI state should be modified (relaxed) for this case.

Proposal 3: RAN4 should discuss the scenarios where UE can assume same RF error between time instances of setB measurement and those of setA prediction.

# References

- R4-2414446, ‘AI/ML ad-hoc meeting minutes’, Moderator (Qualcomm)

- R4-2414323, ‘WF on AI/ML’, Qualcomm

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG RAN #105.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118.

- 38.133, Requirements for support of radio resource management, V 17.11.0.

- R4-2417212, WF on Requirements for AI/ML air interface, 3GPP TSG-RAN WG4 #112bis.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 #118bis.

- R1-2308259, “Evaluations on AI/ML for Beam Management”, Qualcomm Incorporated, 3GPP TSG-RAN WG1 #114.

- R4-2417211, “Simulation assumptions for beam prediction”, 3GPP TSG-RAN WG4 #112bis