3GPP TSG-RAN WG4 Meeting #113	R4-2419756

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Maintenance discusison for mobility aspects in NR_NTN Rel-18

Agenda item:	5.21.3

Document for:	Discussion

Introduction

In the present contribution we discuss maintenance issues related to NR_NTN_enh work item, specially when mobility scenarios are considered. The topics to be discussed in the present section are:

- Capabilities and interruption time for Satellite Switch

- Time to fix GNSS when measuring a NTN cell

- High mobility scenarios for timing advance.

Discussion

Scheduling restrictions during soft satellite switch

In RAN4 #109, it was established that scheduling restrictions are expected in when soft satellite switching is configured, depending on certain conditions [3]. The conclusion of this discussion was provided as a LS-Reply to RAN2[4]:

- If the UE starts synchronization to a different satellite from T-ServiceStart, scheduling restriction is expected, if: - the UE is not capable of parallelMeasurementWithoutRestriction-r17, or - the UE is not capable of simultaneousRxDataSSB-DiffNumerology but the SSB has a different SCS than PDCCH/PDSCH - Note: RAN4 is investigating how to minimize the duration of the scheduling restriction.

In RAN4 #110, the discussion has further progressed and the following agreement was made:

### Issue 5-2-S: Soft’ Satellite switch (5-2-S1 and -S2 from RAN4#110 are merged) Agreement (online): - Scheduling restrictions over [t-ServiceStart ~ t-Service] for UE incapable of parallelMeasurementWithoutRestriction-r17 and/or [differentSCS between SSB and data] - Define scheduling restriction during soft satellite switch from UE perspective, i.e. scheduling restriction are allowed only during SSB occasions of the target satellite (same as 9.2C.5.3) For the scheduling restriction: For RSRP measurement, 1 additional symbol before and after SSB block. For RSRQ measurement, 1 additional symbol before and after RSSI symbols

The problem, as we discussed in [5] is that from the network point of view, it is not possible to know when the UE is applying the scheduling restriction, there is an uncertainty of up to 11 ms in the exact time the scheduling restriction applies. Currently,  the full interval between t-serviceStart and t-service should be considered scheduling-restricted for the UE.

To avoid even more overhead in NTN, it is important and simple to minimize the total scheduling restriction during soft switching operation. In this contribution we propose the following manner to alleviate this problem:

A UE that supports soft satellite switching shall mandatorily support parallelMeasurementWithoutRestriction-r17

On the Time-To-First-Fix

The following agreements were obtained related to the time-to-first fix of the GNSS position when the NTN capable UE is performing measurements on the NTN cell while served by a TN cell:

- T_GNSS is TTFF (Time To First Fix) of which value is left undefined in RRM spec. If UE GNSS has been switched ON, T_GNSS can be assumed zero. - The note below is to be implemented in RRM requirement spec: - the above requirement does not assume UE always performs NTN cell detection/measurement as well as TN cells.

The discussion presented in this section was previously presented in our contribution to RAN4 #109, but we still see it as fit and relevant for the current state of agreements.

Previous understanding in RAN4 was that, due to the doppler and time variation in NTN, the usage of satellite assistance information for NGSO measurements was paramount, such that the UE could apply the right doppler and SMTC corrections to perform accurate measurements. However, this is not assumed as a precondition for GSO.

Therefore, if GNSS is necessary for the NGSO measurements to be performed it needs to be switched on when measurements are performed. We propose to make mandatory the UE to switch GNSS on when measuring a higher priority frequency layer in NTN or a lower priority layer after not finding other suitable cell for more than a given period of time. Hence, we propose:

In NGSO scenarios, GNSS shall be on during measurement procedures for cell reselection, if the NTN capable UE is configured to measure a NTN frequency layer with higher priority.

High mobility scenarios for timing advance

In RAN4 specifications, we have agreed that UEs are allowed to perform autonomous adjustment of timing advance, in order to keep fine tracking of the uplink transmission timing. However, as discussed during Release 17, it was important to consider that there is some uncertainty related to the physical information estimated by the UE, in special the UE position estimated by GNSS, which might fluctuate over time due to variations on UE actual position or in the signals used to estimate the GNSS position. Therefore, in order to prevent a multitude of “back and forth” adjustments, we have agreed in RAN4 to introduce some requirements over the UE autonomous adjustments:

7.1C.2.1 Gradual timing adjustment When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN. 2) The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last one second, shall be Tp_NTN per second. 3) The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200ms, shall be Tq_NTN per 200 ms. Where, the maximum autonomous time adjustment step Tq_NTN and the aggregate adjustment rate Tp_NTN are specified in Table 7.1C.2.1-1. Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate Frequency Range; SCS of uplink signals (kHz); Tq_NTN; Tp_NTN. FR1-NTN; 15; 5.5*64*Tc; 5.5*64*Tc. 30; 5.5*64*Tc; 5.5*64*Tc. 60; N/A; N/A. FR2-NTN; 60; 2.5*64*Tc; 2.5*64*Tc. 120; 2.5*64*Tc; 2.5*64*Tc. NOTE: Tc is the basic timing unit defined in TS 38.211 [6].

As indicated by our highlights, RAN4 specs indicate that only changes on the UE specific part due to satellite position update are not counted as part of the gradual adjustment requirements, which means that the changes in UE position are bound to the requirements above.

Changes in timing caused by updates on UE position are limited by maximum and minimum adjustment rate requirements.

Another important aspect to consider is that airborne devices have been considered an example of application for NTN since Rel-17. For example, in Release 18, and operation in FR2-NTN, a special type of UE was introduced (VSAT) but it was accounted and presented in the WID that two types of devices should be considered (and one of the example cases in the WID is the airborne device):

Fixed VSAT: VSAT used in FSS system at given position; the given position may be a specified fixed point or any fixed point within specified areas. NOTE: Mobile VSAT is excluded from this definition Mobile VSAT: VSAT on moving platform, and which can be further declined in three types: airborne, maritime or land based. NOTE: Mobile VSAT can be also referred to as ESIM or ESOMP

For FR1-NTN, the values currently defined for Tp and Tq are equal to 0,18 us (microseconds), whereas for FR2-NTN the values currently defined for Tp and Tq are equal to 0,08 us (microseconds). These values shall account for twice the changes in the distance between the UE and the base station.

Assuming a speed of light approximated by 300,000 km/s, such correction factors would correspond to distances of respectively 26,8 and 12,2 meters. When applying these values in point 3 above, that would force the variation in distance to be smaller than 134 m in FR1-NTN or 60 m in FR2-NTN every second. This translates into a change in distance caused by a relative speed of 483 km/h (FR1-NTN) or 219 km/h (FR2-NTN). Such maximum speeds of adjustments are not enough to cover for airplane speeds (up to 900 km/h and beyond).

Based on the analysis above, we propose:

RAN4 to design a solution to enable fast correction measurements for high speed UEs (such as airborne devices).

Conclusion

The paper has discussed the need of side conditions for the applicability of mobility requirements for enhanced mobility features in NTN. The following observations and proposals were made:

Proposal 1: A UE that supports soft satellite switching shall mandatorily support parallelMeasurementWithoutRestriction-r17

Proposal 2: In NGSO scenarios, GNSS shall be on during measurement procedures for cell reselection, if the NTN capable UE is configured to measure a NTN frequency layer with higher priority.

Observation 1: Changes in timing caused by updates on UE position are limited by maximum and minimum adjustment rate requirements.

Proposal 3: RAN4 to design a solution to enable fast correction measurements for high speed UEs (such as airborne devices).

- References

- RP-223534, “NR NTN (Non-Terrestrial Networks) enhancements”, Thales

- R4-2406430, “WF on RRM requirements for NR_NTN_enh”, Changsha, China, 15th – 19th April, 2024

- R4-2403492, WF on NTN RRM requirements, Moderator (Qualcomm Incorporated), RAN4#110, Athens, Greece, 26 Feb – 01 Mar, 2024

- R4-2403493, Reply LS on RAN2 agreements for satellite switch with resync, Apple, RAN4#110, Athens, Greece, 26 Feb – 01 Mar, 2024

- R4-2405751, Discussion on Satellite switching with resynchronization, Nokia, RAN4#110bis, Changsha, China, April 15 – April 19, 2024