3GPP TSG RAN WG4 Meeting #113		R4-2419760

Orlando, USA, November 18th-22nd, 2024

Agenda item:	7.12.3.1

Source:	Telecom Italia S.p.A., Vodafone, Deutsche Telekom, Orange

Title:	Proposals for Rel-19 TRP TRS requirements topic

WI/SI:	TRP_TRS_MIMO_OTA_Ph3

Release:	Rel-19

Document for:	Approval

# 1	Introduction

During RAN4#112-bis meeting, first discussion on Rel-19 TRP TRS requirements related to the WI [1] was held which outcomes and agreements have been captured in the WF [2]. This contribution provides some proposals related to some of the issues captured in the WF.

# 2	Working procedure for 2Tx lab alignment activity

Issue 4-1-1: working procedure for 2Tx lab alignment activity

The current working procedure for 2Tx lab alignment campaign doesn’t preclude that new volunteer labs (i.e, lab different from the one that took part to Rel-17 and Rel-18 activities) are included, pending their alignment based on the measurements of the selected LADs. Such new volunteer labs should also complete the alignment for 1Tx as follows:

Proposal 1:

  - Test bands: For volunteer labs that have already been declared aligned in Rel-17 and Rel-18 lab alignment campaigns: focus is only on [n41, n78], two typical bands non-coherent UL-MIMO; (For volunteer labs that have not been declared aligned in Rel-17 and Rel-18: In addition to 2Tx, also complete 1Tx lab alignment on at least n78 (high band) and n28 (low band))

Concerning the number of test cases per band and the use scenario, it is proposed to consider the Talk mode as reference for the alignment, in order to facilitate also the activity of new volunteer labs (to be aligned for 1Tx as well):

Proposal 2:

  - Number of test cases per band: TRP and TRS; BHHL and BHHR at low/mid/high channel;

  - Use scenario: Head and Hand phantom (Talk mode)

# 3	1Tx FR1 TRP TRS requirements

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (assume no large scale measurement campaign, current requirement is a basis)

In relation to the definition of the minimum performance requirements related to PDA hand (i.e., Size 2 devices), it is FFS how to down select between measurement-based approach and the offset-based approach. The following observations can be made:

Observation 1: Size 1 and Size 2 handheld devices are based on completely different design so that it is not possible to define a numerical approach (i.e., offset or more complex formula) to corelate the related performances.

Observation 2: The performances are dependent on the considered bands, meaning that for each one a specific offset should be defined.

Observation 3: The measurements performed on two Size 2 devices are not sufficient to determine any offset with respect to the minimum performance requirements defined for the Size 1 devices being based on a pool of 80 samples.

Based on the above observations, it is proposed the following:

Proposal 3: Define the minimum performance requirements for Size 2 devices based on an complete measurement campaign. The number of minimum devices to be measured (and related thresholds) is FFS and depends on the available number of such device currently present on the market.

Issue 4-2-4: Working procedure for 1Tx requirements related performance measurement campaign

The discussion during the Rel-17 and Rel-18 work on minimum performance requirements definition brought to the attention different issues that should be considered in the current Rel-19 work in order to further improve the adopted working procedure.

Concerning the measurements results, the following observation can be made:

Observation 4: Even if a laboratory is confirmed as aligned, this does not prevent that mistakes can happen in the setup of the measurement environment.

Observation 5: It is not possible to prevent the provisioning of devices that can be defective in the devices pool for the measurement campaign.

Observation 6: For both cases, impact on the provided results that would affect the minimum performance requirements definition are possible. Means to evaluate.

Based on the above:

Proposal 4: Discuss and define a proper methodology (e.g., threshold) to identify and exclude outlier results in the context of the TRP TRS requirements specification phase of the working procedure.

Regarding the testing lab procedures for 1Tx measurements, the results provided for Rel-18 activity were based on the legacy grid providing in high reliability of the provided values. To guarantee the same level of confidence and alignment with the Rel-18, the same approach shall be used for Rel-19:

Proposal 5:

- Legacy grid shall be used for TRP TRS measurement

# 5	Conclusions

In the following, the observations and proposals of the contribution are summarized:

Proposal 1:

- Test bands: For volunteer labs that have already been declared aligned in Rel-17 and Rel-18 lab alignment campaigns: focus is only on [n41, n78], two typical bands non-coherent UL-MIMO; (For volunteer labs that have not been declared aligned in Rel-17 and Rel-18: In addition to 2Tx, also complete 1Tx lab alignment on at least n78 (high band) and n28 (low band))

Proposal 2:

- Number of test cases per band: TRP and TRS; BHHL and BHHR at low/mid/high channel;

- Use scenario: Head and Hand phantom (Talk mode)

Observation 1: Size 1 and Size 2 handheld devices are based on completely different design so that it is not possible to define a numerical approach (i.e., offset or more complex formula) to corelate the related performances.

Observation 2: The performances are dependent on the considered bands, meaning that for each one a specific offset should be defined.

Observation 3: The measurements performed on two Size 2 devices are not sufficient to determine any offset with respect to the minimum performance requirements defined for the Size 1 devices being based on a pool of 80 samples.

Proposal 3: Define the minimum performance requirements for Size 2 devices based on an complete measurement campaign. The number of minimum devices to be measured (and related thresholds) is FFS and depends on the available number of such device currently present on the market.

Observation 4: Even if a laboratory is confirmed as aligned, this does not prevent that mistakes can happen in the setup of the measurement environment.

Observation 5: It is not possible to prevent the provisioning of devices that can be defective in the devices pool for the measurement campaign.

Observation 6: For both cases, impact on the provided results that would affect the minimum performance requirements definition are possible. Means to evaluate.

Proposal 4: Discuss and define a proper methodology (e.g., threshold) to identify and exclude outlier results in the context of the TRP TRS requirements specification phase of the working procedure.

Regarding the testing lab procedures for 1Tx measurements, the results provided for Rel-18 activity were based on the legacy grid providing in high reliability of the provided values. To guarantee the same level of confidence and alignment with the Rel-18, the same approach shall be used for Rel-19:

Proposal 5:

- Legacy grid shall be used for TRP TRS measurement

# 6	References

- RP-241623, “WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3”, vivo, June 2024

- R4-2416590, “Way Forward for [112bis][323] TRP_TRS_Ph3”, vivo, October 2024