3GPP TSG-RAN WG4 Meeting #113									                        R4-2419761

Orlando, US, November 18 – 22, 2024

Agenda item:			7.8.2.3.2

Source:	Ericsson

Title:	On UE RF Rx requirements for IoT-NTN HPUE

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for IoT-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Rx requirements for IoT-NTN HPUE from the approved WF in RAN4#112-bis [2].

# Discussion

In the approved WF [2], the following has been agreed:

Issue 2-1-2: RSD definition

- Proposals

  - Option 1: New RSD tables for different higher power class, similar to NR TN; (vivo)

  - Option 2: New Refsens tables for different higher power class, similar to LTE spec (Table 7.3.1-1b in 36.101); (Ericsson)

  - Option 3: Add additional rows in the existing tables. (Ericsson)

Agreement:

- Follow NR approach for RSD as much as possible.

Issue 2-1-3: PC2 RSD

- Proposals

  - 256:

    - RSD = 0dB (MediaTek, vivo, Samsung)

  - 255:

    - FFS (vivo)

    - Using n3 RSD as reference (Samsung)

  - 254: RSD = 0dB (MediaTek, vivo)

- WF

  - Reuse NR agreements:

- For 256: [0dB] RSD for PC2 for 1Tx and 2Tx (if considered)

  - FFS on the duplexer isolation and PA interference

Similar to NR-NTN, band 256 has a slightly larger duplexing distance than band n1 so at least the same duplexer isolation should be achievable for the two bands. In TS38.101-1 [3], 0dB RSD is specified for n1 for PC2 UE, so the same requirement could be reused for band 256. When it comes to the UL configuration for reference sensitivity, since the only channel bandwidth specified for IoT-NTN category M1 is 1.4 MHz with the maximum transmission bandwidth configuration of 6 RBs, that configuration should be used for REFSENs tests for all Power Classes. Similar to NR-NTN, 0dB RSD should be adopted for band 254 as well since the duplexing distance is very large.

Proposal 1: For bands 256 and 254, specify 0dB RSD for PC2 IoT-NTN UEs.

Issue 2-1-4: PC1 RSD

- Proposals

  - Proposal 1: For PC1 1Tx for full duplex eMTC based Iot-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB (for n254/n255/n256).

- Recommended WF

  - FFS

Since only full duplex eMTC based IoT-NTN is considered for non-handheld UEs with 1Tx, in our view 0dB RSD could be adopted for PC1 as well for all bands.

Proposal 2: Specify 0dB RSD for PC1 with 1Tx for full duplex eMTC based non-handheld UEs for IoT-NTN.

# Conclusion

Based on the discussion in the previous sections we have the following proposals:

Proposal 1: For bands 256 and 254, specify 0dB RSD for PC2 IoT-NTN UEs.

Proposal 2: Specify 0dB RSD for PC1 with 1Tx for full duplex eMTC based non-handheld UEs for IoT-NTN.

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2417205, “WF on HPUE for IoT NTN”, vivo, RAN4#112-bis.

- 3GPP TS 38.101-1 V18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”