3GPP TSG-RAN WG4 Meeting RAN4#113      				R4-2419777 Orlando, USA

October 18 – 22, 2024

Agenda Item:	5.22.2.6

Source:	Lenovo

Title:	On Ambiguity in the Definition of RSCP with Frequency Offsets

Document for:  Approval

- Introduction

The effect of carrier frequency offset on carrier phase measurements was discussed extensively in [1-8].  The fundamental issue is that the double differencing method that is the basis of the carrier phase positioning method assumes that the phase difference between the TRP and the UE depends only on the initial phase of the TRP transmitter, the initial phase of the UE receiver, and the distance between the TRP and the UE.  However, the carrier phase offset between the UE and the TRP is constantly changing due to the carrier frequency offset that always exists in practice, including during the carrier phase measurement. The consequence of this time variation of the carrier phase is that both the carrier phase measurement and the carrier phase difference measurements are not well defined for measurement window sizes larger than one.

Observation 1:	In the presence of a carrier frequency offset, reference signal carrier phase and reference signal carrier phase difference are not well-defined for measurement window sizes larger than one.

In this contribution, we further discuss this issue and propose that an LS be sent to RAN1 requesting clarification of the reference signal carrier phase (RSCP) and reference signal carrier phase difference (RSCPD) definitions in the presence of carrier frequency offsets between the TRP’s.

- Carrier Frequency Tolerances

Carrier phase positioning assumes the existence of initial phase offsets among the DL-PRS transmitters, i.e., gNBs, and among the UE and PRU receivers which are static.  In general, however, there will be frequency offsets among the DL-PRS transmitters which will cause the carrier phase offsets to constantly change relative to each other over time.

The carrier frequency tolerance for the base station is defined in TS 38.104 and is given in the following two tables:

Table 6.5.1.2-1: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

Table 9.6.1.3-1: OTA frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

The carrier frequency tolerance for the UE is defined in TS 38.101-1 for FR1.  In section 6.4.1 of TS 38.101-1, we have the requirement:

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B.

Thus, except for the wide area base station, the frequency accuracy requirement is ± 0.1 PPM which corresponds to a maximum frequency error of ± 102 fc, where fc is expressed in GHz.

- RSCP and RSCPD definitions

# The definitions of reference signal carrier phase (RSCP) and reference symbol carrier phase difference (RSCPD are given in TS 38.215.  The definition of RSCP is copied below:

| Definition | DL reference signal carrier phase (RSCP) is defined as the phase of the channel response at the 1st path delay derived from the resource elements carrying DL PRS configured for the measurement. DL RSCP is associated with the center frequency of the DL positioning frequency layer (PFL) configured for the measurement for RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE modes. For frequency range 1, the reference point for the DL RSCP shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCP shall be the antenna of the UE. |
| --- | --- |

# Given that the UE can only measure carrier phase relative to its serving cell (or reference cell), there is a question as to how to interpret this phase definition when it is constantly changing during the measurement period due to frequency offsets between the serving TP and the measured TP.

# Consider the following highly idealized assumptions:

# A1) The UE perfectly tracks the carrier frequency of the serving TP.

# A2) The UE can perfectly measure the phase of the PRS reference symbols relative to its serving TP.

# Let ∆f denote the frequency offset between the measured TP and the serving TP.  It can be noted that the maximum absolute value of ∆f is 102 fc, for wide area base stations and 2ּ 102 fc for medium range and local area base stations, where fc is expressed in GHz.

# Refer to Figure 1.  The number of symbols in the slot configured with PRS is given by LPRS∈1,2,4,6,12.  Furthermore, the PRS can be repeated in consecutive slots with repetition factor TrepPRS∈1,2,4,6,8,16,32.   In the case that the repetition factor is TrepPRS=1, the number of symbols separating the first and last PRS symbol is equal to LPRS-1. In the case that the repetition factor is greater than 1, the number of symbols separating the first and last PRS symbol is equal to 14∙TrepPRS-1+LPRS-1.

# Figure 1:  Configuration of PRS with TrepPRS > 1.

# The worst-case symbol-to-symbol phase rotation of the measured TP relative to the serving TP is shown in Table 1 as a function of carrier frequency and subcarrier spacing for medium range and local area base stations (± 0.1 PPM tolerance).

|  |  | Δθmax (degrees) |  |
| --- | --- | --- | --- |
| fc (GHz) | Δfmax (Hz) | SCS = 30 kHz | SCS = 60 kHz |
| 2 | 400 | 5.1 | 2.6 |
| 4 | 800 | 10.3 | 5.1 |
| 6 | 1200 | 15.4 | 7.7 |

# Table 1:  Worst case symbol-to-symbol phase rotation of measured TP relative to serving TP vs. carrier frequency and subcarrier spacing.

# The corresponding phase rotation of the last PRS symbol in the last slot relative to the first PRS symbol in the first slot is shown in Table 2 for LPRS = 4 and LPRS = 12 and also for repetition factors TrepPRS =1 and TrepPRS = 4.

|  | Phase Difference in Degrees |  |  |  |
| --- | --- | --- | --- | --- |
|  | TrepPRS =1 |  | TrepPRS =4 |  |
| fc (GHz) | LPRS = 4 | LPRS = 12 | LPRS = 4 | LPRS = 12 |
| 2 | 15.4 | 56.6 | 231.4 | 272.6 |
| 4 | 30.9 | 113.1 | 462.9 | 545.1 |
| 6 | 46.3 | 169.7 | 694.3 | 817.7 |

# Table 2:  Worst case phase rotation in degrees of the last PRS symbol in the last slot relative to the first PRS symbol in the first slot (SCS = 30 kHz).

Based on the results in Table 2, we have the following observations:

Observation 2:	With carrier frequency offsets within the existing tolerances, phase change of the measured TP relative to the serving TP over the measurement period can be extremely large and easily large enough to render the measurements unusable, especially for TrepPRS >1.

Observation 3:	Even a perfect receiver which is perfectly synchronized to its serving TP and which can perfectly measure the carrier phase of the PRS will not know what phase to report since the phase (perfectly measured for each PRS symbol) is constantly changing over the measurement window due to the frequency offset.

Observation 4:	In order for the initial phase offsets of the TP’s to cancel when measurements are combined at the LMF, all UE’s must take measurements in the same way.

# It can be noted that the UE can report RSCP measured for a single symbol.  However, this is an optional feature.  Furthermore, it is not clear how the LMF would combine RSCP measurements if UE’s do not all report measurements taken on the same symbol.

# Based on the above observations, in particular that

- RSCP is not well-defined for measurement windows larger than one with a carrier frequency offset;

- carrier frequency offsets between the TP’s will always be present;

- all UE’s must take measurements the same way for the initial phase offsets to cancel at the LMF so that the feature works;

# we have the following proposal.

Proposal:	RAN4 to send an LS to RAN1 requesting clarification of the definition of RSCP and RSCPD in the presence of carrier frequency offsets between the TP’s.

- Summary

This contribution has considered the impact of carrier frequency offset on the definition of reference signal carrier phase (RSCP) and reference signal carrier frequency difference (RSCPD), and it has been shown that RSCP is not well defined for measurement windows larger than one symbol if there is a carrier frequency offset.

Based on the above, we have the following observations and proposal:

Observation 1:	In the presence of a carrier frequency offset, reference signal carrier phase and reference signal carrier phase difference are not well-defined for measurement window sizes larger than one.

Observation 2:	With carrier frequency offsets within the existing tolerances, phase change of the measured TP relative to the serving TP over the measurement period can be extremely large and easily large enough to render the measurements unusable, especially for TrepPRS >1.

Observation 3:	Even a perfect receiver which is perfectly synchronized to its serving TP and which can perfectly measure the carrier phase of the PRS will not know what phase to report since the phase (perfectly measured for each PRS symbol) is constantly changing over the measurement window due to the frequency offset.

Observation 4:	In order for the initial phase offsets of the TP’s to cancel when measurements are combined at the LMF, all UE’s must take measurements in the same way.

Proposal:	RAN4 to send an LS to RAN1 requesting clarification of the definition of RSCP and RSCPD in the presence of carrier frequency offsets between the TP’s.

# References

- R1-2204807, “Design Aspects of Carrier Phase Measurements for NR Positioning”, Intel, #109-e.

- R4-2311991, “Effect of Carrier Frequency Offset on Carrier Phase Positioning,” Lenovo, Motorola Mobility, RAN4#108.

- R4-2316796,” Carrier Frequency Offset Correction for CPP,” Lenovo, Motorola Mobility, RAN4#108bis.

- R4-2318899, “Correction of Carrier Phase Measurement Errors Due to Carrier Frequency Offsets,” Lenovo, Motorola Mobility, RAN4#109.

- R4-2402603, “Positioning Error Due to Carrier Frequency Offset,” Lenovo, RAN4#110.

- R4-2405873, “Correction of CPP Errors Due to Carrier Frequency Offsets,” Lenovo, RAN4#110bis.

- R4-2409575, “Definition of Carrier Phase and Carrier Phase Difference with Carrier Frequency Offsets,” Lenovo RAN4#111.

- R4-2416505, “On the Need for Additional Scenarios for CPP,” Lenovo, RAN4#112bis.

- 3GPP TS 38.104 NR; Base Station (BS) radio transmission and reception

- 3GPP TS 38.101-1 User Equipment radio transmission and reception; Part 1: Range1 Standalone