3GPP TSG-RAN4 Meeting #113	R4-2419855

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0700 | rev | 1 | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_HST_FR1_enh-Perf) Update FrequencyOccupation for CSI-RS for tracking in HST-SFN CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR1_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test parameter table is not defining any limitation for the number of PRB for the CSI-RS resources for tracking. However, according to TS 38.214 clause 5.1.6.1.1: the UE is not expected to be configured with the periodicity of slots if the bandwidth of CSI-RS resource is larger than 52 resource blocks. As for TDD with SCS 30kHz, the CSI-RS periodicity is defined to be 20 slots, for BW 20 MHz or higher, the nrofRB for CSI-FrequencyOccupation for TRS will be higher than 52. For FDD with SCS 15kHz, the CSI-RS periodicity is defined to be 10 slots, so, for BW 15 MHz or higher, the nrofRB for CSI-FrequencyOccupation for TRS will be higher than 52. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a limitation on the number of PRB for FrequencyOccupation for TDD 20MHz and higher for CSI-RS resources for tracking. Define a limitation on the number of PRB for FrequencyOccupation for FDD 15MHz and higher for CSI-RS resources for tracking. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.2.4, 5.2A.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 5.2A.2.4	Minimum requirements for HST-SFN CA

For HST-SFN CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.2.4-5 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.2.4-3 and Table 5.2A.2.4-4. Test parameters are specified in Table 5.2A.2.4-2, Table 5.2A-2, and Table 5.2A-3 with downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

The test purpose is specified in Table 5.2A.2.4-1.

Table 5.2A.2.4-1: Test purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions in the HST-SFN scenario defined in B.3.2 with CA | 1,2,3 |

Table 5.2A.2.4-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | FDD: 10 for CSI-RS resource 1,2,3,4. TDD: 20 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. |
|  | Frequency Occupation |  | Start PRB = 0 Number of PRB = 52 for TDD with BW 20MHz and higher BW Number of PRB = 52 for FDD with BW 15MHz and higher BW Number of PRB = ceil(BWP size/4)*4 for other case |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| Number of PUCCH ResourceGroups |  |  | 1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases with no more than 2 DL CCs PUCCH format 3 for cases with more than 2 DL CCs |

Table 5.2A.2.4-3: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-13.1 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 12.9 |
| 10 | R.PDSCH.1-8.3 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.1 |
| 15 | R.PDSCH.1-13.2 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.4 |
| 20 | R.PDSCH.1-13.3 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.9 |
| 25 | R.PDSCH.1-13.4 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.0 |
| 30 | R.PDSCH.1-13.5 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.9 |
| 35 | R.PDSCH.1-14.3 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.8 |
| 40 | R.PDSCH.1-14.1 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.0 |
| 45 | R.PDSCH.1-14.4 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.9 |
| 50 | R.PDSCH.1-14.2 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.0 |

Table 5.2A.2.4-4 Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-19.1 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.4 |
| 10 | R.PDSCH.2-19.2 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.7 |
| 15 | R.PDSCH.2-19.3 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.8 |
| 20 | R.PDSCH.2-19.4 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.8 |
| 25 | R.PDSCH.2-19.5 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.1 |
| 30 | R.PDSCH.2-20.1 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.4 |
| 40 | R.PDSCH.2-10.4 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.6 |
| 50 | R.PDSCH.2-20.2 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.7 |
| 60 | R.PDSCH.2-20.3 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.4 |
| 80 | R.PDSCH.2-20.4 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.9 |
| 90 | R.PDSCH.2-20.5 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 15.4 |
| 100 | R.PDSCH.2-21.1 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.8 |

Table 5.2A.2.4-5: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.2.4-3 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.2.4-4 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.4-3 and Table 5.2A.2.4-4 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. |  |  |

## <<< Skip unchanged sections >>>

#### 5.2A.3.4	Minimum requirements for HST-SFN CA

For HST-SFN CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.3.4-5 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.3.4-3 ~ Table 5.2A.3.4-4, with the parameters in Table 5.2A.3.4-2, Table 5.2A-2, Table 5.2A-3, and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

The test purpose is specified in Table 5.2A.3.4-1.

Table 5.2A.3.4-1: Test purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions in the HST-SFN scenario defined in B.3.2 with CA | 1, 2, 3 |

Table 5.2A.3.4-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | FDD: 10 for CSI-RS resource 1,2,3,4. TDD: 20 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. |
|  | Frequency Occupation |  | Start PRB = 0 Number of PRB = 52 for TDD with BW 20MHz and higher BW Number of PRB = 52 for FDD with BW 15MHz and higher BW Number of PRB = ceil(BWP size/4)*4 for other case |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| Number of PUCCH ResourceGroups |  |  | 1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases with no more than 2 DL CCs PUCCH format 3 for cases with more than 2 DL CCs |

Table 5.2A.3.4-3: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-13.1 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 10.5 |
| 10 | R.PDSCH.1-8.3 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 10.7 |
| 15 | R.PDSCH.1-13.2 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.1 |
| 20 | R.PDSCH.1-13.3 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.5 |
| 25 | R.PDSCH.1-13.4 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.6 |
| 30 | R.PDSCH.1-13.5 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.8 |
| 35 | R.PDSCH.1-14.3 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.7 |
| 40 | R.PDSCH.1-14.1 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.9 |
| 45 | R.PDSCH.1-14.4 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.7 |
| 50 | R.PDSCH.1-14.2 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.9 |

Table 5.2A.3.4-4: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-19.1 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.0 |
| 10 | R.PDSCH.2-19.2 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.8 |
| 15 | R.PDSCH.2-19.3 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.1 |
| 20 | R.PDSCH.2-19.4 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.8 |
| 25 | R.PDSCH.2-19.5 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.9 |
| 30 | R.PDSCH.2-20.1 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.2 |
| 40 | R.PDSCH.2-10.4 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.4 |
| 50 | R.PDSCH.2-20.2 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.6 |
| 60 | R.PDSCH.2-20.3 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.5 |
| 80 | R.PDSCH.2-20.4 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.7 |
| 90 | R.PDSCH.2-20.5 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.7 |
| 100 | R.PDSCH.2-21.1 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.7 |

Table 5.2A.3.4-5: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.4-3 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.4-4 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.14-3 and Table 5.2A.3.4-4 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in Section 5.1.1.7. |  |  |

## <<< END OF CHANGES >>>