# 3GPP TSG-RAN WG4 Meeting #113	R4-2420093

# Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.15.4

Source:	Apple

Title:	WF on RRM requirements for NR_RRM_Ph5_Part1

Document for:	Approval

# Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor (7.15.2.1)

### Issue 1-1: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions):

- Option 1(CATT, LGE, CMCC, Apple, HW, vivo, OPPO, ZTE, MTK, QC, Samsung): RAN4 to introduce signal quality (e.g., cell edge) based triggering condition to activate/deactivate multi-Rx for L3 measurement

  - If option 1 is agreed, discuss whether option 1a is agreeable

- Option 2(CATT, Apple, OPPO, MTK, QC, Samsung): RAN4 to introduce UE mobility status based triggering condition to activate/deactivate multi-Rx for L3 measurement

  - Opponent to option 2 (CMCC, Intel): it is proposed not to consider UE mobility as triggering condition for activation/deactivation of FBS

  - If option 2 is agreed, discuss whether option 2a is agreeable.

- Option 3(Nokia, Ericsson): Use L3 event-triggered reporting conditions for triggering FBS

  - Discuss whether option 3 is agreeable.

- Discuss Option 3b and option 10 to decide if the UE FBS status report to network is needed.

- Other options can be discussed online based on companies’ comments.

Agreement:

- RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement.

Agreement:

- Not support UE mobility status based triggering condition in Rel-19.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” :

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

Issue 1-1-3: searcher number assumption for RRM measurement with two panels activated on single carrier

Agreement:

Directly discuss the SSB processing time.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC

Agreement:

- For scenario 1/2/3/4

  - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

    - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms

    - where TSSB_processing = 2 ms

Agreement:

- For scenario 7

  - to consider an additional processing delay (2ms) after the last SMTC periodicity

[Wayforward]

- For scenario 5

  - FFS:

    - Option 1: to consider an additional processing delay (2ms) in the handover delay equation

    - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions.

    - Option 3: no need to add additional processing time in the delay requirement.

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX

Agreement:

L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

### Issue 1-2: Requirement of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Agreement:

For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability:

- FFS: details of the capability

Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

Issue 1-2-3: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

### Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF

Agreement:

- Discuss this capability detail by end of the core part

### Issue 1-4: others

Issue 1-4-1: FFS: other requirement imapcts

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

# Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (6.15.2.2)

### Issue 2-1: Clarification on the bullets in WID for this CSSF optimization

Agreement:

- RAN4 to follow the revised WID (WID RP-242380) in RANP#105 meeting, and no need to have further discussion on this issue.

### Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

### Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

### Issue 2-4: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Issue 2-4-1: Tentative solution 1: measure one serving CC per band

[Wayforwad]:

FFS the following solution:

  - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured,

    - Reuse the legacy principle for neighbour cell measurement

      - Measure PCC if PCC in band or

      - Measure PSCC if PSCC in band or

      - Measure SCC with neighbor cell MO or

      - UE implementation

    - FFS whether indication is per-UE or per-band

      - Other options are not precluded.

  - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

[Wayforward]:

FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1:

  - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement.

    - FFS whether network indication is needed

      - Details is FFS

    - FFS on whether and how PCC and PSCC resource sharing is supported

Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF

Agreement:

RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is:

    - FFS: FR1 scenario (CA and NR-DC) only.

    - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only.

    - FFS: All scenarios.

Issue 2-4-4: Other tentative solutions

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

### Issue 2-5: feature capability of L3 measurement delay reduction by optimizing CSSF for tentative solution 1 and 2

Agreement:

- RAN4 to introduce new individual capability(s) for CSSF reduction in R19.

  - delay the capability detail discussion to the end of the core part.

### Issue 2-7: Other WID scope discussion

Agreement:

- after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable