3GPP TSG-RAN WG4 Meeting #113									                  R4-2420097

Orlando, US, 18 – 22 November, 2024

Agenda item:			7.20.4

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NR_duplex_evo

Document for:	Approval

- Agreements

### Sub-topic 1-1: RRM impacts of UE-to-UE CLI handling

#### Issue 1-1-2: Measurement methods

- Agreements (from Thu AH session)

  - RAN4 shall specify L1 CLI measurement requirements at least for below methods:

    - Method#1: UE measures RSSI within DL subband

    - Method#2: UE measures RSRP of aggressor UE within UL subband

  - Additional methods can be considered based on further RAN1 agreements.

  - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

#### Issue 1-1-3: Rx beam

- Agreements (from Thu AH session)

  - RAN4 to capture the Rx beam determination for L1 CLI measurement based on RAN1 agreement, FFS how to capture, e.g. specify the UE behaviour or use a reference to RAN1 spec.

  - Rx beam sweeping is not needed for L1 CLI measurement.

#### Issue 1-1-6: Measurement period

- Agreements (from Wed AH session)

  - For L1-SRS-RSRP, further discuss the following options based on simulation results:

    - Option 1: requirement specified with [1] sample measurement assumption

    - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement

  - For L1-CLI-RSSI,

    - Confirm measurement period requirements are defined based on 1 sample, and

    - Clarify in the requirements that the measurement is based on DL timing.

#### Issue 1-1-9: Scheduling and measurement restriction

- Agreements (from Thu AH session)

  - For scheduling restriction,

    - For L1-SRS-RSRP,

      - For FR1, R16 requirements are re-used as baseline, FFS exact number of restricted symbols considering the impact of timing assumption

      - For FR2, FFS whether R16 requirements are re-used as baseline, considering the impact of Rx beam determination, and timing assumption

    - For L1-CLI-RSSI, FFS whether R16 requirements are re-used as baseline, considering the measurement is based on DL timing.

  - For measurement restriction,

    - Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

      - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

### Sub-topic 1-2: RRM impacts of SBFD operation

#### Issue 1-2-1: Requirements for SSB based measurement

- Agreements (from online session)

  - For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

  - This can be revisited if it proves to be conflicting with further RAN1 agreements.

#### Issue 1-2-2: Requirements for CSI-RS based L1 measurement

- Agreements (from online session)

  - For CSI-RS colliding with UL transmission

    - For aperiodic CSI-RS resource, existing measurement requirements apply, i.e. the measurement is prioritized over semi-statically configured UL transmission.

    - For periodic and semi-persistent CSI-RS resource, RAN4 will send RAN1 LS for clarification if needed.

  - For CSI-RS puncturing in frequency domain in SBFD symbols,

    - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs.

    - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold.

      - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs.

      - FFS for the DUD configuration.

    - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions.

    - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold.

    - FFS the measurement behavior for DUD configuration.

    - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement.

  - For CSI-RS distributed over SBFD and non-SBFD symbols,

    - Ask RAN1 clarification on whether related RAN1 agreement applies for L1 measurement (L1-RSRP/SINR measurement and RLM/BFD/CBD measurement). Further discuss the wording when drafting the LS.

#### Issue 1-2-3: Requirements for CSI-RS based L3 measurement

- Agreements (from online session)

  - Companies to check the existing requirement, and discuss whether they can be applied to SBFD operation and discuss any additional aspect needs to be addressed.

- Issue 1-2-4: Requirements for scheduling restriction

- Agreements (from Wed AH session)

  - For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction.

  - Further discuss the impact to inter-frequency SSB measurement without gap case.

# Potential issues for further discussion

### Sub-topic 1-1: RRM impacts of UE-to-UE CLI handling

#### Issue 1-1-1: Baseline for defining requirements

- Recommended WF

  - Close the issue, and RAN4 to focus on the exact requirement development rather than the baseline.

#### Issue 1-1-4: Measurement resources

- Recommended WF

  - Further discuss the issue

- Issue 1-1-5: Side condition

- Recommended WF

  - Further discuss the issue

#### Issue 1-1-7: Measurement reporting

- Recommended WF

  - Further discuss the issue

#### Issue 1-1-8: Measurement accuracy

- Recommended WF

  - Further discuss the issue

#### Issue 1-1-10: Measurement capability

- Recommended WF

  - Further discuss the issue

#### Issue 1-1-11: Report mapping

- Recommended WF

  - Further discuss the issue

#### Issue 1-1-12: Requirement applicability

- Recommended WF

  - Further discuss the issue

### Sub-topic 1-2: RRM impacts of SBFD operation

- Issue 1-2-5: Requirements for RACH requirements

- Recommended WF

  - Further discuss the issue.

- Issue 1-2-6: Requirements for UL PC and or spatial relation update

- Recommended WF

  - Interested companies are encouraged to bring proposals on exact RRM impacts, if any

- Issue 1-2-7: Requirements for MG and BWP switch

- Recommended WF

  - Further discuss the issue.

- Issue 1-2-8: Requirements for UL resource muting

- Recommended WF

  - Interested companies are encouraged to bring proposals on exact RRM impacts, if any

- Issue 1-2-9: Requirements for generic SBFD operation

- Recommended WF

  - The issue is covered by Issue 1-2-4. Close this issue without agreement.