3GPP TSG-RAN WG4 Meeting #113	R4-2420237

# Orlando, US, 18th – 22nd November, 2024

Title: 	WF on RRM requirements for NR_ATG_enh

Agenda Item:	7.10.5

Source: 	CMCC

Document for:	Approval

<Topic 1> RRM core requirement

<Sub-topic 1-1> General

Issue 1-1-1: UE antenna type for inter-band CA

FFS

- Proposal 1: To support Case 4 antenna type, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid critical receving performance degradataion, e.g., conditions for intra-band NCCA operation.

- Proposal 2: Confirm the understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

- Proposal 3: Wait for more progress from RF session.

  - Proposal 3-1: If the implementation of Case 4 is not excluded in RF session, RRM requirement and related UE capability should be defined, details can be discussed later.

Issue 1-1-2: UE Antenna Type signaling

FFS

- Proposal 1: If ATG UE can perform measurements on different serving cells using different types of antennas, it is necessary to provide assistance information to the network regarding which antenna panel combination is used on the UE side.

- Proposal 2: To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed with per BC type for the bands in which UE supporting phase antenna array.

<Sub-topic 1-2> Timing

Issue 1-2-1: MRTD for ATG inter-band CA

FFS

- Option 1:

  - For the MRTD except Case#4: 33µs. Further discuss for case 4.

- Option 2:

  - For the MRTD except Case#4: 33µs.

  - If case 4 is not excluded based on RF discussion: 3µs.

- Option 3:

  - For the MRTD for all cases: 33µs.

<Sub-topic 1-3> Signalling characteristics

Issue 1-3-1: Interruption requirements

Agreement:

- Introduce following requirements for R19 ATG

  - Interruptions at SCell addition/release

  - Interruptions at SCell activation/deactivation

  - Interruptions at direct SCell activation

  - Interruptions at fast SCell activation

  - Interruptions during measurements on deactivated SCC

  - Interruptions due to SCell dormancy

  - Interruptions due to UE-specific CBW change

  - Interruptions due to Active BWP switching Requirement

  - Interruptions when identifying CGI of an NR cell with autonomous gaps

- Not introduce following requirements for R19 ATG

  - Interruptions at UL carrier RRC reconfiguration

  - Interruptions at inter-frequency SFTD measurement

  - Interruptions at NR SRS carrier based switching

  - Interruptions due to PUCCH SCell activation/deactivation

  - Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]

- FFS whether following requirements need to be introduced

  - Interruptions at NR SRS antenna port switching

- Further discuss the potential impact from UE antenna types

- Further discuss how to reflect the interruption requirement for R19 ATG

Issue 1-3-2: SCell Activation and Deactivation Delay

FFS

- Proposal 1: The SCell activation delay requirements will not be impacted by ATG UE antenna type.

- Proposal 2: Inter-band SSB-less SCell activation requirements can apply to omnidirectional antenna type.

- Proposal 3: SCell activation requirements shall be further investigated for potential impact, considering the UE antenna types.

Issue 1-3-3: SCell Addition/Activation mechanism

FFS

- Proposal 1: Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

- Proposal 2: Further consider distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition

- Proposal 3: Should not consider distance triggered SCell addition and/or activation; existing activation methods are sufficient

Issue 1-3-4: Active BWP switch delay on multiple CCs

Agreement:

- Introduce following requirements and the legacy can be reused as baseline

  - Simultaneous DCI based BWP switch delay on multiple CCs

  - Simultaneous RRC based BWP switch delay on multiple CCs

  - Simultaneous Timer based BWP switch delay on multiple CCs

  - non-simultaneous Timer based BWP switch delay on multiple CCs

Issue 1-3-5: Pre-configured measurement gap activation/deactivation delay upon SCell activation/deactivation

Agreement:

- Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

<Sub-topic 1-4> Measurement procedure

Issue 1-4-1: Capabilities for Support of Event Triggering and Reporting Criteria

Agreement:

- For capabilities for Support of Event Triggering and Reporting Criteria, configured NR serving frequencies will include PCell and SCells carrier frequencies in the total number of NR reporting criteria for intra-frequency category.

Issue 1-4-2: Pre-configured measurement gap requirement

Agreement:

- SCell related triggering conditions should be considered for network-controlled and autonomous activation/deactivation mechanism of pre-configured measurement gap, and the legacy requirement can be reused.

Issue 1-4-3: SCell activation triggered measurement reporting requirement

Agreement:

- The measurement reporting requirement for SCell activation Triggered Reporting shall be introduced and the legacy requirements can be reused.

Issue 1-4-4: Deactivated SCell measurement

Agreement:

- beam sweeping factor N1 is needed.

- FFS on L3/L1 measurement sharing factor

Issue 1-4-5: Scheduling Availability/Restrictions of UE performing measurements on FR1 TDD band

Agreement:

- For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, the scheduling restriction extension to intra-band/inter-band CA is identified for R19 ATG

  - When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

  - When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

Issue 1-4-6: Scheduling Availability/Restrictions of ATG UE with the antenna array performing measurements on FR1

FFS

- Proposal 1: For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, the scheduling restriction extension to intra-band/inter-band CA should be identified for R19 ATG

  - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

  - For inter-band CA,

    - For case 2, case 3 and case 5, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to SCell/PCell will not apply to another serving cell in a different band.

    - If case 4 is considered, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

Issue 1-4-7: Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

FFS

- Proposal 1:

  - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

    - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

    - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

- Proposal 2:

  - For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inta-band carrier aggregation:

    - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

  - For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inter-band carrier aggregation:

    - For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

    - For ATG UE with the antenna array on PCell and one or more omni-directional antenna on SCell, there are no scheduling restrictions on FR1 SCel configured in other bands than the bands in which PCell is configured.

    - For ATG UE with the omni-directional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

    - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

    - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

Issue 1-4-8: Sharing factor between L1 measurement and RRM measurement

FFS

- Proposal 1: Reuse the legacy sharing factor Klayer1_measurement and Psharing factor between L1 and RRM measurements

Reference

[1] R4-2418275, Topic summary for [113][217] NR_ATG_enh