3GPP TSG-RAN WG4 Meeting #113	R4-2420329

Orlando, US, 18-22 Nov, 2024

Title: 	WF on HPUE RF requirements

Agenda Item:	7.1.1.1

Source: 	Samsung

Document for:	Approval

1. PC1.5 for intra-band contiguous and non-contiguous UL CA

1.1 MPR values for intra-band contiguous ULCA (with TxD)

Agreement:

- Class B would not be urgent. As common understanding, RAN4 will work on Class C in this Rel-19 WID

- Companies are encouraged to provide the MPR values for both handheld UE and FWA in next meeting

1.2 PSD assumption to derive MPR/A-MPR requirements for intra-band NC ULCA (with dualPA)

Agreement:

- Equal PSD as starting point

  - Companies are encouraged to check the MPR difference with 6dB PSD imbalance

- Inform RAN5 about the PSD condition for MPR derivation in RAN4

- The MPR difference compared to PC2 is supposed to be less than 3dB for PC1.5 in order to have performance gain over PC2

- Companies to clarify the PA configuration assumptions for MPR evaluation for PC3, PC2 and PC1.5 intra-band NC UL CA, for example, (PC3+PC3) for PC3

1.3 MPR applicability for intra-band contiguous and non-contiguous ULCA

Proposal：

  - No matter how th MPR is evaluated, equal or unbalanced PSD, the MPR specified for the total power covers unequal UL PSD and transmission bandwidths across CCs.  (Ericsson)

Agreement:

- FFS for NC CA in next meeting

- Stop the discussion for contiguous CA because equal PSD has already been agreed as assumption to derive MPR requirements

1.4 PCMAX,c and PCMAX for intra-band NCCA (with dual PA)

Proposals：

  - Proposal 1: For PCMAX, no change of current spec (ZTE)

  - Proposal 2: PCMAX,C is 26dBm   (Xiaomi)

  - Proposal 3: Proposal on PC1.5 PCmax accounting for dualPA architecture limitations and allowing up to 6dB PSD imbalance: (Skyworks)

- For a UE supporting PC1.5 intra-band non-contiguous ULCA and Signalling DualPA IE

    - PCMAX,c = 26dBm

    - PPowerClass,CA = min{29,26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dBm

    - Where RB1 and RB2 are the number of active RBs in CC1 and CC2 respectively and SCS1 and SCS2 are the SCS used in CC1 and CC2 respectively.

  - Proposal 4: (Ericsson)

- For a non-contiguous intra-band combination with PC1.5 or PC2 capability supporting dualPA-Architecture

- it is specified that the PCMAX,f,c is upper bounded by PPowerClass – 3 dB or the nominal value for PC3 whichever is the largest (in the absence of power boosting)

- the total configured output power PCMAX for the band combination is given by clause 6.2A.4.1.2 for intra-band non-contiguous CA in the current version of 38.101-1, no change for PC1.5

- Spec implementation example:

  - Proposal 5: (LGE)

- For dualPA-Architecture of PC1.5 intra-band non-contiguous UL CA, consider following condition if up to 6dB PSD imbalance is assumed for MPR requirement.

- Spec implementation example:

  - Proposal 6: (Huawei)

- A UE is not required to meet the configured maximum power per CC Pcmax,c during intra-band ULCA operations, and Pcmax,c is calculated only for PHR purpose.

- Consider Ppowerclass,c = 26dBm for DualPA, and Ppowerclass,CA=29dBm as the reference power level for MPR for both PA architectures when defining the configured max output power PCMAX requirements for PC1.5 intra-band ULCA.

Agreement:

- PCMAX,C is upper bounded by 26dBm

  - How to reflect it in spec is FFS

- For PCMAX, FFS in next meeting

  - Option 1: No change of current spec

  - Option 2: PPowerClass,CA = min{29,26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dBm assuming 6dB PSD difference ,

or, =26+10*log(1+min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2)) assuming equal PSD,

can be used as a starting point.

1.5 Relative power tolerance

Proposal：(Huawei)

  - Consider to add the equal-PSD condition as in the relative power tolerance to the configured transmitted power requirements for intra-band ULCA.

Agreement:

- FFS in next meeting

2. 2-band Inter-band UL NR-CA/EN-DC with 2Tx and/or 3Tx

  - Configurations

Agreement:

- To create and maintain a configuration table listing all configurations can be considered in Rel-19 in next meeting, for information purpose

- FFS specific MSD calculation procedure details for the band combination including PC2 FDD as a constituent band (details in section 4.1)

  - No need to enter such combination into the WID

  - Re-organize 2Tx/3Tx Notes in clause 5 and clause 6

Agreement:

- Companies are encouraged to consider the necessity and possibility to re-organize the Notes in future meetings

  - Release independence of 3Tx

Agreement:

- For FWA, Rel-17

- For handheld UE, FFS in next meeting

3. Increasing UE transmission power

  - PCMAX tolerance

Agreement:

- Modify PCMAX tolerance for inter-band NR-CA as following

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 31 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

- Provide draft CR for TS 38.101-1 (a revision of R4-2417096 which was endorsed in RAN4#112bis) in Feb meeting to reflect above agreement

4. General aspect

4.1 MSD rules

Moderator Note: It is covered by R4-2420380 (Skyworks, Murata, Nokia, Qualcomm)

4.2 Duty cycle solution for SAR compliance

Agreement:

- Introduction of new duty-cycle IE is not considered

- For ENDC (FDD+FDD), only reply on P-MPR, and there is no further spec changes needed

- Following as starting point

  - For PC1.5 of intra-band contiguous and non-contiguous CA:

for sub-clause 6.2A.1.1 and 6.2A.1.2 of 38.101-1, refer to R4-2419448

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.1-1/2 and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 23dBm or lower; - shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less t24han one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 26dBm or lower; - shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2.

  - For PC1.5 of inter-band CA:

for sub-clause 6.2A.1.3 of 38.101-1, refer to R4-2419448

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.3-1 and the supported power class enables the higher maximum output power than that of the default power class: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or – if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38. 306 (The exact evaluation period is no less than one radio frame); or – if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 26dBm or lower; – shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else; – shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 50% (DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, represent the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306, if UE supports power class 1.5. maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.306, otherwise. For NR Band x or NR Band y, – if power class of band x in the band combination is power class 1.5 and the corresponding UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent; – the corresponding maxDutyNR,x is equal to 0.5*maxUplinkDutyCycle-PC2-FR1 – else if the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x is equal to 25%; – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

  - For EN-DC (FDD+TDD)

for sub-clause 6.2B.1.3 of 38.101-3 refer to R4-2418744

- If a UE supports a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class: - If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2: - – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the power class 2, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or - – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the power class 2, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or - – shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2H.4. - – else– if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or - – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame) - – shall apply all requirements for the supported power classpower class 2 and set the configured transmitted power class as specified in sub-clause 6.2H.4. - – else - – shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2H.4; - else - – shall apply all requirements for the supported power class and set the configured transmitted power as specified sub-clause 6.2H.4;

  - For EN-DC(TDD+TDD)

for sub-clause 6.2B.1.3 of 38.101-3 (similar as R4-2418744)

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class: – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4; – Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame): – shall apply all requirements for the power class 2supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4. Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame): – shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.