3GPP TSG-RAN WG4 Meeting #113															R4-2420333

# Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.3.4

Source:	OPPO, LGE

Title:	WF on NR sidelink intra-band NC CA

Document for:	Approval

Introduction

This WF captures most the agreements for NR SL intra-band non-contiguous CA.

WF agreements

Sub-topic 1-1 A-MPR requirement

- Issue 1-1-1: A-MPR simulation cases for intra-band NCCA with NS_33

Agreement:

- Evaluate 10+10MHz channel bandwidth combination

WF agreement:

- Use 5855MHz to 5925MHz with 7 channels for A-MPR channel configuration.

- Issue 1-1-2: A-SEM requirements for intra-band NCCA with NS_33

- Proposals:

  - Proposal 1: For intra-band NCCA, the A-SEM of two non-contiguous 10MHz carriers, the overlapping part of the A-SEM for each carrier will use the higher limit as the combined A-SEM.

  - Proposal 2: For intra-band CCA A-SEM, only consider the A-SEM of one side of the 10MHz carrier.

  - Observation 1: Spectrum mask is defined for 10MHz channel bandwidth only

  - Observation 2: Spectrum mask is only applicable within 5855 to 5925MHz.

- Moderator WF:

Further discuss and agree on the A-SEM for A-MPR simulation

  - Apply the NR NCCA SEM principle

Agreement:

  - Apply the NR NCCA SEM principle

- Issue 1-1-3: A-SE requirements for intra-band NCCA with NS_33

- Proposals:

  - Observation 1: The out-of-band domain is defined as ±250 % of the channel bandwidth, and then the out-of-band (OOB) domain for the 5 GHz ITS frequency band is 5 835 MHz to 5 855 MHz at the lower part and 5 925 MHz to 5 945 MHz at the higher part of the frequency band.

  - Observation 2: Maximum EIRP power density for UE emissions into the out-of-band domain is -30dBm/MHz.

  - Proposal 1: To use the A-SE limit to derive the derive the A-MPR requirement.

WF agreement:

- The A-SEM requirement applies only within 5855 MHz to 5925 MHz according to ETSI EN 302 571.

- The A-SE requirement applies as shown in Table below and according to ETSI EN 302 571.

| Protected band | Frequency range (MHz) |  |  | Maximum Level (EIRP) | MBW (MHz) |
| --- | --- | --- | --- | --- | --- |
| Frequency range | 5835 | - | 5855 | -30 | 1 |
| Frequency range | 5925 | - | 5945 | -30 | 1 |

Sub-topic 2-1 PC3 non-contiguou CA MPR

- Issue 2-1-1: PSSCH/PSCCH with 1x23dBm PA + 1LO

- Proposals

  - Proposal 1 (LGE):

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.0 | ≤ 16.5 |
| 5.04 ≤ B < 10.08 | ≤ 9.0 | ≤ 15,5 |
| 10.08 ≤ B |  | ≤ 15.0 |

  - Proposal 2 (OPPO):

    - For -30dBm/MHz IM3:

    - For 1PA 1LO architecture:

    - 3.6≤B＜9	11

    - 9≤B＜13.5	10.5

    - 13.5≤B		8

WF agreement:

Table 1. PSSCH/PSCCH MPR for SL non-contiguous CA with 1x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ [10.0] | ≤ 16.5 |
| 5.04 ≤ B < 10.08 | ≤ [9.0] | ≤ 15,5 |
| 10.08 ≤ B |  | ≤ 15.0 |

- Issue 2-1-2: PSSCH/PSCCH with 2x20dBm PA + 1LO

- Proposals

  - Proposal 1 (LGE):

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.5 | ≤ 17.0 |
| 5.04 ≤ B < 10.08 | ≤ 9.5 | ≤ 16.0 |
| 10.08 ≤ B |  | ≤ 15.5 |

  - Proposal 2 (OPPO):

    - For -30dBm/MHz IM3:

    - For 2PA 1LO architecture:

- 3.6≤B＜9	13

- 9≤B＜13.5	12

- 13.5≤B		11

WF agreement:

Table 2. PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ [10.5] | ≤ 17.0 |
| 5.04 ≤ B < 10.08 | ≤[ 9.5] | ≤ 16.0 |
| 10.08 ≤ B |  | ≤ 15.5 |

- Issue 2-1-3: PSSCH/PSCCH with 2x20dBm PA + 2LO

- Proposals

  - Proposal 1 (LGE):

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 6.5 | ≤ 13.5 |
| 5.04 ≤ B < 10.08 | ≤ 6.5 | ≤ 12.5 |
| 10.08 ≤ B < 20.16 |  | ≤ 11.5 |
| 20.16 ≤ B |  | ≤ 10.5 |

  - Proposal 2 (OPPO):

For 2PA 2LO architecture:

3.6≤B＜9	11

9≤B＜13.5	10

13.5≤B		8

WF agreement:

Table 3. PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ [6.5] | ≤ 13.5 |
| 5.04 ≤ B < 10.08 | ≤ [6.5] | ≤ 12.5 |
| 10.08 ≤ B < 20.16 |  | ≤ 11.5 |
| 20.16 ≤ B |  | ≤ 10.5 |

Sub-topic 2-2 PC2 non-contiguou CA MPR

- Issue 2-2-1: PSSCH/PSCCH with 1x26dBm PA + 1LO

- Proposals

  - Proposal 1 (LGE):

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.0 | ≤ 18.5 |
| 5.04 ≤ B < 10.08 | ≤ 10.0 | ≤ 17.0 |
| 10.08 ≤ B |  | ≤ 16.0 |

  - Proposal 2 (OPPO):

- For 1PA 1LO architecture:

- 3.6≤B＜9	14

- 9≤B＜13.5	11

- 13.5≤B		10

WF agreement:

Table 4. PSSCH/PSCCH MPR for SL non-contiguous CA with 1x26dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ [11.0] | ≤ 18.5 |
| 5.04 ≤ B < 10.08 | ≤ [10.0] | ≤ 17.0 |
| 10.08 ≤ B |  | ≤ 16.0 |

- Issue 2-2-2: PSSCH/PSCCH with 2x23dBm PA + 1LO

- Proposals

  - Proposal 1 (LGE):

- Table 2-11 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.5 | ≤ 19.0 |
| 5.04 ≤ B < 10.08 | ≤ 11.0 | ≤ 17.5 |
| 10.08 ≤ B |  | ≤ 16.5 |

  - Proposal 2 (OPPO):

- For 2PA 1LO architecture:

- 3.6≤B＜9	16.5

- 9≤B＜13.5	12.5

- 13.5≤B		11.5

WF agreement:

Table 5. PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ [11.5] | ≤ 19.0 |
| 5.04 ≤ B < 10.08 | ≤ [11.0] | ≤ 17.5 |
| 10.08 ≤ B |  | ≤ 16.5 |

- Issue 2-2-3: PSSCH/PSCCH with 2x23dBm PA + 2LO

- Proposals

  - Proposal 1 (LGE):

- Table 2-12 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 7.0 | ≤ 15.0 |
| 5.04 ≤ B < 10.08 |  | ≤ 13.0 |
| 10.08 ≤ B < 20.16 |  | ≤ 12.0 |
| 20.16 ≤ B |  | ≤ 10.0 |

  - Proposal 2 (OPPO):

- For 2PA 2LO architecture:

- 3.6≤B＜9		12

- 9≤B＜13.5	11

- 13.5≤B		10

WF agreement:

Table 6. PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ [7.0] | ≤ 15.0 |
| 5.04 ≤ B < 10.08 |  | ≤ 13.0 |
| 10.08 ≤ B < 20.16 |  | ≤ 12.0 |
| 20.16 ≤ B |  | ≤ 10.0 |

Online agreements

For intra-band NC CA:

Issue 3-1-1: PC2 PSFCH MPR

Agreement:

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 2LO | ≤ 9.5 | ≤ 8.5 | ≤ 7.5 | ≤ 16.5 | ≤ 16.0 | ≤ 15.0 |

Issue 3-1-2: PC3 PSFCH MPR

Agreement:

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x23dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x20dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x20dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 7.0 | ≤ 16.0 | ≤ 15.5 | ≤ 15.0 |

Issue 4-1-1: PC2 S-SSB MPR

Agreement:

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x26dBm + 1LO | ≤ 13.0 | ≤ 12.0 | ≤ 22.0 | ≤ 20.0 |
| 2x23dBm + 1LO | ≤ 14.0 | ≤ 13.0 | ≤ 23.0 | ≤ 21.0 |
| 2x23dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 17.0 | ≤ 15.0 |

Issue 4-1-2: PC3 S-SSB MPR

Agreement:

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x23dBm + 1LO | ≤ 12.0 | ≤ 10.5 | ≤ 20.0 | ≤ 18.0 |
| 2x20dBm + 1LO | ≤ 13.0 | ≤ 11.5 | ≤ 21.0 | ≤ 20.0 |
| 2x20dBm + 2LO | ≤ 8.5 | ≤ 7.5 | ≤ 15.0 | ≤ 14.0 |

For intra-band C CA:

Issue 1-1-1: PSFCH MPR

Agreement:

|  | MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | R ≤ 0. 3 | 0.3 < R ≤ 0. 5 | 0.5 < R ≤ 1.0 |
| 1x26dBm+1LO | ≤ 5.0 | ≤ 10.0 | ≤ 14.0 |
| 2x23dBm+1LO | ≤ 6.0 | ≤ 11.0 | ≤ 15.0 |

Issue 1-1-2: S-SSB MPR

Agreement:

|  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | Inner | Outer1 | Outer2 |
| 1x26dBm + 1LO | ≤ 4.0 | ≤ 9.5 | ≤ 14.0 |
| 2x23dBm + 1LO | ≤ 4.5 | ≤ 10.0 | ≤ 14.5 |