3GPP TSG-RAN4 Meeting #113  	                                                                     R4-2420342

Orlando, US, 18th – 22nd November, 2024

Agenda item:			6.1

Source:	CMCC

Title:	WF on HPUE for NR bands

Document for:	Approval

Introduction

In Rel-18, companies conducted research on n26 support for PC2. Since there was no consensus among companies regarding the results of A-MPR, the relevant outcomes have all been captured in TR 38.896. For Rel-19, we need to conduct further research on this issue.

Way forward on PC2 1Tx/2Tx A-MPR:

FDD band PC2 A-MPR requirement will be captured in the single table to accommodate 1Tx and 2Tx.

- One set of requirements to cover 1Tx and 2Tx based on the worst case

- The PC2 A-MPR may not exceed the PC3 A-MPR by more than 3dB.

Way forward on Band n26 A-MPR

There are four A-MPR requirements that need to be defined for n26 PC2, specifically NS-12, NS-13, NS-14, and NS-15.

- The A-MPR for these NS values need to be defined based on the analysis results from TR 38.896 and any additional relevant input.

- Companies are encouraged to provide further analysis on NS-12, NS-13, and NS-15 with 3 MHz channel bandwidth.

- Companies are encouraged to analyze the impact of A-MPR when supporting 2Tx.

Reference

[1] R4-2410561, “TP for TR 38.896 to add PC2 for n26”, China Unicom

[2] TR 38.896