3GPP TSG-RAN WG4 Meeting # 113                                                           R4-2420362

Orlando, US, 18th – 22nd November, 2024

Title: 	                 TP to TR 38.769 for A-IoT BS TX requirements

Source: 	CATT

Agenda item:	7.21.3.1

Document for:	Approval

- Introduction

This contribution provides a TP to TR 38.769 for A-IoT BS TX requirements. The agreements of TX requirements in the WF [2] are included in the TP.

Reference

[1]TR 38.769, V1.0.0

[2] R4-2417088, “WF on impacts of A-IoT on RF requirements”, Huawei, RAN4#112bis

TP for TR 38.769

<Start of Text Proposal>

### 6.7.2	Ambient IoT BS

      - General

For Release 19, the RF requirement discussion focus on BS type 1-C for Ambient IoT indoor scenarios. The RF requirement for ambient IoT BS is studied based on the assumption that ambient IoT BS only transmits and receives ambient IoT signal.

Current the coexisting scope focuses on device 1 and device 2a, it includes the scenarios and cases which describes the different factors to be considered for the ambient IoT BS. These factors are listed in Table 6.7.2-1 below. CW signal can be transmitted by A-IoT BS if manufacture declares this capability. CW can be transmitted by a separate node, therefore CW related RF requirements can be specified separately for ambient IoT BS or a separate CW node.

Table 6.7.2-1: Factors impacting the ambient IoT BS operation to device 1 and device 2a

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | A-IoT BS transmitting the CW signal |
| --- | --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum | Yes |
|  | DL spectrum | DL spectrum | DL spectrum | No |
|  | DL spectrum | UL spectrum | UL spectrum | Yes |
|  | DL spectrum | UL spectrum | UL spectrum | No |

For the scenarios only considering device 1 and device 2a, simultaneously CW and R2D transmission will interfere the R2D reception, as such the R2D and D2R transmission should be operated in TDM for device 1 and device 2a.

      - Ambient IoT BS TX requirements

6.7.2.1.1 Base station output power

Base station output power requirement is potentially needed, RAN4 agrees to use current requirements of Micro NR BS type 1-C as the starting point.

6.7.2.1.2 Output power dynamic

Output power dynamic requirement usually includes RE power control dynamic range requirement and total power dynamic range requirement. Power boosting requirement is not needed in R19 with the assumption that ambient IoT BS only transmits and receives AIOT signal.

6.7.2.1.3 Transmit ON/OFF power

Regarding the transmit on/off requirement, it usually includes of transmitter off power requirement and transmitter transient period requirement, RAN4 agrees to further discuss in the WI phase.

6.7.2.1.4 Transmit signal quality

Frequency error and modulation quality (EVM) are potentially needed. TAE is only applicable for BS transmitting from multiple antennas via MIMO, CA, or combination of them. A-IoT BS doesn’t support these features, so TAE requirement is not needed.

6.7.2.1.5 Unwanted emissions

Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement are potentially needed.

<End of Text Proposal>