3GPP TSG-RAN WG4 Meeting#113                                  R4-2420363

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.21.3.1

Source: 	ZTE Corporation, Sanechips, Ericsson

Title: 	TP to TR38.769: Ambient BS Rx requirements

Document for:	Approval

Introduction

Based on the agreement reached in the previous RAN4 meetings. In this contribution, we would like to provide TP to TR38.769: Ambient BS Rx requirements.

Text proposal

## <<Start of Change>>

6.7.2.2 Ambient IoT BS -RX

6.7.2.2.1 Reference sensitivity level

Regarding the reference sensitivity level for A-IoT BS, RAN4 reached the following consensus:

- For BS type 1-C,

PREFSENSdBm=−174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

-	BW is the transmission bandwidth for D2R.

-	NF is the noise figure of A-IoT BS, equal to 10 dB for Medium Range BS.

-	IM is the implementation margin, equal to 2dB.

-	SNR is the SNR value for D2R decoding, the detailed metrics would be discussed in the WI phase.

-	Desens target is considered for CW signal interference cancellation in D1T1 scenario. The desens level for CW signal cancellation might be different for D1T1-A1/D1T1-A2 and D1T1-B. For D1T1-A2, CW signal interference is coming from self-interference and also potential CW interference from other nodes transmitting CW signal. For D1T1-A1, the CW signal interference is coming from other nodes transmitting CW inside the topology. For D1T1-B, the CW signal interference is coming from CW nodes transmitting outside topology.

6.7.2.2.2 Dynamic range

For NR and LTE, receiver dynamic range requirement is to ensure that the base station can receive high throughput also in the presence of increased interference levels. The interference level is derived from simulation of co-channel interference from the same operator. It measures the effects of base station receiver impairments. For Ambient IoT indoor deployment, the co-channel interference comes from co-channel device transmission and CW interference. The necessity and the value can be studied at WI phase.

6.7.2.2.3 In-band selectivity and blocking

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study and wanted signal configuration and its corresponding configuration could be postponed to WI phase.

- In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase.

6.7.2.2.4 Out-of-band blocking

Regarding Out-of-band blocking requirement, the existing out-of-band blocking requirement may be still applicable for A-IoT BS except for the wanted signal level.

6.7.2.2.5 Receiver spurious emissions

Regarding the receiver spurious emission requirement, this requirement might differ as following:

- For D1T1-A1 and D1T1-B, the receiver spurious emission requirement is still needed due to is half duplex operation mode.

- For D1TA-A2, due to the simultaneous transmission of CW signal and D2R reception, if the spurious emission requirement of CW signal is exceeding the receiver spurious emission requirement, then the receiver spurious requirement is not needed similar as FDD BS otherwise this requirement is still needed to regulate the receiver RF performance.

6.7.2.2.6 Receiver intermodulation

- Regarding the general intermodulation requirement, it is agreed to postpone to WI phase due to the potential relationship between D2R operation assumption e.g. single tone or multiple tone D2R assumption.

- Regarding the narrowband intermodulation requirement, it is potentially necessary due to the potential Narrow band blocking at the operating frequency range, however its detailed requirement could be postponed to WI phase.

6.7.2.2.7 In-channel selectivity

Regarding the receiver in-channel selectivity requirement, this is agreed to be discussed in WI phase.

## <<End of Change>>