3GPP TSG-RAN WG4 Meeting # 113												R4-2420365

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo, Huawei

Title: 	TP on the intermediate UE Tx RF requirement

Agenda Item:	7.21.3.3

Document for:	Approval

- Introduction

In the previous meeting, the RF requirements for the intermediate UE were discussed, and this contribution summarize the conclusion so far.

- Text proposal

## <<Start of Change>>

6.7.3	Intermediate node (UE)

6.7.3.1 General

The intermediate UE, under NW control, can send Ambient IoT data and/or signalling to the device, and receive the backscatter signal from the device. The intermediate UE can also operate as a legacy NR UE. For simplicity, non-concurrent operation of Ambient IoT and NR for intermediate UE is assumed, and focus on the case where the intermediate UE acts as an A-IoT Reader.

Considering the intermediate UE need to support both A-IoT and NR operation and if the requirements in NR and A-IoT is the same, it is possible to verify only on the NR side.

6.7.3.2 TX

### 6.7.3.2.1 Maximum output power

The goal of the maximum output power is to achieve the DL coverage. To reuse of the legacy NR UE hardware, PC3 NR UE can serve as a baseline for intermediate UE using FDD UL spectrum as R2D spectrum. High power UE is not precluded in the future release for better coverage.

### 6.7.3.2.2 Output power dynamics

Power control is not designed for A-IoT R2D link currently, and intermediate UE transmit at maximum output power for better coverage, making a minimum output power and power control accuracy requirements unnecessary. However, during the co-existence study, some companies propose that implementing the power control can help intermediate UE to mitigate the interference to NR system, so this requirement will be further discussed in WI phase depending on RAN1 specification.

Transmit OFF power is defined to ensure the UE will not affect the reception of other UE or BS when the transmitter is inactive. Since for A-IoT standalone operation, the HD-FDD mode could be one option for outside topology and the requirement can be further discussed in WI phase.

For the transmit ON/OFF time mask, refer to the RFID specification, a settling time requirement is used to assess signal ripple characteristics. It can be determined whether or not this settling time is necessary in WI phase. For the intermediate node that has capability to transmit CW signal, the transient time between R2D to CW transmission also needs to be further discussed.

### 6.7.3.2.3 Transmitted signal quality

Frequency error mainly depends on the timing and synchronization performance and PLL performance within transceiver chain. A-IoT intermediate UE is expected to reuse the existing timing and synchronization network directly. Meanwhile, similar PLL performance of A-IoT intermediate UE as NR UE is also expected as they are operating at the same frequency range. With the above considerations, frequency error requirement of NR UE can be considered reusing for A-IoT intermediate UE.

For legacy NR UE，transmit modulation quality includes EVM, carrier leakage and In-band emissions. These requirements may still be needed as R2D waveform is based on OFDM transmitter.

The carrier leakage requirement intends to reduce the DC offset which will distort the constellation for QAM signal. For OOK signal, the DC offset will increase the amplitude of waveform and the carrier leakage should also be low enough to ensure the received signal still within the receiver dynamic range.  Whether and how to define this requirement can be further discussed in WI phase

The in-band emission in NR can limit the interference between UEs, but when intermediate UE perform as A-IoT reader, this requirement is unnecessary if all RB within the channel bandwidth always allocated for A-IoT transmission.

### 6.7.3.2.4 Output RF spectrum emissions

Output RF spectrum emissions for legacy NR UE includes occupied bandwidth, out of band emission (ACLR), spurious emissions and transmit intermodulation.

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage β/2 (=0.5%) of the total mean transmitted power. Occupied bandwidth: Potentially needed for intermediate UE. The specific value is dependent on the physical layer design of R2D, which has not yet been finalized at this stage.

ACLR can be defined based on the co-existence study. During SI co-existence evaluation, the IBE of legacy NR is used as emission leakage model, and performance degradation is observed in the results, so the ACLR better than IBE may be required, and the details can be determined in WI phase.

Existing spurious emission requirement for legacy NR UE can be applicable, since this is coming from the ITU-R recommendation SM.329.

The transmit intermodulation in NR is the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna. this requirement can be further discussed in WI.

6.7.3.3 CW for D2T2

### 6.7.3.3.1 Output power/Output power dynamic range

The output power of CW is needed to ensure the link quality for D2R. For the intermediate node that has capability to transmit CW signal, the MPR/A-MPR needs to be studied as the CW waveform defined with unmodulated sinusoid waveform is new waveform.

Considering the CW and R2D cannot transmitting simultaneously, the output power dynamic related requirements, e.g., transmit OFF power, can be further discussed in WI phase.

### 6.7.3.3.2 Transmit signal quality

Since the D2R signal transmission for device1/2a closely rely on the CW, and the signal quality may need to be considered e.g., frequency error which needs more discussion.

### 6.7.3.3.3 RF spectrum emission/Spurious emissions/Phase noise

The emission related requirement needs to be specified. Meanwhile, close-in phase noise may drown the D2R signal. CW cancellation can provide help in the CW inside topology scenario, but the phase noise might play a critical role for CW outside topology. In addition, the regulation is also needed to be considered. Therefore, the details of how to specify RF spectrum emission, and whether and how to specify spurious emission and phase noise can be further discussed in WI phase.

## <<End of Change>>

- References

[1] R4-2417088, WF on impacts of Ambient IoT on RF requirements, Moderator (Huawei), RAN4#112bis, Oct.,2024