# 3GPP TSG-RAN WG4 Meeting #113	R4-2420368

Orlando, United States, 18th – 22th November, 2024

Agenda item:	7.21.3.2

Source: 	Xiaomi

Title: 	TP for TR 38.769 on RF requirements for device 2a

Document for:	Approval

Introduction

This TP to TR 38.769 is to capture RF requirements for device 2a based on the agreements and conclusions in previous RAN4 meetings.

Text proposal

<Start of text proposal>

## 6.7	RF requirements study

### 6.7.4	Ambient IoT Device

#### 6.7.4.1	Device 1

#### 6.7.4.2	Device 2a

- 6.7.4.2.1	Transmitter characteristics

6.7.4.2.1.1	Transmitter power

For device 2a, the transmitter architecture is based on backscattering technology, and the backscattering power or power backscattering loss is potentially needed to be defined to ensure the coverage target. The maximum output power is potentially not needed.

The minimum backscattered modulated power can be specified for device 2a. The backscatter power is the amount of power reflected back from the device 2a when a successful command sequence is completed at the device at a specific sensitivity level.

The backscatter loss due to the modulation can be defined as scattering cross-section or modulation efficiency in equation (1). It can be observed that the modulation efficiency is defined as the ratio of the power radiated by the device to the power density incident on its antenna.

The backscatter loss or cross section of the device 2a can be defined as

σ=PbackscatterS=Ra2Za+Zc1,c22  ∙ λ2∙Gd2π 			(1)

The received backscattered signal at the test device:

Preceived=Pbackscatter∙Gr4πr2=Pt∙Gt4πr2 ∙Gr4πr2∙σ 				(2)

Where Pbackscatter is the power backscattered back in the direction of the transmitter defined in clause 6.7.4.1.1, and S is the power density of the incident radio wave on the device antenna defined in clause 6.7.4.1.1.

6.7.4.2.1.2	Output power dynamic

For device 2a, power control is not designed for backscattering transmission, and RAN4 concluded that all the output power dynamic requirements are not relevant for backscattering type devices.

- Transmit OFF power: when there is no incoming CW signal, the device 2a does not emit any high-power level. The requirement of transmit OFF power is potentially not needed.

- Transmit ON/OFF time mask: The time mask requirement is to test the ON/OFF time period for RF chain. For device 2a, this time period is not crucial and it is potentially not needed.

- Power control: Power control is not designed for backscattering transmission, the power control requirement is potentially not needed for device 2a.

6.7.4.2.1.3	Output RF spectrum emissions

- Occupied bandwidth: The occupied bandwidth is the width of a frequency band containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. Occupied bandwidth is potentially needed for Device 2a. The specific value is dependent on the physical layer design of D2R, which has not yet been finalized at this stage.

- ACLR: According to the co-existence simulation, the ACLR requirement is potentially not needed for device 2a.

- 6.7.4.2.2	Receiver characteristics

6.7.4.2.2.1	Reference sensitivity

The reference sensitivity is the minimum mean power applied to device antenna port to guarantee the receive performance. The requirement of reference sensitivity for device 2a is potentially needed.

6.7.4.2.2.2	Adjacent channel selectivity (ACS)

For device 2a, the RF BPF is not assumed in RF architecture for the reason of cost and size consideration, So the ACS requirement which relies on digital filter is potentially not needed.

6.7.4.2.2.3	Adjacent Subcarrier selectivity (ASCS)

Same as ACS, potentially, no need to define ASCS.

6.7.4.2.2.4	In-band blocking

Same as ACS, potentially, no need to define in-band blocking requirement.

6.7.4.2.2.5	Out-of-band blocking

Same as ACS, potentially, no need to define out-of-band blocking requirement.

6.7.4.2.2.6	Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. The received power comes from R2D signal. For device 2a, LNA is not assumed in RF architecure. Hence, there is potentially no need to define spurious emissions.

6.7.4.2.2.7	Spurious response

Same as ACS, there is potentially no need to define spurious response.

6.7.4.2.2.8	Intermodulation Characteristic

Same as ACS, there is potentially no need to define intermodulation characteristic.

<End of text proposal>