3GPP TSG-RAN4 Meeting #113R4-2420370	R4-2420370

# Orlando, US, 18th – 22nd November, 2024

Title:	TP for TR: Testability aspects of A-IoT device

Source:	Qualcomm Incorporated, Sony

Agenda item:	7.21.3.2

Release:	Rel-19

SI: 	FS_Ambient_IoT_solutions

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

In RAN4#112bis an agreement for testability aspect was agreed as below. In this paper we provide a TP for testability aspects.

Issue 3-5: testability - Take OTA test as the baseline for at least Devices 1 and 2a - FFS on the side conditions in the OTA test setup including CW signal and RF energy source, etc.

# 2. 	Text Proposal to TR 38.769

<Start of Text Proposal>

#### 6.7.4.4 Testability

##### 6.7.4.4.1 Testability of device

Based on the architecture described in clause 5, with such highly integrated architectures for device 1/2a/2b, conducted antenna connectors are assumed not to be available. Therefore, OTA testing is considered as the baseline testing methodology for device 1, device 2a and device 2b RF verification. Far-filed measurement system in an anechoic chamber could be considered as the baseline setup.

To be fully functional, A-IoT devices require a dedicated energy source for harvesting to be provided during the test. According to the SI description, the devices under consideration are either battery-less or with limited energy storage capability (i.e., using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other power source that could be seen suitable. Note that the following description on the energy harvesting is just an example to illustrate the testability issues for Energy Harvesting. It doesn’t mean to define the Energy Harvesting procedure. Device goes from OFF state to ON state when it detects energy and has been able to accumulate sufficient amount of energy for the parts to function. This Energy Harvesting Sensitivity (EHS) is the value when device starts to harvest energy. Device behaviour principle with non-constant energy availability is shown in Figure 6.7.4.4.2-1.

Figure 6.7.4.4.2-1. An example of energy harvesting principle

Energizing signal is a trigger for device to turn on. This is specifically for device type 1 and 2a. After device turns on, it needs to monitor the A-IoT R2D signal and device should respond to network in test environment once the signal from network is detected successfully. Device may also decide to sleep or continue monitoring.

In testing phase, for a device that harvests RF energy, tester must provide sufficient RF waveform electrical field density to the device to ensure it is turned on and time gate the RF testing to the ON periods of the A-IoT device. An external antenna may be needed to provide the CW and also provide RF energy, while a separate testing antenna is also needed, which are illustrated in Figure 6.7.4.1.2-2.

Figure 6.7.4.1.2-2. A possible OTA test setup with CW and RF energy source

Therefore, RAN4 needs to define a list of testable requirements in the WI phase and also the time domain pattern for the tests.

##### 6.7.4.4.2 Summary of testability issues

RAN4 take OTA as the baseline testing methodology for device 1, device 2a and device 2b RF verification. The following test setup issues can be discussed and decided in the WI phase: power level of RF energy source and CW signal, duration time of harvesting, ON-OFF testing pattern, isolation between the CW antenna and the testing antenna, etc.

<End of Text Proposal>