3GPP TSG-RAN WG4 Meeting #113	R4-2420380

Orlando, US, 18-22 Nov, 2024

Title: 	WF on MSD rule

Agenda Item:	7.1.1.1

Source: 	Skyworks, Murata, Nokia, Qualcomm

Document for:	Approval

# 1. Background

The following online agreements were reached based on the contributions summarized in [1].

Agreement:

- Work on the feasible unified Equations/LUT/Simplified guidelines for MSD for HPUE band combinations as normative requirements

  - Targeting at introduction of unified Equations/LUT/Simplified guidelines to replace the existing method using HPUE MSD tables at the end of Rel-19 or earlier if agreed.

  - The existing MSD requirements for band combinations for HUPE won’t be changed.

  - Discuss whether the request procedure for HPUE band combinations should be kept

- For Rel-19, follow tradition procedure, including the request of HPUE band combinations, until the work on the feasible unified Equations/LUT/Simplified guidelines is finished.

This WF captures the guidelines and assumptions to provide 1UL and 2UL HPUE MSD simplification rules.

# 2. WF on 1UL HPUE MSD simplification

Background:

Figure 1 summarises the proposed 1UL HPUE (PC2 and PC1.5) delta MSD curves vs. the legacy Rel-18 MSD test points (“+” dots). Figure 1 left shows that three PC2 rules were presented at this meeting:

- A single 3dB PC2 de-rating from the agreed PC3 MSD (Fig.1 left, light blue line [2]),

- A single PC2 look-up table with plateau at ~ 6dB (Fig.1 left, orange curve [3]),

- One PC2 LUT to specify PC2 1Tx MSD, another PC2 LUT to specify the PC2 2Tx MSD (Fig.1 left, dark blue and green curves [4,5]). These curves assume 9dB 1Tx PC3 interference imbalance.

Figure 1: Agreed Rel-18 MSD test points plotted with “+”. Proposed PC2 1Tx/2Tx (left), PC2 1.5 (2Tx - right) [2,3,4,5].

It can be observed that at the end of Rel-18, the 1UL PC1.5 MSD may be smaller than the 1UL PC2 2Tx MSD [6]. This inconsistency may be accounted for different assumptions on the 1Tx PC3 interference Main/Diversity imbalance when the 1UL PC1.5 MSD were specified in Rel-17 vs the assumptions used in Rel-18 for 1UL PC2 2Tx MSD.

Reminder on 1Tx PC3 interference imbalance: The 1Tx Main/Div interference is the level of interference which corresponds to the agreed MRC combined PC3 MSD level [6]. In most TP for TRs, 10dB antenna to antenna isolation results in 9dB Main/Div interference imbalance. An example of PC2 Tx MSD calculation based on the 1Tx PC3 interference level is reproduced from [6] in Figure 2.

Figure 2: Example of PC2 1Tx / 2Tx MSD evaluation assuming 1Tx PC3 9dB interference imbalance for CA_n25-n77 [6].

The impact of the 1Tx Main/Div interference imbalance onto the maximum “plateau” value reached by PC2_2Tx is reproduced from [6] in Figure 3. The PC2_2Tx plateau reaches 3dB, ~5.5dB and ~8dB for 0dB, 6dB and 9dB imbalance respectively.

Figure 3: Impact of 0dB, 6dB, 9dB 1Tx Main/Div interference imbalance on PC2_2Tx. The impact of 0.5dB reverse IMD is plotted in dashed lines, and dots capture some of the agreed Rel-18 MSD PC2 1Tx test points as reference.

Agreement:

- For new Rel-19 inter-band CA combinations within FR1, and for MSD due to harmonic, receiver harmonic mixing and cross-band isolation interference, the 1UL HPUE MSD requirements are specified using a delta MSD relative to the default power class 3 MSD as:

PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5,

  - Companies are invited to evaluate two options,

- option 1: RAN4 keeps on specifying two sets PC2 requirements [4,5]:

  - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB,

  - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB,

  - option 2: RAN4 specifies a single set PC2 requirement [3]:

    - one PC2 LUT/equation PC2 MSD = PC31Tx MSD + PC2, e.g. plateau at ~6dB.

- To ensure that 1UL PC1.5 MSD is greater than 1UL PC2 2Tx MSD, companies are invited to derive the HPUE delta MSD LUT/Equations values assuming:

  - a [9dB] 1Tx Main/Div interference imbalance. Imbalance assumptions less than 9dB are not precluded,

  - [0.5]dB RevIM (Rtx) and 3dB interferer increase for harmonic/harmonic mixing and for crossband isolation. .

- Handling of band combination without MSD for PC3 but with small or moderate PC2/PC1.5 MSD:

  - option 1: FFS LUT rule and FFS on test point configuration

  - option 2: Legacy HPUE MSD tables may still be used to capture an exception MSD test point

  - option 3: no HPUE MSD requirement for those cases.

- For proponents of LUT simplification approach, FFS the LUT granularity (e.g. 1dB steps)

# 3. WF on 2UL HPUE MSD simplification

Background:

Up to ten different UL-CA (2UL) power class configurations have been identified for Rel-19 [7].

| case # |  | 1 Baseline | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.5 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |

Agreement:

- Companies are invited to

  - Provide 2UL HPUE MSD simplification rules to cover the Rel-19 supported 2UL configurations using either equations/LUT or single value de-ratings relative to the default power class 3 MSD,

  - Study means to minimize the number of LUTs / set of equations that cover all cases.

  - Check if all Rel-19 cases have been correctly captured and to look at the initial proposals in [4] and [7].

  - FFS if only IMD orders are sufficient to design LUT/ set of equations or if both the IMD order and the IMD indexes should be accounted for.

  - FFS to account for other IMD contributors than just the power amplifier.

# 4. References

- R4-2418139 Topic summary for [113][117] NR_ENDC_RF_Ph4_part2_1115_Final, 3GPP TSG-RAN WG4 Meeting #113, Samsung.

- R4-2417999 MSD requirement for new PC2/PC1.5 band combinations, Nokia.

- R4-2419082 MSD for HPUE, 3GPP TSG-RAN WG4 Meeting #113, Qualcomm France.

- R4-2417784 Further discussion on HPUE MSD rules, 3GPP TSG-RAN WG4 Meeting #113, Murata Corporation.

- R4-2419699 1UL HPUE NRCA MSD Simplifications for Rel-19, 3GPP TSG-RAN WG4 Meeting #113, Skyworks Solutions, Inc.

- R4-2419712 1UL HPUE NRCA MSD Observations, 3GPP TSG-RAN WG4 Meeting #113, Skyworks Solutions, Inc.

- R4-2417794 Simplification of HPUE 2UL IMD MSDs, 3GPP TSG-RAN WG4 Meeting #113, Skyworks Solutions, Inc.