3GPP TSG-RAN WG4 Meeting #113															R4-2420386

Orlando, FL, USA, 18th – 22nd November, 2024

Agenda item:			7.2.4

Source:	Samsung

Title:	TP on coexistence simulation results for 14800 to 15350 MHz frequency range

Document for:	Approval

1	Introduction

This document is a text proposal to provide a draft TP structure for 15 GHz coexistence simulation results, and to capture final agreements achieved in this meeting.

2	Text Proposal

-------------------------------------------- TP BEGIN --------------------------------------------

### 6.1.3	Co-existence simulation results

#### 6.1.3.1	Urban macro scenario

##### 6.1.3.1.1	Downlink

For Urban macro scenarios, simulation results for DL throughput loss with baseline assumption ACLR = 45 dBc at BS and ACS = 33 dBc at UE are summarized in Table 6.1.3.1.1-1/2/3/4/5.

Table 6.1.3.1.1-1: DL throughput loss for 14800 – 15350 MHz [Relative ACIR value]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia (R4-2417536) | Average throughput loss in % (15 GHz) | 0.17 |  | 0.28 |  | 0.45 |  | 0.70 |  | 1.06 | 1.55 |
|  | 5%-tile throughput loss in % (15 GHz) | 0.00 |  | 0.18 |  | 0.35 |  | 1.00 |  | 1.38 | 3.53 |
| ZTE | Average throughput loss in % (15 GHz) | 0.33 | 0.41 |  | 0.52 | 0.67 | 0.85 |  | 1.09 |  |  |
|  | 5%-tile throughput loss in % (15 GHz) | 2.50 | 2.75 |  | 3.41 | 4.38 | 4.97 |  | 5/89 |  |  |

Table 6.1.3.1.1-2: DL throughput loss for 14800 – 15350 MHz [Absolute ACIR value]

|  | ACIR (dB) | 5 | 10 | 15 | 17 | 18 | 19 | 20 | 21 | 25 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apple | Throughput lost at Average |  |  | 0.47 |  |  |  | 0.232 |  |  | 0.49 | 0.026 |
|  | Throughput lost at 5%-tile |  |  | 0.623 |  |  |  | 0.247 |  |  | 0.054 | 0.006 |
| CATT | Throughput lost at Average | 0.043 | 0.026 | 0.015 |  |  |  | 0.0082 |  | 0.0042 | 0.0023 |  |
|  | Throughput lost at 5%-tile | 0.16 | 0.15 | 0.14 |  |  |  | 0.11 |  | 0.10 | 0.046 |  |
| MediaTek | Throughput lost at Average | 7.41 | 4.61 | 2.74 |  | 1.97 |  |  | 1.39 | 0.86 | 0.44 |  |
|  | Throughput lost at 5%-tile | 35.10 | 22.03 | 12.45 |  | 8.57 |  |  | 5.90 | 3.78 | 1.70 |  |
| Vivo | Throughput lost at Average |  | 3.11 | 1.79 |  |  |  | 0.91 |  | 0.46 | 0.24 |  |
|  | Throughput lost at 5%-tile |  | 17.71 | 10.13 |  |  |  | 6.21 |  | 2.67 | 1.49 |  |
| Qualcomm | Throughput lost at Average | 6.4 | 3.8 | 2.2 |  |  |  | 1.2 |  | 0.7 | 0.4 |  |
|  | Throughput lost at 5%-tile | 34.5 | 20.4 | 10 |  |  |  | 5.4 |  | 2.5 | 1.5 |  |
| Ericsson | Throughput lost at Average |  |  |  | 0.0134 | 0.0121 | 0.01 | 0.009 | 0.008 |  |  |  |
|  | Throughput lost at 5%-tile |  |  |  | 8 | 7 | 6.24 | 5.25 | 4.6 |  |  |  |
| Huawei | Throughput lost at Average | 7.3 | 4.5 | 2.6 |  |  |  | 1.4 |  | 0.8 | 0.4 |  |
|  | Throughput lost at 5%-tile | 34.9 | 23.3 | 14.0 |  |  |  | 7.8 |  | 3.8 | 1.8 |  |
| Samsung | Throughput lost at Average | 4 | 3 | 2 |  |  |  | 1 |  | 0 | 0 |  |
|  | Throughput lost at 5%-tile | 34 | 20 | 12 |  |  |  | 7 |  | 3 | 2 |  |

In addition, coexistence simulation results for 3 active UEs and Indoor Probability 0% are summarized in Table 6.1.3.1.1-3. and 6.1.3.1.1-4 below.

Table 6.1.3.1.1-3: DL throughput loss for 14800 – 15350 MHz [Relative ACIR with 3 active UEs]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia (R4-2417537) | Average throughput loss in % (15 GHz) | 0.31 |  | 0.51 |  | 0.82 |  | 1.29 |  | 2.00 | 3.02 |
|  | 5%-tile throughput loss in % (15 GHz) | 0 |  | 0 |  | 2.11 |  | 2.11 |  | 2.99 | 4.86 |

Table 6.1.3.1.1-4: DL throughput loss for 14800 – 15350 MHz [Absolute ACIR with Indoor Probability 0%]

| Company | Throughput loss | Absolute ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Ericsson | Average throughput loss in % (15 GHz) | 0.015 | 0.014 | 0.0125 | 0.01 | 0.01 | 0.009 | 0.008 |
|  | 5%-tile throughput loss in % (15 GHz) | 11 | 10 | 8 | 7.5 | 7.2 | 6.7 | 5.5 |

Based on the above simulation results, the required DL ACIR for 14800 – 15350 MHz for urban macro scenarios are in summarized in Table 6.1.3.1.1-5. The range of ACIR is derived as 20-26 dB, and the range of 22-24 dB by excluding the highest/lowest.

Table 6.1.3.1.1-5: DL simulation results for 14800 – 15350 MHz

|  | ACIR (dB) |
| --- | --- |
| Company | Urban macro |
| Nokia | -12 dB (relative) |
| Apple | 28-30 |
| CATT | 24 |
| MediaTek | 23-24 |
| Vivo | 22 |
| Qualcomm | 22 |
| Ericsson | 20-24 |
| ZTE | -8 dB (relative) |
| Huawei | 26 |
| Samsung | 24-25 |
| [Average in range] | 20-26 |
| [Excluding the highest/lowest] | 22-24 |

##### 6.1.3.1.2	Uplink

For Urban macro scenarios, simulation results for UL throughput loss with baseline assumption ACLR = 30 dBc at UE and ACS = 45 dBc at BS are summarized in Table 6.1.3.1.2-1/2/3/4/5.

Table 6.1.3.1.2-1: UL throughput loss for 14800 – 15350 MHz [Relative ACIR value]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia (R4-2417536) | Average throughput loss in % (15 GHz) | 0.07 |  | 0.12 |  | 0.20 |  | 0.30 |  | 0.46 | 0.68 |
|  | 5%-tile throughput loss in % (15 GHz) | 1.14 |  | 2.21 |  | 3.52 |  | 3.55 |  | 3.61 | 3.73 |
| ZTE | Average throughput loss in % (15 GHz) | 0.26 | 0.30 |  | 0.34 | 0.39 | 0.46 |  | 0.55 |  |  |
|  | 5%-tile throughput loss in % (15 GHz) | 6.36 | 7.20 |  | 8.95 | 9.10 | 9.36 |  | 10.98 |  |  |

Table 6.1.3.1.2-2a: UL throughput loss for 14800 – 15350 MHz [Absolute ACIR value]

|  | ACIR (dB) | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 20 | 22 | 23 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apple | Throughput lost at Average |  |  | 0.6 |  |  |  |  | 0.3 | 0.13 | 0.099 | 0.079 | 0.06 | 0.022 |
|  | Throughput lost at 5%-tile |  |  | 0.59 |  |  |  |  | 0.22 | 0.14 | 0.131 | 0.1 | 0.1 | 0.01 |
| CATT | Throughput lost at Average |  |  | 0.027 |  |  |  |  | 0.018 | 0.01 |  |  | 0.0055 | 0.0024 |
|  | Throughput lost at 5%-tile |  |  | 0.088 |  |  |  |  | 0.0032 | 0.0001 |  |  | 3.1e-5 | 9.7e-6 |
| MediaTek | Throughput lost at Average |  |  | 2.12 |  |  |  |  | 1.15 | 0.60 |  |  | 0.3 | 0.14 |
|  | Throughput lost at 5%-tile |  |  | 8.48 |  |  |  |  | 4.05 | 1.94 |  |  | 0.75 | 0.16 |
| Vivo | Throughput lost at Average |  |  | 0.80 |  |  |  |  | 0.38 | 0.18 |  |  | 0.08 | 0.04 |
|  | Throughput lost at 5%-tile |  |  | 5.77 |  |  |  |  | 2.82 | 0.93 |  |  | 0.14 | 0 |
| Qualcomm | Throughput lost at Average |  |  | 1.4 |  |  |  |  | 0.7 | 0.4 |  |  | 0.18 | 0.09 |
|  | Throughput lost at 5%-tile |  |  | 7.4 |  |  |  |  | 2.8 | 1.8 |  |  | 0.7 | 0.4 |
| Ericsson | Throughput lost at Average | 0.0088 | 0.0078 | 0.0068 | 0.006 | 0.0052 | 0.0046 | 0.004 |  |  |  |  |  |  |
|  | Throughput lost at 5%-tile | 6 | 5.66 | 5 | 4.6 | 4.22 | 3.6 | 3.3 |  |  |  |  |  |  |
| Huawei | Throughput lost at Average |  |  | 5.0 |  |  |  |  | 2.7 | 1.3 |  |  | 0.6 | 0.3 |
|  | Throughput lost at 5%-tile |  |  | 20.7 |  |  |  |  | 13.5 | 7.6 |  |  | 3.67 | 1.5 |

In addition, coexistence simulation results for 3 active UEs and Indoor Probability 0% are summarized in Table 6.1.3.1.2-3.and 6.1.3.1.2-4 below.

Table 6.1.3.1.2-3: UL throughput loss for 14800 – 15350 MHz [Relative ACIR with 3 active UEs]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia (R4-2411537) | Average throughput loss in % (15 GHz) | 0.23 |  | 0.33 |  | 0.48 |  | 0.72 |  | 1.08 | 1.63 |
|  | 5%-tile throughput loss in % (15 GHz) | 1.81 |  | 3.40 |  | 3.47 |  | 5.06 |  | 6.14 | 9.49 |

Table 6.1.3.1.2-4: UL throughput loss for 14800 – 15350 MHz [Absolute ACIR with Indoor Probability 0%]

| Company | Throughput loss | Absolute ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| Ericsson | Average throughput loss in % (15 GHz) | 0.02 | 0.017 | 0.015 | 0.013 | 0.012 | 0.01 | 0.009 |
|  | 5%-tile throughput loss in % (15 GHz) | 12.52 | 11 | 10 | 8.2 | 7.2 | 6 | 5 |

Based on the above simulation results, the required UL ACIR for 14800 – 15350 MHz for urban macro scenarios are in summarized in Table 6.1.3.1.2-5. Many results suggest ACIR around e.g. 13-15dB excluding highest and lowest values.

Table 6.1.3.1.2-5: UL simulation results for 14800 – 15350 MHz

|  | ACIR (dB) |
| --- | --- |
| Company | Urban macro |
| Nokia | -9 (relative) |
| Apple | 22-24 |
| CATT | 20 |
| MediaTek | 15-17 |
| Vivo | 12 |
| Qualcomm | 13 |
| Ericsson | 11-14 |
| ZTE | -10 (relative) |
| Huawei | 25 |
| [Average in range excluding the highest/lowest] | 13-15 |

#### 6.1.3.2	Dense urban scenario (void)

#### 6.1.3.3	Indoor scenario (void)

-------------------------------------------- TP BEGIN --------------------------------------------