3GPP TSG-RAN WG4 Meeting # 113	R4-2420404

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.8.5

Source:	vivo

Title:						WF on HPUE for IoT NTN

Document for:		Approval

- Tx Requirements

- Tx requirements

Issue 1-1-1: For NB-Iot Based Iot-NTN, whether and how to define GSM_ACLR.

- Proposals

  - Option 1: Remove this requirement, [at least for HPUE]

  - Option 2: Keep this requirement and reusing the PC3 requirement for HPUE.

  - Option 3: Others

Agreement:

- Remove GSM_ACLR for NB-IoT based NTN HPUE.

Issue 1-1-2: PC1/2 ACLR should not below PC3 requirements, based on the assumption that PC3 requirement can be relaxed if necessary.

- Proposals

  - Option 1: Yes.

  - Option 2: No

  - Option 3: Others

Agreement:

- HPUE ACLR should not be relaxed than that for PC3 for NB-IoT NTN.

Issue 1-1-3: For NB-Iot based NTN, considering to extend the SEM for PC2 and PC1 as following

Agreement: For NB-Iot based NTN, consider PC3 SEM as a starting point. More evaluation can be considered.

Issue 1-1-4: SEM relaxation for the case UE CBW is much narrower compared to BS CBW

- Proposals

  - When UE PC2 ACLR is decided, regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing UE SEM requirements within SAN CBW but still meet ITU SEM regulatory requirements.

- WF

  - FFS whether MPR reduction technique currently discussed in Rel-19 can be considered here

Issue 1-1-5: NB-Iot based Iot-NTN, MPR=0 reference point

- Proposals

  - Proposal 1. Considering center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.

  - Option 2: Others

Agreement:

- As the starting point, consider the center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.

Issue 1-1-6: NB-Iot based Iot NTN,  P-MPR has not been defined.

- Proposals

  - Revise the configured transmitted Power section for NB-Iot Based Iot-NTN in 36.102 to incorporate P-MPR. Details FFS.

Agreement:

- Revise the configured transmitted Power section for NB-Iot Based Iot-NTN in 36.102 to incorporate P-MPR.

  - Details are FFS.

- Other solutions for SAR are not precluded.

Issue 1-1-7: General guideline for eMTC based Iot-NTN,

- Proposals

  - As a general guideline, for eMTC based Iot-NTN, the HPUE requirements can using legacy LTE requirements as starting point [unless otherwise stated.]

- WF

  - FFS

Issue 1-1-8: The nominal MOP tolerance]

Background:

HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP.

From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

As for IoT-NTN eMTC and NB1/NB2, the actual NTN-IoT PC2 HD-FDD TX output power upper bound could be 28.8dBm (i.e., 26.8dBm+2dB) but exceeds conventional PC2 TX upper power bound of 28dBm.

From ETSI EN 301 908, PC3 TX output power upper bound can be specified higher (i.e., 23dBm +2.7dB or 23dBm +3dB).

- Proposals

  - As for IoT-NTN eMTC and NB1/NB2, regarding solution for PC2 HD-FDD TX MOP upper bound higher than FD-FDD TX MOP upper bound, consider a specific or general solution for all IoT-NTN bands. The solution is applicable for solving the same issue in PC3 and PC5. The options are as follows.

- Option1: Add additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When UEs recognizes and supports NTN HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD]. |  |  |  |  |  |  |

- Option 2: Add additional UE capability and NW signal to control NTN HD-FDD power increment.

- Option 2-1:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB. |  |  |  |  |  |  |

- Option 2-2:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 255 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 254 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 253 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 252 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The reference power is increased by ΔPNTNHdFdd. |  |  |  |  |  |  |

- Option 3: Add new power classes (e.g., PC4.5_NTN-HD-FDD, PC2.5_NTN-HD-FDD, PC1.75_NTN-HD-FDD).

- Option 4:

  - Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance can be considered as starting point (same as for band n14) for PC2 for IoT-NTN HPUE.

- WF

  - FFS

- Feasibility & Requirements Application

Issue 1-2-1: Discuss the unsettled requirements applicability for Iot-NTN:

- Agreement (Ad Hoc):

  - No 2Tx PC1/PC2 requirement will be specified for IoT NTN in Rel-19;

  - Only consider non-handheld for PC1 1Tx eMTC based with full duplex Iot-NTN

  - FFS whether requirements should be defined for PC1 1Tx handheld UE for NB-Iot based Iot-NTN

Updated Status: (For Information)

Table: Current status for requirements consideration and feasibility for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

- SAR Related issues

- SAR Related issues

Issue 2-1-2: If introduced, whether and how to enhancement current duty cycle method in TN.

- Proposals

  - Proposal 1:

    - To consider the serving time of each beam, or the beam service time ratio of the UE, as a potential upper bound of the UL ’duty-cycle’ for the NTN HPUE. (Samsung)

    - To further discuss the duty-cycle solution, and to check if it can be altered for NTN scenario to allow UE to stay in its declared power class as much as possible, which helps NTN HPUE to close the link in emergencies and compensate the rain/cloud fading during natural disasters. (Samsung)

  - Proposal 2:

    - Besides of maxUplinkdutycycle capability reporting, UE working status like near the head mode (SAR is required) or other modes like handheld mode (SAR is not required) also need to be indicated. (oppo)

    - BSR (buffer status report) can be reused to report the UE working status of following transmission time. (oppo)

  - Proposal 3:

    - Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value. (Ericsson, Sony)

    - If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details: (Ericsson, Sony)

      - If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

        - DPC = ‘00’: the criteria to report DPC is not met

        - DPC = ‘01’: ΔPPowerClass = 0 dB

        - DPC = ‘10’: ΔPPowerClass = 3 dB

        - DPC = ‘11’: ΔPPowerClass = 6 dB

      - If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

        - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

        - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

        - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

        - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

  - Proposal 4:

    - It is not reasonable for network to send the control message to control SAR of each UE. (Nokia)

    - In principle, applying P-MPR cannot invalidate the purpose of letting IoT-NTN UEs have high Tx power as PC1/PC1.5/PC2 devices do.(Nokia)

    - The combination of using duty cycle and small P-MPR can be considered for high power UE to meet SAR requirements.(Nokia)

- Agreement:

  - Further discuss the proposed schemes related to duty cycle related SAR scheme.

  - Other schemes or no duty cycle related scheme are not precluded.

- Rx Requirments

- RSD (for full duplex eMTC based)

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based)

- Proposals

  - N256:

    - 0 dB (vivo, oppo, Ericsson)

    - [1] dB (Qualcomm)

  - N255:

    - 0 dB (oppo)

    - Using n3 RSD as reference (Samsung, last meeting)

  - N254:

    - [0] dB (vivo, Samsung, Ericsson)

- WF

  - N254: [0] dB

  - Other bands FFS;

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Proposals

  - Option 1: 0dB for n254/n255/n256 (vivo, oppo)

  - Option 2: others

- WF

  - [0] dB for n254/n255/n256

Issue 3-1-x: General for RSD

Way forward:

- Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.