3GPP TSG-RAN WG4 Meeting #113	R4-2420429

Orlando, USA, 18 – 22 November 2024

Source:	Ericsson

Title:	TP for TR 38.718-11-11: DC_68A_n1A

Agenda item:			6.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.719-11-11 to include DC_68A_n1A as defined in WID [1].

# ---Start of changes---

## 6.1	Inter-band DC within FR1

### 6.1.X	DC_68_n1

#### 6.1.X.1	Configuration for DC

Table 6.1.X.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n1A | DC_68A_n1A | No |

#### 6.1.X.2	Maximum output power for DC

Table 6.1.X.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n1A | 23 | +2/-3 |

#### 6.1.X.3	Spurious emission band UE co-existence for DC

According to TS 36.101 t, there is no requirements for B68  on  specific UE protection in addition to a set of protected bands.  According to TS 38.101-1, band n1 has no regulatory emissions requirements so there is no need to include this combination in UE co-existence table.

#### 6.1.X.4	Co-existence analysis for DC

Table 6.1.X.4-1 Band 68 and Band n1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2 792 | 3 490 |  |
| n1 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3 606 |  |
| DL1 | 2110 | 2170 | N/A | - | D | - |  | UL harmonic |
| DL23 | 4220 | 4340 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  | - | - | N/A | - | N/A |  |
| DL4 |  |  | - | N/A | N/A | N/A | N/A |  |
| DL54 |  |  | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | The 3rd harmonic of band 68 may fall into RX frequencies of band n1. |  |  |  |  |  |
| UL/DL harmonics |  | n1 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| 68 | flow | fhigh | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | D | N/A | N/A | N/A |  |
| Analysis |  |  | The 2nd harmonic mixing of band n1 may fall into 5th DL of band 68. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.X.4-3: Band 68 and Band n1 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1192 – 1282 |  | 2618 – 2708 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 464 – 584 |  | 3112 – 3262 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3316 – 3436 |  | 4538 – 4688 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 114 – 264 |  | 5032 – 5242 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2384 – 2564 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4014 – 4164 |  | 6458 – 6668 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5236 – 5416 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6952 – 7222 |  | 812 – 992 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4304 – 4544 |  | 1656 – 1866 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8378 – 8648 |  | 4712 – 4892 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7156 – 7396 |  | 5934 – 6144 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.X.4-3, there is no IMD issue.

#### 6.1.X.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n1 as specified in TS 38.101-3.

Table 6.1.X.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n1 | 0.6 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.X.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n1 | 0.2 | - |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

#### 6.1.X.6	Analysis of MSD requirements

The REFSENS requirement for DC_68_n1 due to UL harmonic are reused from DC_28_n1 and are shown in table 6.1.X.6-1:

Table 6.1.X.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 68 | n1 | 5 | 15 | 8 | 5 | 10.2 | NOTE 3 | UL3/DL1 direct-hit |

The coexistence study shows the possibility of 2nd  harmonic mixing of band n1 may fall into 5th DL of band 68. However the impact of harmonic mixing is not considered severe therefor no MSD needs to be defined.

---End of changes---

# Reference

[1] 	 RP-241786, Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands, Nokia