3GPP TSG-RAN4 Meeting #113	 R4-2420504

# Orlando, USA, 18th – 22th November, 2024

Source: 	Huawei, Hisilicon, CMCC, Qualcomm, vivo

Title: 	TP to TR38.769 Co-existence simulation

Agenda Item:	7.21.2.2

Document for:	Approval

Introduction

In the last two meetings, the formal co-existence simulation assumptions for Rel-19 Ambient IoT were approved in [1][2]. Companies provided the co-existence evaluation results this meeting.

In this contribution, we provide the draft TP to TR 38.769 for Co-existence evaluation based on [3][4].

Reference

[1] R4-2414304, WF on Co-existence study for ambient IoT, CMCC, RAN4#112, August 2024

[2] R4-2417196, WF on co-existence study for A-IOT, CMCC, RAN4#112bis, October 2024

[3] R4-2414306, TP to TR38.769 for Co-existence evaluation assumptions, Huawei, Hisilicon, RAN4#112, August 2024

[4] R4-2416356, TP to TR38.769 Co-existence simulation, Huawei, Hisilicon, RAN4#112bis, October 2024

Text Proposal to TR 38.769

------------Start of the Change-------------

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.848: "Study on Ambient IoT (Internet of Things) in RAN".

[R1-1]	RP-240826: "Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR".

[R1-2]	3GPP TR 38.869: "Study on low-power Wake-up Signal and Receiver for NR".

[R1-3]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[R1-4]	EPC Radio-Frequency Identity Protocols Class-1 Generation-2 UHF RFID Protocol for Communications at 860 MHz – 960 MHz.

9.4.1.1 References

[R1-9411-1]	R1-2407608, "Discussion on evaluation assumptions and results for Ambient IoT devices", FUTUREWEI, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-2]	R1-2407636, "Evaluation assumptions and results for Ambient IoT", Ericsson, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-3]	R1-2407643, "Evaluation assumptions and results for Ambient IoT", Nokia, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-4]	R1-2407667, "Evaluation methodology and assumptions for Ambient IoT", Huawei, HiSilicon, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-5]	R1-2407705, "Discussion on evaluation assumptions and results for Ambient IoT", Spreadtrum Communications, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-6]	R1-2407760, "Discussion on Link Level simulation of A-IoT", Tejas Network Limited, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-7]	R1-2407860, "Evaluation methodologies assumptions and results for Ambient IoT", vivo, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-8]	R1-2407968, "Evaluation assumptions and results for Ambient IoT", Xiaomi, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-9]	R1-2408046, "The evaluation methodology and preliminary results of Ambient IoT", CATT, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-10]	R1-2408065, "Discussion on Ambient IoT evaluations", ZTE Corporation, Sanechips, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-11]	R1-2408145, "Discussion on evaluation assumptions and results for A-IoT", OPPO, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-12]	R1-2408233, "Discussion on evaluation assumptions and results for Ambient IoT", HONOR, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-13]	R1-2408355, "Analysis for Inventory Completion Time for Multiple A-IoT Devices", Wiliot Ltd., RAN1#118bis, Hefei, China, October 2024.

[R1-9411-14]	R1-2408374, "Discussion on ambient IoT evaluation framework", NEC, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-15]	R1-2408464, "On remaining evaluation assumptions and results for AIoT", Apple, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-16]	R1-2408598, "Link level simulation for Ambient IoT R2D", Panasonic, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-17]	R1-2408670, "Discussion on Ambient IoT evaluation", LG Electronics, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-18]	R1-2408699, "Evaluation assumptions and results for A-IoT", MediaTek Inc., RAN1#118bis, Hefei, China, October 2024.

[R1-9411-19]	R1-2408731, "Evaluation results for Ambient IoT", InterDigital, Inc., RAN1#118bis, Hefei, China, October 2024.

[R1-9411-20]	R1-2408785, "Study on evaluation assumptions and results for Ambient IoT", NTT DOCOMO, INC., RAN1#118bis, Hefei, China, October 2024.

[R1-9411-21]	R1-2408849, "Evaluation Assumptions and Results", Qualcomm Incorporated, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-22]	R1-2408930, "Discussion on Evaluation assumptions and results", CEWiT, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-23]	R1-2408938, "Evaluation assumption and results for AIoT", IIT Kanpur, Indian Institute of Tech (M), RAN1#118bis, Hefei, China, October 2024.

[R1-9411-24]	R1-2408966, "Discussion on the evaluation assumptions for Ambient IoT devices", Lenovo, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-25]	R1-2408987, "Ambient IoT evaluations", Sony, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-26]	R1-2409006, "Discussion on evaluation assumptions and results for Ambient IoT", CMCC, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-27]	R1-2409022, "Discussion on evaluation assumptions and results for Ambient IoT", China Telecom, RAN1#118bis, Hefei, China, October 2024.

[R1-9411-28]	R1-2409027, "Considerations for evaluation assumptions and results", Samsung, RAN1#118bis, Hefei, China, October 2024.

9.4.1.2 References

9.4.2.1 References

[R1-9421-1]	R1-2407610, "Discussion on physical layer design for Rel-19 Ambient IoT devices", FUTUREWEI, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-2]	R1-2407628, "Discussion on general aspects of physical layer design for Ambient IoT", TCL, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-3]	R1-2407638, "General aspects of physical layer design for Ambient IoT", Ericsson, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-4]	R1-2407645, "General aspects of physical layer design for Ambient IoT", Nokia, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-5]	R1-2407669, "On general aspects of physical layer design for Ambient IoT", Huawei, HiSilicon, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-6]	R1-2407707, "Discussion on general aspects of physical layer design for Ambient IoT", Spreadtrum Communications, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-7]	R1-2407734, "Discussion on general aspects of physical layer design for Ambient IoT", China Telecom, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-8]	R1-2407862, "Discussion on General Aspects of Physical Layer Design", vivo, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-9]	R1-2407906, "Discussion on general aspects of A-IoT physical layer design", CMCC, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-10]	R1-2407970, "Discussion on physical layer design of Ambient IoT", Xiaomi, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-11]	R1-2408048, "Discussion on general aspects of physical layer design", CATT, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-12]	R1-2408147, "Discussion on general aspects of physical layer design of A-IoT communication", OPPO, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-13]	R1-2408206, "Discussion on general aspects of ambient IoT physical layer design", NEC, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-14]	R1-2408250, "Discussion on general aspects of physical layer design", Sharp, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-15]	R1-2408272, "Discussion on Physical Layer Design for Ambient-IoT", EURECOM, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-16]	R1-2408308, "On General Physical Layer Design Considerations for Ambient IoT Applications", Lekha Wireless Solutions, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-17]	R1-2408410, "General aspects of Ambient IoT physical layer design", Sony, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-18]	R1-2408466, "On remaining general physical layer design aspects for AIoT", Apple, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-19]	R1-2408568, "Discussion on general aspects of physical layer design", ETRI, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-20]	R1-2408599, "General aspects of physical layer design for Ambient IoT", Panasonic, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-21]	R1-2408647, "Considerations for general aspects of Ambient IoT", Samsung, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-22]	R1-2408672, "General aspects of Ambient IoT physical layer design", LG Electronics, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-23]	R1-2408685, "Discussion on physical layer design for Ambient IoT", Comba, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-24]	R1-2408701, "General aspects of physical layer design", MediaTek Inc., RAN1#118bis, Hefei, China, October 2024.

[R1-9421-25]	R1-2408730, "On the general aspects of physical layer design for Ambient IoT", InterDigital, Inc., RAN1#118bis, Hefei, China, October 2024.

[R1-9421-26]	R1-2408765, "Discussion on A-IoT physical layer design", ASUSTeK, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-27]	R1-2408787, "Study on general aspects of physical layer design for Ambient IoT", NTT DOCOMO, INC., RAN1#118bis, Hefei, China, October 2024.

[R1-9421-28]	R1-2408851, "General aspects of physical layer design", Qualcomm Incorporated, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-29]	R1-2408875, "Discussion on multiple access for D2R", LG Uplus, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-30]	R1-2408941, "Discussion on General aspects of physical layer design of AIoT", IIT Kanpur, Indian Institute of Tech (M), RAN1#118bis, Hefei, China, October 2024.

[R1-9421-31]	R1-2408967, "Discussion on the physical layer design aspects for Ambient IoT devices", Lenovo, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-32]	R1-2409005, "Discussion on general aspects of physical layer design for Ambient IoT", ZTE Corporation, Sanechips, RAN1#118bis, Hefei, China, October 2024.

[R1-9421-33]	R1-2406315, "Consideration on general aspects of physical layer", Fujitsu, RAN1#118, Maastricht, Netherlands, August 2024.

[R1-9421-34]	R1-2407119, "General aspects of physical layer design for Ambient IoT", ITL, RAN1#118, Maastricht, Netherlands, August 2024.

[R1-9421-35]	R1-2407088, "Discussion on General aspects of physical layer design", CEWiT, RAN1#118, Maastricht, Netherlands, August 2024.

[R1-9421-yy]	J. K. Wolf and R. D. Blakeney, “An exact evaluation of the probability of undetected error for certain shortened binary CRC codes,” Proc. Milcom ’88, vol. 1, pp. 287-292, 1988.

9.4.2.2 References

[R1-9422-1]	R1-2407611, "Discussion on Frame Structure and Timing Aspects for Ambient IoT", FUTUREWEI, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-2]	R1-2407639, "Frame structure and timing aspects for Ambient IoT", Ericsson, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-3]	R1-2407646, "Frame structure and timing aspects for Ambient IoT", Nokia, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-4]	R1-2407670, "On frame structure and timing aspects of Ambient IoT", Huawei, HiSilicon, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-5]	R1-2407708, "Discussion on frame structure and timing aspects for Ambient IoT", Spreadtrum Communications, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-6]	R1-2407907, "Discussion on frame structure and timing aspects for A-IoT", CMCC, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-7]	R1-2407971, "Discussion on frame structure and timing aspects for Ambient IoT", Xiaomi, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-8]	R1-2408049, "Study of Frame structure and timing aspects for Ambient IoT", CATT, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-9]	R1-2408068, "Discussion on frame structure and physical layer procedure for Ambient IoT", ZTE Corporation, Sanechips, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-10]	R1-2408148, "Discussion on frame structure and timing aspects of A-IoT communication", OPPO, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-11]	R1-2408234, "Discussion on frame structure and timing aspects for Ambient IoT", HONOR, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-12]	R1-2408251, "Discussion on frame structure and timing aspects", Sharp, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-13]	R1-2408411, "Frame structure and timing aspects for Ambient IoT", Sony, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-14]	R1-2408434, "Frame structure and timing aspects of Ambient IoT", InterDigital, Inc., RAN1#118bis, Hefei, China, October 2024.

[R1-9422-15]	R1-2408467, "On remaining frame structure and timing aspects for AIoT", Apple, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-16]	R1-2408536, "Discussion on A-IoT Frame Structure and Timing Aspects", Panasonic, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-17]	R1-2408648, "Considerations for frame structure and timing aspects", Samsung, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-18]	R1-2408673, "Frame structure and timing aspects for Ambient IoT", LG Electronics, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-19]	R1-2408702, "Frame structure and timing aspects", MediaTek Inc., RAN1#118bis, Hefei, China, October 2024.

[R1-9422-20]	R1-2408742, "Considerations for frame structure and timing aspects", Semtech Neuchatel SA, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-21]	R1-2408920, "Discussion on frame structure and timing aspects for Ambient IoT", TCL, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-22]	R1-2408990, "Study on frame structure and timing aspects for Ambient IoT", NTT DOCOMO, INC., RAN1#118bis, Hefei, China, October 2024.

[R1-9422-23]	R1-2409008, "Discussion on Frame structure, random access, scheduling and timing aspects for Ambient IoT", vivo, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-24]	R1-2409026, "Discussion on frame structure and physical layer procedures for Ambient IoT", Lenovo, RAN1#118bis, Hefei, China, October 2024.

[R1-9422-25]	R1-2409057, "Frame structure and timing aspects", Qualcomm Incorporated, RAN1#118bis, Hefei, China, October 2024.

9.4.2.3 References

[R1-9423-1]	R1-2407612, "Discussion on D2R and R2D Channel/Signal Aspects for Ambient IoT", FUTUREWEI, RAN1#118bis, Hefei, China, October 2024.

[R1-9423-2]	R1-2407671, "Physical channels and signals for Ambient IoT", Huawei, HiSilicon, RAN1#118bis, Hefei, China, October 2024.

[R1-9423-3]	R1-2407864, "Discussion on  Downlink and uplink channel/signal aspects", vivo, RAN1#118bis, Hefei, China, October 2024.

[R1-9423-4]	R1-2408050, "DL and UL Physical Channels/signals design in support of Ambient IoT devices", CATT, RAN1#118bis, Hefei, China, October 2024.

[R1-9423-5]	R1-2408069, "Discussion on channel and signal  for Ambient IoT", ZTE Corporation, Sanechips, RAN1#118bis, Hefei, China, October 2024.

[R1-9423-6]	R1-2408149, "Discussion on downlink and uplink channel/signal aspects for A-IoT", OPPO, RAN1#118bis, Hefei, China, October 2024.

[R1-9423-7]	R1-2408532, "Considerations on Intermediate UE in A-IoT", Continental Automotive, RAN1#118bis, Hefei, China, October 2024.

[R1-9423-8]	R1-2408703, "Downlink and uplink channel/signal aspects", MediaTek Inc., RAN1#118bis, Hefei, China, October 2024.

[R1-9423-9]	R1-2408853, "Downlink and uplink channel/signal aspects", Qualcomm Incorporated, RAN1#118bis, Hefei, China, October 2024.

[R1-9423-10]	R1-2408932, "Discussion on Downlink and Uplink channel/signal aspects", CEWiT, RAN1#118bis, Hefei, China, October 2024.

[R1-9423-11]	R1-2408943, "Discussion on Downlink and Uplink channel signal aspects for AIoT", IIT Kanpur, Indian Institute of Tech (M), RAN1#118bis, Hefei, China, October 2024.

[R1-9423-12]	R1-2409023, "Downlink and uplink channel/signal aspects for Ambient IoT", Ericsson, RAN1#118bis, Hefei, China, October 2024.

RAN2 References

[R2-1]	3GPP TR 23.700-13: "Study on Architecture support of Ambient power-enabled Internet of Things".

[R2-2]	3GPP TR 33.713: "Study on security aspects of Ambient Internet of Things (AIoT) services in 5G".

[R2-3]	3GPP TS 38.300: "NR and NG-RAN Overall description; Stage-2".

[R2-4]	3GPP TR 22.369: "Service requirements for Ambient power-enabled IoT".

9.4.2.4 References

[R1-9424-1]	R1-2405855, "On external carrier wave for backscattering based Ambient IoT device", Huawei, HiSilicon, RAN1#118. Maastricht, Netherlands, August 2024.

[R1-9424-2]	R1-2408854, "Waveform characteristics of carrier-wave provided externally to the Ambient IoT device", Qualcomm Incorporated, RAN1#118bis, Hefei, China, October 2024.

[R4-1]	R4-2417981, " Collection of simulation results for Ambient-IoT co-existence study ", Huawei, HiSilicon, RAN4#113, Orlando, USA, November 2024.

[R4-2]	K. Fors, E. Axell, S. Linder and P. Stenumgaard, "On the Impact of CW interference on 5G NR," 2019 International Symposium on Electromagnetic Compatibility - EMC EUROPE, Barcelona, Spain, 2019, pp. 1049-1054.

[R4-3]	R4-2418182, “Further evaluation on D2T2 co-existence”, vivo, RAN4#113, Orlando, USA, November 2024.

[R4-4]	R4-2418852, “Discussion on Ambient IoT Coexistence Evaluations”, Qualcomm Incorporated, RAN4#113, Orlando, USA, November 2024.

------------Next Change-------------

- 6.6	Coexistence of ambient IoT and NR/LTE

### 6.6.1	Co-existence scenarios and cases

The coexistence evaluation is conducted considering the different scenarios listed in Table 6.6.1-1

Table 6.6.1-1: Co-existence scenarios

| Deployment scenario No. (Case No.) | Topology | Spectrum |
| --- | --- | --- |
| 1-1(a/b/c/d) | D1T1-A2- NR UE only outdoor | R2D: DL CW2D and D2R: DL |
| 1-2(a/b/c/d) | D1T1-A2- NR UE indoor | R2D: DL CW2D and D2R: DL |
| 2-1(e/f/c/d) | D1T1-B- NR UE only outdoor | R2D: DL CW2D and D2R: UL |
| 2-2(e/f/c/d) | D1T1-B- NR UE indoor | R2D: DL CW2D and D2R: UL |
| 3 (Optional)(a/b/g/h) | D1T1-A2- NR UE indoor | R2D: UL CW2D and D2R: UL |
| 5-1(e/f/g/h) | D2T2-A2- NR UE only outdoor | R2D: UL CW2D and D2R: UL |
| 5-2(e/f/g/h) | D2T2-A2- NR UE indoor | R2D: UL CW2D and D2R: UL |
| 6-1(Optional)(a/b/g/h) | D2T2-B- NR UE only outdoor | R2D: UL CW2D and D2R: DL |
| 6-2(Optional)(a/b/g/h) | D2T2-B- NR UE indoor | R2D: UL CW2D and D2R: DL |

The main co-existence scenario considered for ambient IoT is D1T1 and D2T2. The deployment parameters are described in Table 6.6.2.1-1.

The co-existence evaluation captures cases where NR and A-IoT (Reader/device) are both victim and aggressor networks. It is to evaluate impact on legacy NR networks if A-IoT is introduced in indoor scenario, also to understand impact of the legacy NR network on A-IoT system. The co-existence cases are listed in Table 6.6.1-2.

Table 6.6.1-2: Co-existence cases

| Case No. | Aggressor | Victim | note |
| --- | --- | --- | --- |
| a | Device | NR DL | D2R |
| b | NR DL | reader | D2R |
| c | Reader | NR DL | R2D |
| d | NR DL | device | R2D |
| e | Device | NR UL | D2R |
| f | NR UL | reader | D2R |
| g | Reader | NR UL | R2D |
| h | NR UL | device | R2D |

------------Next Change-------------

### 6.6.2	Co-existence evaluation assumptions

- 6.6.2.1	Deployment

Simulation assumptions related to network layout is captured for D1T1 and D2T2 in Table 6.6.2.1-1.

Table 6.6.2.1-1. Deployment parameters for D1T1 and D2T2

| Parameter | D1T1 | D2T2 |  |
| --- | --- | --- | --- |
| Carrier frequency | 900 MHz (Band n8) |  |  |
| BW for NR | 10MHz with 15KHz SCS |  |  |
| BW for A-IOT system | 180kHz, for 15kHz SCS |  |  |
| Waveform (R2D) | OOK waveform generated by OFDM modulator |  |  |
| Waveform (CW) | Unmodulated single tone |  |  |
| A-IoT DL power control | No |  |  |
| A-IoT UL power control | No |  |  |
| Traffic model | Full buffer |  |  |
| Frequency reuse | 1 |  |  |
| NR BS deployment (outdoor), i.e. scenario 1-1 and 1-2 | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 A-IOT indoor scenario per sector the minimum 2D distance between macro BS and indoor factory center is set as 100m. Inter-NR BS distance: 750m NR BS height: 25m |  |  |
| NR BS deployment (indoor), i.e. scenario 2-1 and 2-2 | NR indoor BS deployed co-site with A-IoT indoor reader; NR indoor BS ISD as 20m; NR indoor Min BS-UE distance: 0m; NR indoor UE uniformly distributed. NR indoor BS Tx power assumed [24] dBm Antenna pattern of A-IoT reader is reused. The self-interference cancellation of R2D in UL for co-located scenario can be reported by interested company. |  |  |
| A-IoT reader deployment / Intermediate UE dropping | For D1T1-A2 and D1T1-B: 18 A-IoT readers on a square lattice with spacing D, located D/2 from the walls. L=120m x W=60m; D=20m Reader height = 8 m Room height = 10m 2 A-IoT readers are activated in one drop as baseline. Minimum distance between active readers: 60m as baseline | For D2T2-A2 and D2T2-B: The intermediate UEs selected from the fixed positions. L=120m x W=50m; D=20m Intermediate UE height = 1.5 m Room height = 3m Number of intermediate UE for simulation: 2 UE at one drop. Minimum distance between intermediate UEs: 60m as baseline |  |
| CW deployment | For D1T1-A2 and D2T2-A2, CW and A-IoT reader are collocated. For D1T1-B and D2T2-B, CW topology layout is the same as A-IoT reader. For each device, the nearest CW node will be activated during the simulation, and CW node is not co-located with any activated reader |  |  |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 1m |  |  |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. For NR UE indoor, uniformly distributed, Option1: 10% indoor, 90% outdoor, Option2: 100% indoor UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |  |  |
| NR BS Inter-site distance | MCL of 70 dB |  |  |
| Pathloss model | PL= PLb+PLtw+PLin+N(0, σP2) - PLin = 0.5 * d2D-in where d2D-in is the distance to nearest factory/office boundary on the line between Tx and Rx point. Set maximum value of d2D-in as [25m] as optional |  |  |
|  | PLb : NLOS and LOS in TR38.901 NR BS – NR UE: Uma Outdoor NR UE – A-IoT reader/ device: Umi Device – A-IoT reader: InF-DH Indoor NR UE – device: InH-Office Indoor NR UE –A-IoT reader: InF-DH |  | PLb : NLOS and LOS in TR38.901 NR BS – NR UE: Uma Outdoor NR UE – intermediate UE/ device: Umi Device – Intermediate UE: InH-Office Indoor NR UE – device: InH-Office Indoor NR UE – intermediate UE: InH-Office |
| O2I penetration loss | High penetration loss as in TR 38.901 |  |  |

#### 6.6.2.2	NR BS/ A-IoT reader/ intermediate UE/ CW RF characteristics

The NR BS and A-IoT reader are defined for two different antenna configurations as illustrated in Table 6.6.2.2-1 and Table 6.6.2.2-2. Assumptions related to CW RF characteristics relevant for different deployment scenarios are captured in Table 6.6.2.2-4.

Table 6.6.2.2-1: NR BS RF parameters

| NR BS Parameter | Values for evaluation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:15 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |
| ACLR | Option 1: 30dBc, Option 2: 17dBc |

Table 6.6.2.2-2: A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | 33 dBm |
| A-IoT reader receiver Noise Figure (dB) | 10 |
| A-IoT reader antenna gain (dBi) | 6 |
| ACLR | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB) |
| ACS | Same as legacy NR BS |
| Antenna configuration | Antenna Array Geometry：equals to omni-directional antenna pattern in GCG in horizontal Antenna pattern (horizontal): AHϕ=-min12ϕϕ3dB2,Am, φ3dB = 90°, Am = 15 dB Antenna pattern (horizontal): AVθ=-min12θ-90°θ3dB2,SLAv, θ3dB= 90°, SLAv = 15 dB Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss): 6 |

Table 6.6.2.2-3: Intermediate UE RF parameters

| intermediate UE parameters | Values for evaluation |
| --- | --- |
| Intermediate UE total Tx power (dBm) | Baseline: 23 Optional: Tx power control is allowed |
| Gain of antenna intermediate UE (dBi) | 0 |
| Intermediate UE receiver Noise Figure (dB) | 9 |
| Antenna configuration | Omni directional antenna |

Table 6.6.2.2-4: CW RF parameters

| CW parameters | D1T1 | D2T2 |
| --- | --- | --- |
| Tx power (dBm) | If UL spectrum is used, UE Tx power is assumed, i.e. 23dB If DL spectrum is used, A-IoT reader Tx power is assumed, i.e. 33 dBm | Intermediate UE Tx power is assumed. |
| Antenna gain | Same as A-IoT reader | Same as intermediate UE |

- 6.6.2.3	NR UE/ A-IoT device RF characteristics

Assumptions relevant for modelling the NR UE and A-IoT device RF characteristics are captured in Table 6.6.2.3-1 and Table 6.6.2.3-2.

Table 6.6.2.3-1: NR UE RF parameters

| NR UE Parameter | Values for evaluation |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

Table 6.6.2.3-2: A-IoT device RF parameters

| A-IoT device parameters | Device 1 | Device 2a |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | [0] |
| A-IoT device reflection （backscatter）loss (dB) | OOK: -6 dB | OOK: -6 dB |
| A-IoT device power gain of reflection amplifier (dB) | N/A | 10(M) |
| A-IoT Device receiver sensitivity (dBm) Use this value to determine whether device can camp on the cell. | -36 | [-45] |
| A-IoT device noise figure (dB) | 24 | [20] |
| Guard band | 0PRB | 0PRB |

### 6.6.3	Co-existence simulation methodology

#### 6.6.3.1	Coexistence evaluation methodology

The coexistence evaluation methodology can be summarized as:

1)	Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions.

2)	UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader or intermediate UE based on coupling.

- NOTE: For A-IoT devices are associated to A-IoT reader or intermediate UE, there are two ways applied by companies:

(a) The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other. The receiving power of device meets the device receiver sensitivity.

(b) The device can access the reader with best receiving power among of 12 or 18 total readers in one building. The receiving power of device meets the device receiver sensitivity.

3)	Once association is done, round robin scheduling is used.

4) For inter-system interference (between A-IoT and NR):

-	If SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible.

-	Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

#### 6.6.3.2. SINR definition

For D1T1-A1and -B /D2T2-A1and -B

SINR definition for D2R:

- SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interference+residual CW interference after cancellation

- SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference+residual CW interference after cancellation

For the CW outside scenarios (D1T1-B and D2T2-B), the impact to SINR due to phase noise are also evaluated. The baseline D2R SNR for CW outside can be calculated by

baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterference+phase noisewithin total receiver bandwidth+residual CW interference after cancellationin linear scale

Phase noise can be modelled as XdBc/Hz, e.g., -112dBc/Hz compared to the received power for D2R.

The noise and intra-system interference are within total receiver bandwidth, and the residual CW interference after cancellation is in linear scale.

SINR definition for R2D:

- signal power of device to the noise and interference within 10MHz is baseline assumption

- Consider [180KHz] noise and interference bandwidth after BB LPF as optional

- 6.6.3.3	Coupling loss

The Coupling Loss (CL) is defined as the loss in signal between NR BS-to- NR UE, A-IoT reader -to- A-IoT device and intermediate UE -to- A-IoT device. CL is defined as the loss including propagation loss and antenna gains.

- 6.6.3.4 Impact from CW to NR

In 6.6.3.2, the SINR definition includes the impact from CW to D2R which is treat as intra-system interference. However, Whether and how the CW will influence the NR system also needs to be studied. In [R4-2], the impact of CW signal under different assumptions are investigated and the evaluation results are shown in figure 6.6.3.4-1.

Figure 6.6.3.4-1 Impact from CW to NR system [R4-2]

The results shows that when the CW is orthogonal with NR subcarrier, i.e., α=0, the influence of interference is much lower and also easier to be removed after FFT, only if the power of CW interference does not exceed the dynamic range of receiver.

Based on the analysis above, the impact from CW to NR is not modelled in the co-existence evaluation for simplicity.

- 6.6.4	Co-existence evaluation results

The simulation results submitted for the ambient IoT and NR co-existence study from all contributing companies in RAN4 were summarized into three attached files in [R4-1] to this report.

### 6.6.5	Summary of co-existence evaluation

The sub-clause captures the co-existence simulation results for ambient IoT categorized by scenarios and cases. The conclusions below are derived from the coexistence results.

#### 6.6.5.1 Scenario 1-1: D1T1-A2- NR UE only outdoors (R2D/ CW2D/ D2R: DL)

Scenario 1-1 considers D1T1 topology, all the NR UE outdoors and Readers/CW collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.6.5.1-1.

Table 6.6.5.1-1: Scenario 1-1 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| b | NR DL | reader | SINR degradation is less than 1dB under the assumption of 30dBc inband leakage for legacy NR gNB. Inter-system interference is negligible. 2 companies observed some SINR degradation under the assumption of 17dBc inband leakage for legacy NR gNB. |
| c | Reader | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

#### 6.6.5.2 Scenario 1-2: D1T1-A2- NR UE indoor (R2D/ CW2D/ D2R: DL)

Scenario 1-2 considers D1T1 topology, 10% and 100%(Optional)of the NR UE indoors and Reader/CW collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.6.5.2-1.

Table 6.6.5.2-1: Scenario 1-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| b | NR DL | reader | SINR degradation is less than 1dB under the assumption of 30dBc inband leakage for legacy NR gNB. Inter-system interference is negligible. 1 company observed some SINR degradation under the assumption of 17dBc inband leakage for legacy NR gNB. |
| c | Reader | NR DL | Under the assumption of 10% NR UE indoor, NR DL throughput degradation is observed. Under the assumption of 100% NR UE indoor, significant NR DL throughput degradation is observed. |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

#### 6.6.5.3 Scenario 2-1: D1T1-B- NR UE only outdoor (R2D: DL, CW2D/ D2R: UL)

Scenario 2-1 considers D1T1 topology, all the NR UE outdoors and Readers/CW non-collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.6.5.3-1.

Table 6.6.5.3-1: Scenario 2-1 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
| f | NR UL | reader | SINR degradation is less than 1dB. Inter-system interference is negligible. |
| c | Reader | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

#### 6.6.5.4 Scenario 2-2: D1T1-B- NR UE indoor (R2D: DL, CW2D/ D2R: UL)

Scenario 2-2 considers D1T1 topology, 10% and 100%(Optional)of the NR UE indoors and Readers/CW non-collocated, with DL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.6.5.4-1.

Table 6.6.5.4-1: Scenario 2-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
| f | NR UL | reader | Under the assumption of 10% and 100% NR UE indoor, significant SINR degradation is observed |
| c | Reader | NR DL | Under the assumption of 10% and 100% NR UE indoor, NR DL throughput degradation is observed. |
| d | NR DL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

#### 6.6.5.5 Scenario 5-1: D2T2-A2- NR UE only outdoor (R2D/ CW2D/ D2R: UL)

Scenario 5-1 considers D2T2 topology, all the NR UE outdoors and Readers/CW collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.6.5.5-1.

Table 6.6.5.5-1: Scenario 5-1 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
| f | NR UL | reader | SINR degradation is less than 1dB. Inter-system interference is negligible. |
| g | Reader | NR UL | NR UL throughput degradation is observed |
| h | NR UL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

#### 6.6.5.6 Scenario 5-2: D2T2-A2- NR UE indoor (R2D/ CW2D/ D2R: UL)

Scenario 5-2 considers D2T2 topology, 10% and 100%(Optional)of the NR UE indoors and Readers/CW collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.6.5.6-1.

Table 6.6.5.6-1: Scenario 5-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| e | Device | NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
| f | NR UL | reader | Significant SINR degradation is observed for both 10% NR indoor UE and 100% NR indoor UE assumption. |
| g | Reader | NR UL | NR UL throughput degradation is observed |
| h | NR UL | device | Under the assumption of 10% NR UE indoor, SINR degradation is larger than 1dB. Under the assumption of 100% NR UE indoor, significant SINR degradation is observed. |

#### 6.6.5.7 Scenario 6-1 (Optional): D2T2-B- NR UE only outdoor (R2D: UL, CW2D/ D2R: DL)

Scenario 6-1 considers D2T2 topology, all the NR UE outdoors and Readers/CW non-collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.6.5.7-1.

Table 6.6.5.7-1: Scenario 6-1 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |
| b | NR DL | reader | SINR degradation is less than 1dB under the assumption of 30dBc inband leakage for legacy NR gNB. Inter-system interference is negligible. |
| g | Reader | NR UL | NR UL throughput degradation is observed. |
| h | NR UL | device | SINR degradation is less than 1dB. Inter-system interference is negligible. |

#### 6.6.5.8 Scenario 6-2 (Optional): D2T2-B- NR UE indoor (R2D: UL, CW2D/ D2R: DL)

Scenario 6-2 considers D2T2 topology, 10% and 100%(Optional)of the NR UE indoors and Readers/CW non-collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.6.5.8-1.

Table 6.6.5.8-1: Scenario 6-2 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| b | NR DL | reader | SINR degradation is less than 1dB under the assumption of 30dBc inband leakage for legacy NR gNB. Inter-system interference is negligible. 1 company observed some SINR degradation under the assumption of 17dBc inband leakage for legacy NR gNB |
| g | Reader | NR UL | NR UL throughput degradation is observed. |
| h | NR UL | device | Under the assumption of 10% NR UE indoor, SINR degradation is larger than 1dB. Under the assumption of 100% NR UE indoor, significant SINR degradation is observed. |

#### 6.6.5.9 Scenario 3 (Optional): D1T1-A2- NR UE indoor (R2D: UL, CW2D/ D2R: DL)

Scenario 3 optionally considers D1T1 topology, 10% and 100%(Optional)of the NR UE indoors and Readers/CW collocated, with UL spectrum applying to reader-to-device link. The conclusions of each cases are listed per scenario in Table 6.6.5.9-1.

Table 6.6.5.9-1: Scenario 3 co-existence conclusions

| Case No. | Aggressor | Victim | Co-existence conclusion |
| --- | --- | --- | --- |
| a | Device | NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
| b | NR DL | reader | 1 company provides simulation results based on assumption of 30dBc NR gNB inband leakage and observed less than 1dB SINR degradation. 1 company provides simulation results based on assumption of 17dBc NR gNB inband leakage and observed larger than 1 dB SINR degradation. |
| g | Reader | NR UL | NR UL throughput degradation is observed. |
| h | NR UL | device | 1 company provides simulation results based on assumption of 10% NR UE indoor and observed less than 1dB SINR degradation 1 company provides simulation results based on assumption of 100% NR UE indoor and observed larger than 1 dB SINR degradation. |

#### 6.6.5.10	General remarks on coexistence findings

For D1T1 deployment scenario 1-1 (R2D, CW2D/D2R: DL) and scenario 2-1(R2D: DL, CW2D/D2R: UL), which has no legacy NR UE indoor, negligible performance degradation for NR system is observed, and less than 1 dB SINR degradation due to inter-system interference from NR to ambient IOT system is observed. Co-existence between ambient IOT and NR is feasible, based on performance metric of NR throughput degradation and ambient IOT SINR degradation, no additional coexistence measures are required. Ambient IOT intra-system performance evaluated by link level simulation is also one of the key aspects affecting ambient IOT system performance, which will have impact to the outage of ambient IOT system.

For the deployment scenarios with legacy NR UE indoor (scenario 1-2, 2-2, 5-2, 6-2), which either significant NR throughput degradation or significant SINR degradation for ambient IOT system is observed, some existing mechanisms based on gNB implementation can be considered to avoid/mitigate the interference. For example, for RRC idle mode, NR UEs can camp on a frequency different from ambient IOT system based on frequency priority configured by network; for RRC connected mode, network can handover NR UEs to a different cell (e.g. indoor gNB on a different frequency) based on existing measurement report triggering and event. Besides, other interference management solutions to reduce the inter-system interference from ambient IOT to proximity NR UE and vice versa can also be considered.

For the D2T2 deployment scenarios (scenario 5-1, 5-2, 6-1, 6-2), NR UL throughput degradation is observed. The mechanisms to mitigate the interference can be considered. For example, enhanced emission requirements for intermediate UE or power control for intermediate UE [R4-3] [R4-4].

Additionally, RAN4 has also considered using guard band to mitigate the interference issue between ambient system and NR system.

------------End of the Change-------------