3GPP TSG-RAN5 Meeting #105	R5-246173

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3473 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TT value to LTM inter TC 6.3.2.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MUTT analysis is finished in R5-246171. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update parameters based on the TT analysis. - Delete editors note - As discussed in R5-246170, update PDCCH configuration. Since we need to analysis the demodulation criteria, concrete PDCCH config is needed. Considering PDCCH demodulation is not the requirement for this TC, aggregation level 16 is recommended. This will update to the test case as well. - Correct wrong power calculations in Table 6.3.2.4.2.5-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case remain incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 39.903 CR 0850, 0855 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Power calculations in Table 6.3.2.4.2.5-2 is incorrect obviously. This change should also be implemented in RAN4 spec 38.133. RAN4 dependency, R4-2417901 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

6.3.2.4.2	PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM

Editor's Note: This test case is incomplete in following aspects:

LTM specific configurations are missing.

Test applicability is missing in TS 38.522.

[ ] still exist in this case

PRACH configuration in Table 6.3.2.4.2.5-1 and Table 6.3.2.4.2.5-2 needs to be clarified in RAN4

Interruption requirement needs to be clarified by RAN4

<<< Skip unchanged parts>>>

6.3.2.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.3.2.4.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-5 Table H.3.1-7 with conditions INTER-FREQ, SSB Index Table A.7.1-1 with PRACH.6 FR1 PDCCH configuration: Table A.1.3.1-1 CCR 1.2 FDD for test 1, Table A.1.3.2-1 CCR 1.2 TDD for test 2 and Table A.1.3.2-2 CCR 2.4 FDD for test 3 with following exceptions: aggregation level 16, allocated RB 48. |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.3.2.4.2.4.3-2: ReportConfigNR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -22 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.4.2.4.3-3: MeasGapConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-70 with condition GAP_FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR1 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mgrp | ms80 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.3.2.4.2.5	Test requirement

Both PDCCH order RACH delay, transmit timing requirement and the interruption requirements are tested by using the parameters in table 6.3.2.4.2.5-1, and 6.3.2.4.2.5-2.Table 6.3.2.4.2.5-1: General test parameters for Inter-frequency PDCCH-ordered RACH test from FR1 to FR1 from FR1 to FR1

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |
|  | Neighbouring cell |  | Cell 2 |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 1 |  |  |
| A3-Offset |  | dB | -11 |  | Original value is -6 dB, change to -11 dB after TT Analysis |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Measurement gap periodicity |  | ms | 80 |  |  |
| DRX |  |  | OFF |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 2 s |  | RTD between cells is less than CP |
| reportQuantityRS-Indexes |  |  | Rsrp |  |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |  |
| includeBeamMeasurements |  |  | True |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 2 |  | Cell 2 |
|  | PRACH configuration |  | FR2 PRACH configuration 5 |  |  |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |
|  | n-TimingAdvanceOffset | Tc | 25600 |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | N/A | As specified in TS 38.133 [6] clause A.3.16B. In test 1, CandidateTCI-State#1 is configured for early TCI state activation. In test 2, CandidateTCI-State#1 is configured for TCI state indication in cell switch command. |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.1 |  |  |
| T2 |  | s | 0.2 |  |  |

Table 6.3.2.4.2.5-2: Cell specific test parameters for NR FR1-FR1 Inter-frequency PDCCH-ordered RACH test

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1-T2 | T1-T2 |
| NR RF Channel Number |  |  |  | 1 | 2 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DRx Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  |
| TRS configuration for serving cell |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | 8 | 4 |
|  |  |  | dB | 8 | 4 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -94 |
|  | Config 3 |  | dBm/SCS | -87 | -91 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -64.59 |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -58.49 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

<<< END OF CHANGES 1>>>