3GPP TSG-RAN5 Meeting #105	R5-246265

# Orlando, US, 18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5523 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PC1 operation for bands 40, 42 and 106 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of PC1 operation for bands 40, 42 and 106 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PC1 operation for bands 40, 42 and 106 in: - 6.2.2_1 Maximum Output Power for HPUE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1 operation in bands 40, 42 and 106 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 6.2.2_1	Maximum Output Power for HPUE

Editor's note: For a transition period until RAN5#81, this test case in version 15.2.0 of 36.521-1 is allowed to be used.

#### 6.2.2_1.1	Test purpose

Same test purpose as in clause 6.2.2.1.

#### 6.2.2_1.2	Test applicability

This test case applies to all types of E-UTRA UE Power Class 1 or Power Class 2 release 10 and forward.

#### 6.2.2_1.3	Minimum conformance requirements

The following defines the maximum output power for any transmission bandwidth within the channel bandwidth for Power Class 1 and Power Class 2 UE. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2_1.3-1: HPUE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 31 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 38 | N/A | N/A | 26 | ±2 | N/A | N/A | N/A | N/A |
| 40 | 31 | +2/-31 | 26 | ±2 | N/A | N/A | N/A | N/A |
| 41 | N/A | N/A | 26 | ±22 | N/A | N/A | N/A | N/A |
| 42 | 31 | +2/-31 | 26 | +2/-3 | N/A | N/A | N/A | N/A |
| 72 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 87 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 88 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 106 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| Note 1: The above tolerances are applicable for UE(s) that support up to 4 E-UTRA operating bands. For UE(s) that support 5 or more E-UTRA bands the maximum output power is expected to decrease with each additional band and is FFS Note 2: For transmission bandwidths (Figure 5.4.2-1, Table 5.4.4-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: N/A Note 4: PPowerClass is the maximum UE power specified without taking into account the tolerance Note 5-6: N/A |  |  |  |  |  |  |  |  |

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in [5] of 23 dBm or lower is indicated in the cell or if the TDD frame configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

The normative reference for this requirement is TS 36.101 clause 6.2.2.

#### 6.2.2_1.4	Test description

Same test description as in clause 6.2.2.4 with the following exceptions:

-	Instead of Table 6.2.2.4.1-1 use Table 6.2.2_1.4-1

Table 6.2.2_1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 1 | 1 |
| 1.4MHz |  |  | QPSK | 5 | 5 |
| 3MHz |  |  | QPSK | 1 | 1 |
| 3MHz |  |  | QPSK | 4 | 4 |
| 5MHz |  |  | QPSK | 1 | 1 |
| 5MHz |  |  | QPSK | 8 | 8 |
| 10MHz |  |  | QPSK | 1 | 1 |
| 10MHz |  |  | QPSK | 12 | 12 |
| 15MHz |  |  | QPSK | 1 | 1 |
| 15MHz |  |  | QPSK | 16 | 16 |
| 20MHz |  |  | QPSK | 1 | 1 |
| 20MHz |  |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For E-UTRA bands not applied with Note 2 in Table 6.2.2_1.3-1: The 1 RB allocation shall be tested at RB#0 for low and mid range, RB #max for high range test frequency. The RBstart of non-1RB allocation shall be RB #0 for low and mid range, RB# (max +1 - RB allocation) for high range test frequency. Note 3: For E-UTRA bands applied with Note 2 in Table 6.2.2_1.3-1: - If the test channel bandwidth is larger than 4MHz, then the 1 RB allocation shall be tested at both RB #0 and RB #max. - If the test channel bandwidth is smaller or equal to 4MHz, then the 1 RB allocation shall be tested at RB #0. - If the test channel bandwidth = (FUL_high - FUL_low) specified by the operating band, then only one frequency range shall be tested and the 1 RB allocation shall be tested at RB #0, RB # and RB #max. - For non-1RB allocation, test frequency is middle range, and the RBstart shall be RB #0. |  |  |  |  |  |

-	Message contents are according to TS 36.508 [7] subclause 4.4.3.2 with the following exceptions:

Table 6.2.2_1.4-2: SystemInformationBlockType1: Test point 1

| Derivation Path: 36.508 [7] clause 4.4.3.2, Table 4.4.3.2-1 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 31 dBm 26 dBm Not present | for Power Class 1 other than Band 14 for Power Class 2 other than Band 41 for Power Class 2 Band 41 and for Power Class 1 Band 14 See Table 6.2.2_1.3-1 |  |

#### 6.2.2_1.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2_1.5-1

Table 6.2.2_1.5-1: HPUE Power Class test requirements

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 31 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 38 | N/A | N/A | 26 | ±2.7 | N/A | N/A | N/A | N/A |
| 40 | 31 | +2.7/-3.71 | 26 | ±2.7 | N/A | N/A | N/A | N/A |
| 41 | N/A | N/A | 26 | ±2.72 | N/A | N/A | N/A | N/A |
| 42 | 31 | +2.7/-3.71 | 26 | +3/-4 | N/A | N/A | N/A | N/A |
| 72 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 87 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 88 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 106 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| Note 1: The above tolerances are applicable for UE(s) that support up to 4 E-UTRA operating bands. For UE(s) that support 5 or more E-UTRA bands the maximum output power is expected to decrease with each additional band and is FFS Note 2: For transmission bandwidths (Figure 5.4.2-1, Table 5.4.4-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |  |  |  |  |  |  |

-------- End of changes --------