3GPP TSG-RAN5 Meeting #105	R5-246266

# Orlando, US, 18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5524 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PC1 operation for band 12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of PC1 operation for band 12 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PC1 operation for band 12 in: - 6.2.2_1 Maximum Output Power for HPUE - 6.2.2_4 Additional Maximum Power Reduction (A-MPR) for HPUE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1 operation in band 12 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2_1, 6.2.4_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 36.905 ... CR 0275 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Test point analysis in R5-246264 (CR 0275) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 6.2.2_1	Maximum Output Power for HPUE

Editor's note: For a transition period until RAN5#81, this test case in version 15.2.0 of 36.521-1 is allowed to be used.

#### 6.2.2_1.1	Test purpose

Same test purpose as in clause 6.2.2.1.

#### 6.2.2_1.2	Test applicability

This test case applies to all types of E-UTRA UE Power Class 1 or Power Class 2 release 10 and forward.

#### 6.2.2_1.3	Minimum conformance requirements

The following defines the maximum output power for any transmission bandwidth within the channel bandwidth for Power Class 1 and Power Class 2 UE. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2_1.3-1: HPUE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 14 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 31 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 38 | N/A | N/A | 26 | ±2 | N/A | N/A | N/A | N/A |
| 40 | N/A | N/A | 26 | ±2 | N/A | N/A | N/A | N/A |
| 41 | N/A | N/A | 26 | ±22 | N/A | N/A | N/A | N/A |
| 42 | N/A | N/A | 26 | +2/-3 | N/A | N/A | N/A | N/A |
| 72 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 87 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| 88 | 31 | +2/-31 | N/A | N/A | N/A | N/A | N/A | N/A |
| Note 1: The above tolerances are applicable for UE(s) that support up to 4 E-UTRA operating bands. For UE(s) that support 5 or more E-UTRA bands the maximum output power is expected to decrease with each additional band and is FFS Note 2: For transmission bandwidths (Figure 5.4.2-1, Table 5.4.4-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: N/A Note 4: PPowerClass is the maximum UE power specified without taking into account the tolerance Note 5-6: N/A |  |  |  |  |  |  |  |  |

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in [5] of 23 dBm or lower is indicated in the cell or if the TDD frame configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

The normative reference for this requirement is TS 36.101 clause 6.2.2.

#### 6.2.2_1.4	Test description

Same test description as in clause 6.2.2.4 with the following exceptions:

-	Instead of Table 6.2.2.4.1-1 use Table 6.2.2_1.4-1

Table 6.2.2_1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 1 | 1 |
| 1.4MHz |  |  | QPSK | 5 | 5 |
| 3MHz |  |  | QPSK | 1 | 1 |
| 3MHz |  |  | QPSK | 4 | 4 |
| 5MHz |  |  | QPSK | 1 | 1 |
| 5MHz |  |  | QPSK | 8 | 8 |
| 10MHz |  |  | QPSK | 1 | 1 |
| 10MHz |  |  | QPSK | 12 | 12 |
| 15MHz |  |  | QPSK | 1 | 1 |
| 15MHz |  |  | QPSK | 16 | 16 |
| 20MHz |  |  | QPSK | 1 | 1 |
| 20MHz |  |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For E-UTRA bands not applied with Note 2 in Table 6.2.2_1.3-1: The 1 RB allocation shall be tested at RB#0 for low and mid range, RB #max for high range test frequency. The RBstart of non-1RB allocation shall be RB #0 for low and mid range, RB# (max +1 - RB allocation) for high range test frequency. Note 3: For E-UTRA bands applied with Note 2 in Table 6.2.2_1.3-1: - If the test channel bandwidth is larger than 4MHz, then the 1 RB allocation shall be tested at both RB #0 and RB #max. - If the test channel bandwidth is smaller or equal to 4MHz, then the 1 RB allocation shall be tested at RB #0. - If the test channel bandwidth = (FUL_high - FUL_low) specified by the operating band, then only one frequency range shall be tested and the 1 RB allocation shall be tested at RB #0, RB # and RB #max. - For non-1RB allocation, test frequency is middle range, and the RBstart shall be RB #0. |  |  |  |  |  |

-	Message contents are according to TS 36.508 [7] subclause 4.4.3.2 with the following exceptions:

Table 6.2.2_1.4-2: SystemInformationBlockType1: Test point 1

| Derivation Path: 36.508 [7] clause 4.4.3.2, Table 4.4.3.2-1 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 31 dBm 26 dBm Not present | for Power Class 1 other than Band 14 for Power Class 2 other than Band 41 for Power Class 2 Band 41 and for Power Class 1 Band 14 See Table 6.2.2_1.3-1 |  |

#### 6.2.2_1.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2_1.5-1

Table 6.2.2_1.5-1: HPUE Power Class test requirements

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 14 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 31 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 38 | N/A | N/A | 26 | ±2.7 | N/A | N/A | N/A | N/A |
| 40 | N/A | N/A | 26 | ±2.7 | N/A | N/A | N/A | N/A |
| 41 | N/A | N/A | 26 | ±2.72 | N/A | N/A | N/A | N/A |
| 42 | N/A | N/A | 26 | +3/-4 | N/A | N/A | N/A | N/A |
| 72 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 87 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| 88 | 31 | +2.7/-3.71 | N/A | N/A | N/A | N/A | N/A | N/A |
| Note 1: The above tolerances are applicable for UE(s) that support up to 4 E-UTRA operating bands. For UE(s) that support 5 or more E-UTRA bands the maximum output power is expected to decrease with each additional band and is FFS Note 2: For transmission bandwidths (Figure 5.4.2-1, Table 5.4.4-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

### 6.2.4_1	Additional Maximum Power Reduction (A-MPR) for HPUE

#### 6.2.4_1.1	Test purpose

Same test purpose as in clause 6.2.4.1 with the follow exception:

-	Instead of Table 6.2.2.3-1  use Table 6.2.2_1.3-1

#### 6.2.4_1.2	Test applicability

The requirements of this test apply in test case 6.6.2.2 Additional Spectrum Emission Mask and 6.6.3.3 Additional spurious emissions for network signalled values NS_04, NS_06 and NS_44 to all types of E-UTRA Power Class 1 or Power Class 2 UE release 10 and forward.

NOTE:	As a result TC 6.2.4_1 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4_1.3	Minimum conformance requirements

For UE Power Class 1 or Power Class 2 the specific requirements and identified sub-clauses are specified in Table 6.2.4_1.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified in Table 6.2.4_1.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3_1. For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2.5_1 apply.

The normative reference for this requirement is TS 36.101 clause 6.2.4.

Table 6.2.4_1.3-1: Additional Maximum Power Reduction (A-MPR) / Spectrum Emission requirements

| Network Signalling value | Requirements (sub-clause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_04 | 6.6.2.2.3.2, 6.6.3.3.3.1 | 41 | 5,10,15,20 | Table 6.2.4_1.3-2 |  |
| NS_06 | 6.6.2.2.3.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.4.2-1 | Table 6.2.4_1.3-4 (NOTE 2) |
| NS_44 | 6.6.3.3.33 | 38 (Note 1) | 5, 10, 15, 20 | Table 6.2.4_1.3-3 |  |
| NOTE 1: Applicable for power class 2 UE in E-UTRA carrier with channel bandwidth is confined within 2570 MHz and 2615 MHz. NOTE 2: Applicable only for power class 1 UE in Band 12. Not applicable for all other cases. |  |  |  |  |  |

Table 6.2.4_1.3-2: A-MPR requirements for "NS_04" for Power Class 2 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | ≤ 2500.5 |  |  |  |  | > 2500.5 |
|  | RBstart | 0 - 8 |  |  | 9 - 24 |  | 0 - 24 |
|  | LCRB [RBs] | > 0 |  |  | > 0 |  | > 0 |
|  | A-MPR [dB] | ≤ 3 |  |  | 0 |  | 0 |
| 10 | Fc [MHz] | ≤ 2504 |  |  |  |  | > 2504 |
|  | RBstart | 0 – 8 |  |  | 9 - 35 | 36 - 49 | 0 - 49 |
|  | LCRB [RBs] | ≤ 15 | > 15 and < 25 | ≥ 25 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 45 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 3 | ≤ 1 | 0 | 0 |
| 15 | Fc [MHz] | ≤ 2510.8 |  |  |  |  | > 2510.8 |
|  | RBstart | 0 – 13 |  |  | 14 – 59 | 60 – 74 | 0 - 74 |
|  | LCRB [RBs] | ≤ 18 | > 18 and < 36 | ≥ 36 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 62 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 3 | 0 | 0 |
| 20 | Fc [MHz] | ≤ 2517.5 |  |  |  |  | > 2517.5 |
|  | RBstart | 0 – 22 |  |  | 23 – 76 | 77 – 99 | 0 - 99 |
|  | LCRB [RBs] | ≤ 18 | > 18 and < 40 | ≥ 40 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 86 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 3 | 0 | 0 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |  |  |  |

For a power class 2 capable UE operating in Band 41, A-MPR according to Table 6.2.4.3-2, Table 6.2.4.3-3, or Table 6.2.4.3-4 for power class 3 is allowed when an IE P-max as defined in [5] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6.

Table 6.2.4_1.3-3: A-MPR requirements for "NS_44" for Power Class 2 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Fc [MHz] | ≥2605 |  |  |  | <2605 |
|  | RBstart | 0 – 5 |  | 38-49 |  | 0 - 49 |
|  | LCRB [RBs] | ≥ 45 |  | < 13 |  | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | NA |  | N/A |
|  | A-MPR [dB] | ≤ 1 |  | ≤ 1 |  | 0 |
| 15 | Fc [MHz] | ≥2597.5 |  |  |  | <2597.5 |
|  | RBstart | 0 – 18 | 19-30 |  | 54-74 | 0 - 74 |
|  | LCRB [RBs] | N/A | ≥ 45& <56 |  | < 17 | > 0 |
|  | RBstart + LCRB [RBs] | ≥ 63 | N/A |  | NA | N/A |
|  | A-MPR [dB] | ≤ 1 | ≤ 1 |  | ≤ 1 | 0 |
| 20 | Fc [MHz] | ≥2590 |  |  |  | <2590 |
|  | RBstart | 0 – 31 | 32-55 |  | 70-99 | 0 - 99 |
|  | LCRB [RBs] | N/A | ≥ 45& <68 |  | < 19 | > 0 |
|  | RBstart + LCRB [RBs] | ≥ 76 | N/A |  | NA | N/A |
|  | A-MPR [dB] | ≤ 2 | ≤ 1 |  | ≤ 1 | 0 |

Table 6.2.4_1.3-4: A-MPR requirements for "NS_06" for Power Class 1 UE in Band 12

| Channel bandwidth [MHz] | LCRB | Allocation positions | Modulation / A-MPR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 1.4 | all | all | 0 | 0 | 0 | 0 |
| 3 | = 15 | RBstart = 0 | 1 | 0.5 | 0 | 0 |
| 5 | 15 | RBstart 2 or RBstart + LCRB 23 | 1 | 0.5 | 0 | 0 |
| 10 | 18 | RBstart 2 or RBstart + LCRB 48 | 1 | 0.5 | 0 | 0 |

#### 6.2.4_1.4	Test description

Same test description as in clause 6.2.4.4 with the following exceptions:

Instead of Table 6.2.4.4.1-1 to 6.2.4.4.1-26  use Table 6.2.4_1.4-1 or Table 6.2.4_1.4-2.

Table 6.2.4_1.4-1: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range Additional test frequencies: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_05 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 (Note 1) | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 (Note 1) | 5MHz |  |  | QPSK | 16 | 9 |
| 3 (Note 2) | 5MHz |  |  | QPSK | 25 | 0 |
| 4 (Note 1) | 5MHz |  |  | 16QAM | 16 | 9 |
| 5 (Note 2) | 5MHz |  |  | 16QAM | 25 | 0 |
| 6 (Note 1) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 1) | 10MHz |  |  | QPSK | 1 | 49 |
| 8 (Note 1) | 10MHz |  |  | QPSK | 24 | 0 |
| 9 (Note 1) | 10MHz |  |  | QPSK | 40 | 10 |
| 10 | 10MHz |  |  | QPSK | 50 | 0 |
| 11 | 10MHz |  |  | 16QAM | 50 (Note 3) | 0 |
| 12 (Note 1) | 15MHz |  |  | QPSK | 1 | 0 |
| 13 (Note 1) | 15MHz |  |  | QPSK | 1 | 74 |
| 14 (Note 1) | 15MHz |  |  | QPSK | 32 | 0 |
| 15 (Note 1) | 15MHz |  |  | QPSK | 60 | 15 |
| 16 | 15MHz |  |  | QPSK | 75 | 0 |
| 17 | 15MHz |  |  | 16QAM | 75 (Note 3) | 0 |
| 18 (Note 1) | 20MHz |  |  | QPSK | 1 | 0 |
| 19 (Note 1) | 20MHz |  |  | QPSK | 1 | 99 |
| 20 (Note 1) | 20MHz |  |  | QPSK | 36 | 0 |
| 21 (Note 1) | 20MHz |  |  | QPSK | 75 | 25 |
| 22 | 20MHz |  |  | QPSK | 100 | 0 |
| 23 | 20MHz |  |  | 16QAM | 100 (Note 3) | 0 |
| Note 1: Only applicable for Low Range. Note 2: Only applicable for Mid, High Range and additional test frequencies. Note 3: Applies only for UE-Categories ≥2. |  |  |  |  |  |  |

Table 6.2.4_1.4-2: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 25 |
| 2 | 5MHz |  |  | QPSK | 8 |
| 3 | 5MHz |  |  | 16QAM | 8 |
| 4 | 10MHz |  |  | QPSK | 50 |
| 5 | 10MHz |  |  | QPSK | 12 |
| 6 | 10MHz |  |  | 16QAM | 12 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |

Table 6.2.4_1.4-3: Test Configuration Table (network signalled value "NS_44")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low Range, Mid Range Additional test frequencies: For 5 MHz bandwidth: 2612.5 MHz, NUL= 38175 For 10 MHz bandwidth: 2610 MHz, NUL= 38150 For 15 MHz bandwidth: 2607.5 MHz, NUL= 38125 For 20 MHz bandwidth: 2605 MHz, NUL= 38100 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_44 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Note 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 24 |
| 2 | Note 1 | 5MHz |  |  | QPSK | 25 | 0 |
| 3 | Note 1 | 5MHz |  |  | 16QAM | 1 | 24 |
| 4 | Note 1 | 5MHz |  |  | 16QAM | 25 | 0 |
| 5 | Note 1 | 10MHz |  |  | QPSK | 1 | 49 |
| 6 | Note 1 | 10MHz |  |  | QPSK | 45 | 5 |
| 7 | Low/Mid | 10MHz |  |  | QPSK | 50 | 0 |
| 8 | Note 1 | 10MHz |  |  | 16QAM | 1 | 49 |
| 9 | Note 1 | 10MHz |  |  | 16QAM | 45 | 5 |
| 10 | Low/Mid | 10MHz |  |  | 16QAM | 50 | 0 |
| 11 | Note 1 | 15MHz |  |  | QPSK | 75 | 0 |
| 12 | Note 1 | 15MHz |  |  | QPSK | 45 | 30 |
| 13 | Note 1 | 15MHz |  |  | QPSK | 1 | 74 |
| 14 | Low/Mid | 15MHz |  |  | QPSK | 75 | 0 |
| 15 | Note 1 | 15MHz |  |  | 16QAM | 75 | 0 |
| 16 | Note 1 | 15MHz |  |  | 16QAM | 45 | 30 |
| 17 | Note 1 | 15MHz |  |  | 16QAM | 1 | 74 |
| 18 | Low/Mid | 15MHz |  |  | 16QAM | 75 | 0 |
| 19 | Mid/Note 1 | 20MHz |  |  | QPSK | 100 | 0 |
| 20 | Mid/Note 1 | 20MHz |  |  | QPSK | 45 | 55 |
| 21 | Mid/Note 1 | 20MHz |  |  | QPSK | 1 | 99 |
| 22 | Low | 20MHz |  |  | QPSK | 100 | 0 |
| 23 | Mid/Note 1 | 20MHz |  |  | 16QAM | 100 | 0 |
| 24 | Mid/Note 1 | 20MHz |  |  | 16QAM | 45 | 55 |
| 25 | Mid/Note 1 | 20MHz |  |  | 16QAM | 1 | 99 |
| 26 | Low | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: Only applicable for additional test frequencies. |  |  |  |  |  |  |  |

Table 6.2.4_1.4-4: Test Configuration Table (network signalled value "NS_06") for Power Class 1 UE in Band 12

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 1.4MHz, 3MHz, 5MHz, 10MHz |  |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low | 1.4MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | High | 1.4MHz |  |  | QPSK | 1 | 5 |
| 3 | Low | 1.4MHz |  |  | QPSK | 6 | 0 |
| 4 | Low | 1.4MHz |  |  | 16QAM | 1 | 0 |
| 5 | High | 1.4MHz |  |  | 16QAM | 1 | 5 |
| 6 | Low | 1.4MHz |  |  | 16QAM | 6 | 0 |
| 7 | Low | 1.4MHz |  |  | 64QAM | 1 | 0 |
| 8 | High | 1.4MHz |  |  | 64QAM | 1 | 5 |
| 9 | Low | 1.4MHz |  |  | 64QAM | 6 | 0 |
| 10 | Low | 1.4MHz |  |  | 256QAM | 1 | 0 |
| 11 | High | 1.4MHz |  |  | 256QAM | 1 | 5 |
| 12 | Low | 1.4MHz |  |  | 256QAM | 6 | 0 |
| 13 | Low | 3MHz |  |  | QPSK | 15 | 0 |
| 14 | Low | 3MHz |  |  | 16QAM | 15 | 0 |
| 15 | Low | 3MHz |  |  | 64QAM | 15 | 0 |
| 16 | Low | 3MHz |  |  | 256QAM | 15 | 0 |
| 17 | Low | 5MHz |  |  | QPSK | 25 | 0 |
| 18 | Low | 5MHz |  |  | 16QAM | 25 | 0 |
| 19 | Low | 5MHz |  |  | 64QAM | 25 | 0 |
| 20 | Low | 5MHz |  |  | 256QAM | 25 | 0 |
| 21 | Low | 10MHz |  |  | QPSK | 50 | 0 |
| 22 | Low | 10MHz |  |  | 16QAM | 50 | 0 |
| 23 | Low | 10MHz |  |  | 64QAM | 50 | 0 |
| 24 | Low | 10MHz |  |  | 256QAM | 50 | 0 |

#### 6.2.4_1.5	Test requirements

The maximum output power, derived in step 2 of clause 6.2.4_1.4.2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.4_1.5-1 and 6.2.4_1.5-2. The allowed A-MPR values specified in Table 6.2.4_1.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3_1. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5_1.3-1 apply.

Table 6.2.4_1.5-1: HPUE Power Class 1 test requirements (network signalled value "NS_06")

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) |
| --- | --- | --- | --- |
| 1 | 14 | 31 | +2.7 / -4.7 |
| 2 | 14 | 31 | +2.7 / -3.7 |
| 3 | 14 | 31 | +2.7 / -4.7 |
| 4 | 14 | 31 | +2.7 / -4.7 |
| 5 | 14 | 31 | +2.7 / -3.7 |
| 6 | 14 | 31 | +2.7 / -4.7 |

Table 6.2.4_1.5-2: UE Power Class 2 test requirements (network signalled value "NS_04")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 3 | 1.5 | 21.5 | 2.0 | 28.7 | 18.8 |
| 2 | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 3 | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 4 | 2 | 0 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 5 | 2 | 0 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 6 | 0 | 5 | 1.5 | 19.5 | 3.5 | 28.7 | 15.3 |
| 7 | 0 | 0 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 8 | 1 | 2 | 0 | 23 | 2.0 | 28.7 | 20.3 |
| 9 | 1 | 1 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 10 (Low Range) | 1 | 3 | 0 | 22 | 2.0 | 28.7 | 19.3 |
| 10 (Mid, High Range, 2504.1 MHz) | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 11 (Low Range) | 2 | 3 | 0 | 24 | 2.0 | 28.7 | 18.3 |
| 11 (Mid, High Range, 2504.1 MHz ) | 2 | 0 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 12 | 0 | 5 | 1.5 | 19.5 | 3.5 | 28.7 | 15.3 |
| 13 | 0 | 0 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 14 | 1 | 2 | 0 | 23 | 2.0 | 28.7 | 20.3 |
| 15 | 1 | 3 | 0 | 22 | 2.0 | 28.7 | 19.3 |
| 16 (Low Range) | 1 | 4 | 0 | 21 | 2.0 | 28.7 | 18.3 |
| 16 (Mid, High Range, 2510.9 MHz) | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 17 (Low Range) | 2 | 4 | 0 | 24 | 2.5 | 28.7 | 16.8 |
| 17 (Mid, High Range, 2510.9 MHz) | 2 | 0 | 0 | 25 | 2.0 | 28.7 | 21.3 |
| 18 | 0 | 5 | 1.5 | 19.5 | 3.5 | 28.7 | 15.3 |
| 19 | 0 | 0 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 20 | 1 | 2 | 0 | 23 | 2.0 | 28.7 | 20.3 |
| 21 | 1 | 3 | 0 | 22 | 2.0 | 28.7 | 19.3 |
| 22 (Low Range) | 1 | 4 | 0 | 21 | 2.0 | 28.7 | 18.3 |
| 22 (Mid, High Range, 2517.6 MHz) | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 23 (Low Range) | 2 | 4 | 0 | 20 | 2.5 | 28.7 | 16.8 |
| 23 (Mid, High Range, 2517.6 MHz) | 2 | 0 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4_1.5-3: UE Power Class 2 test requirements (network signalled value "NS_44")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 2 | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 3 | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 4 | 2 | 0 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 5 | 0 | 1 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 6 | 1 | 1 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 7 | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 8 | 1 | 1 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 9 | 2 | 1 | 0 | 23 | 2.0 | 28.7 | 20.3 |
| 10 | 2 | 0 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 11 | 1 | 1 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 12 | 2 | 1 | 0 | 23 | 2.0 | 28.7 | 20.3 |
| 13 | 0 | 1 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 14 | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 15 | 2 | 1 | 0 | 23 | 2.0 | 28.7 | 20.3 |
| 16 | 2 | 1 | 0 | 23 | 2.0 | 28.7 | 20.3 |
| 17 | 1 | 1 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 18 | 2 | 0 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 19 | 1 | 2 | 0 | 23 | 2.0 | 28.7 | 20.3 |
| 20 | 1 | 1 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 21 | 0 | 1 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 22 | 1 | 0 | 0 | 25 | 2.0 | 28.7 | 22.3 |
| 23 | 2 | 2 | 0 | 22 | 2.0 | 28.7 | 19.3 |
| 24 | 2 | 1 | 0 | 23 | 2.0 | 28.7 | 20.3 |
| 25 | 1 | 1 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| 26 | 2 | 0 | 0 | 24 | 2.0 | 28.7 | 21.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4_1.5-4: UE Power Class 1 test requirements (network signalled value "NS_06") in Band 12

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 31 | 2.0 | 33.7 | 27.3 |
| 2 | 0 | 0 | 0 | 31 | 2.0 | 33.7 | 27.3 |
| 3 | 1 | 0 | 0 | 30 | 2.0 | 33.7 | 26.3 |
| 4 | 1 | 0 | 0 | 30 | 2.0 | 33.7 | 26.3 |
| 5 | 1 | 0 | 0 | 30 | 2.0 | 33.7 | 26.3 |
| 6 | 2 | 0 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 7 | 2 | 0 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 8 | 2 | 0 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 9 | 3 | 0 | 0 | 28 | 2.0 | 33.7 | 24.3 |
| 10 | 0 | 0 | 0 | 31 | 2.0 | 33.7 | 27.3 |
| 11 | 0 | 0 | 0 | 31 | 2.0 | 33.7 | 27.3 |
| 12 | 5 | 0 | 0 | 26 | 2.0 | 33.7 | 22.3 |
| 13 | 1 | 1 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 14 | 2 | 0.5 | 0 | 28.5 | 2.0 | 33.7 | 24.8 |
| 15 | 3 | 0 | 0 | 28 | 2.0 | 33.7 | 24.3 |
| 16 | 5 | 0 | 0 | 26 | 2.0 | 33.7 | 22.3 |
| 17 | 1 | 1 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 18 | 2 | 0.5 | 0 | 28.5 | 2.0 | 33.7 | 24.8 |
| 19 | 3 | 0 | 0 | 28 | 2.0 | 33.7 | 24.3 |
| 20 | 5 | 0 | 0 | 26 | 2.0 | 33.7 | 22.3 |
| 21 | 1 | 1 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 22 | 2 | 0.5 | 0 | 28.5 | 2.0 | 33.7 | 24.8 |
| 23 | 3 | 0 | 0 | 28 | 2.0 | 33.7 | 24.3 |
| 24 | 5 | 0 | 0 | 26 | 2.0 | 33.7 | 22.3 |

-------- End of changes --------