3GPP TSG-RAN5 Meeting #105	R5-246370

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4679 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to CHO TC 8.1.4.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With current FR2 power level configuration, in T2, the A5 Entering condition 1 is not satisified as the PCell (-91+4) is still greater than Thresh1 (-90). Therefore the Thresh1 needs to be adjusted to an higher value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The a5-Threshold1 for FR2 was modified to -83. 2. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors will exist these cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

8.1.4.4.1	Conditional handover / Success / A3 / A5 / A3+A5

8.1.4.4.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supporting conditional handover and receiving an RRCReconfiguration message including ConditionalReconfiguration }

ensure that {

when { any CHO execution condition is not satisfied }
    then { UE maintains connection with source gNB and starts evaluating the CHO execution conditions for the candidate cell(s) }

}

(2)

with { UE in NR RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE receives an RRCReconfiguration message including ConditionalReconfiguration and only event A3 is configured as conditional handover trigger event }
    then { UE starts evaluating the CHO execution condition A3 and perform conditional handover to the neighbour cell triggered in conditional configuration execution }

}

(3)

with { UE in NR RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE receives an RRCReconfiguration message including ConditionalReconfiguration and only event A5 is configured as conditional handover trigger event }
    then { UE starts evaluating the CHO execution condition A5 and perform conditional handover to the neighbour cell triggered in conditional configuration execution }

}

(4)

with { UE in NR RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE receives an RRCReconfiguration message including ConditionalReconfiguration and both event A3 and event A5 are configured as conditional handover trigger events }
    then { UE starts evaluating the CHO execution condition A3 and A5 and perform conditional handover to the neighbour cell triggered in conditional configuration execution }

}

(5)

with { UE in NR RRC_CONNECTED state and supporting conditional handover }

ensure that {

when { UE perform conditional handover procedure successfully }
    then { UE releases stored CHO configurations }

}

8.1.4.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300 clause 9.2.3.4.1 and 9.2.3.4.2 and TS 38.331 clause 5.3.5.1, 5.3.5.13.1, 5.3.5.13.4 and 5.3.5.13.5. Unless otherwise stated these are Rel-16 requirements.

[TS 38.300, clause 9.2.3.4.1]

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once a handover is executed (legacy handover or conditional handover execution).

The following principles apply to CHO:

-	The CHO configuration contains the configuration of CHO candidate cell(s) generated by the candidate gNB(s) and execution condition(s) generated by the source gNB.

-	An execution condition may consist of one or two trigger condition(s) (CHO events A3/A5, as defined in [12]). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell.

-	Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration), the UE executes the HO procedure as described in clause 9.2.3.2, regardless of any previously received CHO configuration.

-	While executing CHO, i.e. from the time when the UE starts synchronization with target cell, UE does not monitor source cell.

CHO is not supported for NG-C based handover in this release of the specification.

[TS 38.300, clause 9.2.3.4.2]

As in intra-NR RAN handover, in intra-NR RAN CHO, the preparation and execution phase of the conditional handover procedure is performed without involvement of the 5GC; i.e. preparation messages are directly exchanged between gNBs. The release of the resources at the source gNB during the conditional handover completion phase is triggered by the target gNB. The figure below depicts the basic conditional handover scenario where neither the AMF nor the UPF changes:

Figure 9.2.3.4.2-1: Intra-AMF/UPF Conditional Handover

0/1.	Same as step 0, 1 in Figure 9.2.3.2.1-1 of clause 9.2.3.2.1.

2.	The source gNB decides to use CHO.

3.	The source gNB requests CHO for one or more candidate cells belonging to one or more candidate gNBs. A CHO request message is sent for each candidate cell.

4.	Same as step 4 in Figure 9.2.3.2.1-1 of clause 9.2.3.2.1.

5.	The candidate gNB(s) sends CHO response (HO REQUEST ACKNOWLEDGE) including configuration of CHO candidate cell(s) to the source gNB. The CHO response message is sent for each candidate cell.

6.	The source gNB sends an RRCReconfiguration message to the UE, containing the configuration of CHO candidate cell(s) and CHO execution condition(s).

NOTE 1:	CHO configuration of candidate cells can be followed by other reconfiguration from the source gNB.

NOTE 1a:	A configuration of a CHO candidate cell cannot contain a DAPS handover configuration.

7.	The UE sends an RRCReconfigurationComplete message to the source gNB.

7a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

8.	The UE maintains connection with the source gNB after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source gNB, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to the target gNB. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

8a/b	The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE 2:	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

8c.	The source gNB sends the HANDOVER CANCEL message toward the other signalling connections or other candidate target gNBs, if any, to cancel CHO for the UE.

[TS 38.331, clause 5.3.5.1]

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups, to add/modify/release conditional handover configuration, to add/modify/release conditional PSCell change configuration. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

[TS 38.331, clause 5.3.5.13.1]

The network configures the UE with one or more candidate target SpCells in the conditional reconfiguration. The UE evaluates the condition of each configured candidate target SpCell. The UE applies the conditional reconfiguration associated with one of the target SpCells which fulfils associated execution condition. The network provides the configuration parameters for the target SpCell in the ConditionalReconfiguration IE.

The UE performs the following actions based on a received ConditionalReconfiguration IE:

1>	if the ConditionalReconfiguration contains the condReconfigToRemoveList:

2>	perform conditional reconfiguration removal procedure as specified in 5.3.5.13.2;

1>	if the ConditionalReconfiguration contains the condReconfigToAddModList:

2>	perform conditional reconfiguration addition/modification as specified in 5.3.5.13.3;

[TS 38.331, clause 5.3.5.13.4]

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync in the received condRRCReconfig to be applicable cell;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the condExecutionCond associated to condReconfigId:

3>	if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4>	consider the event associated to that measId to be fulfilled;

3>	if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4>	consider the event associated to that measId to be not fulfilled;

2>	if event(s) associated to all measId(s) within condTriggerConfig for a target candidate cell within the stored condRRCReconfig are fulfilled:

3>	consider the target candidate cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

3>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

NOTE:	Up to 2 MeasId can be configured for each condReconfigId. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold.

[TS 38.331, clause 5.3.5.13.5]

The UE shall:

1>	if more than one triggered cell exists:

2>	select one of the triggered cells as the selected cell for conditional reconfiguration execution;

1>	for the selected cell of conditional reconfiguration execution:

2>	apply the stored condRRCReconfig of the selected cell and perform the actions as specified in 5.3.5.3;

NOTE:	If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

8.1.4.4.1.3	Test description

8.1.4.4.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 4.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for NR cells

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.4.4.1.3.2	Test procedure sequence

Tables 8.1.4.4.1.3.2-1 and 8.1.4.4.1.3.2-2 illustrate the downlink power levels to be applied for NR Cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1", "T2" and "T3", are applied at the point indicated in the Main behaviour description in Table 8.1.4.4.1.3.2-3.

Table 8.1.4.4.1.3.2-1: Power levels in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -91 | -91 | Power levels are such that UE registered on NR Cell 1 and entry condition for event A3 is not satisfied |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -79 | -91 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -91 | -79 | Power levels are such that entry condition for event A5 is satisfied for NR Cell 4 |
| T3 | SS/PBCH SSS EPRE | dBm/ SCS | -79 | -91 | -91 | Power levels are such that entry condition for event A3 and event A5 are satisfied for NR Cell 1 |

Table 8.1.4.4.1.3.2-2: Power levels in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -91 | -91 | Power levels are such that UE registered on NR Cell 1 and entry condition for event A3 is not satisfied |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -82 | -91 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -91 | -82 | Power levels are such that entry condition for event A5 is satisfied for NR Cell 4 |
| T3 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -91 | --91 | Power levels are such that entry condition for event A3 and event A5 are satisfied for NR Cell 1 |

Table 8.1.4.4.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as target candidate cells and configure event A3 as trigger event in NR cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message in NR cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Check: Does the UE initiate a random access procedure transmitting Preamble in NR Cell 2 or in NR cell 4 within 10s? | --> | (PRACH Preamble) | 1 | F |
| 4 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 5 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 2 within 10s? | --> | (PRACH Preamble) | 2 | P |
| 6 | The SS transmits Random Access Response. | <-- | Random Access Response | - | - |
| 7 | The UE transmits RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 2? | - | - | 2 | - |
| 9 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 4 as target candidate cells and configure event A5 as trigger event in NR cell 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message in NR cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 11 | SS adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 12 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? | --> | (PRACH Preamble) | 3 | P |
| 13 | The SS transmits Random Access Response. | <-- | Random Access Response | - | - |
| 14 | The UE transmits RRCReconfigurationComplete message in NR Cell 4. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 15 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4? | - | - | 3 | - |
| 16 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration for intra -frequency event A3 and A5 in NR cell 4. | <-- | NR RRC: RRCReconfiguration | - | - |
| 17 | The UE transmits an RRCReconfigurationComplete message in NR cell 4. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 18 | SS adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 19 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 1 within 10s? | --> | (PRACH Preamble) | 4 | P |
| 20 | The SS transmits Random Access Response. | <-- | Random Access Response | - | - |
| 21 | The UE transmits RRCReconfigurationComplete message in NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 22 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 1? | - | - | 4 | - |
| 23 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 24 | Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 2 within 10s? | --> | (PRACH Preamble) | 5 | F |

8.1.4.4.1.3.3	Specific message contents

Table 8.1.4.4.1.3.3-1: RRCReconfiguration (Step 1, step 9 and step 16, Table 8.1.4.4.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 8.1.4.4.1.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration | Table 8.1.4.4.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.1.3.3-2: MeasConfig (Table 8.1.4.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.4.4.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-condEventA3 | Table 8.1.4.4.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-condEventA5 | Table 8.1.4.4.1.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  | Step 1, Step 9 |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  | Step 1 |
|  | 2 |  | Step 9 |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  | Step 16 |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.1.3.3-3: MeasObjectNR (Table 8.1.4.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 | The SSB of NR cell 1, NR Cell 2 and NR cell 4 have the same ARFCN value as specified in TS 38.508-1 [4] clause 6.2.3 |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.4.4.1.3.3-4: ReportConfigNR-condEventA3 (Table 8.1.4.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition CHO AND EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig-r16 SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.1.3.3-5: ReportConfigNR-condEventA5 (Table 8.1.4.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition CHO AND EVENT_A5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig-r16 SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA5 SEQUENCE { |  |  |  |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | 76 | -80dBm | FR1 |
|  | 73 | -83dBm | FR2 |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | 70 | -86dBm | FR1 |
|  | 66 | -90dBm | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.1.3.3-6: ConditionalReconfiguration (Table 8.1.4.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16 ::= SEQUENCE { |  |  |  |
| condReconfigToRemoveList-r16 | Not present |  |  |
| condReconfigToAddModList-r16 | CondReconfigToAddModList-r16 (Step 1 and step 9) | Table 8.1.4.4.1.3.3-7 | Step 1, Step 9 |
|  | CondReconfigToAddModList-r16 (Step 16) | Table 8.1.4.4.1.3.3-10 | Step 16 |
| } |  |  |  |

Table 8.1.4.4.1.3.3-7: CondReconfigToAddModList-r16 (Step 1 and step 9, Table 8.1.4.4.1.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 2 entries |  |  |
| CondReconfigToAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO with condition HO_NR cell 2 | Table 8.1.4.4.1.3.3-8 | Step 1 |
|  | RRCReconfiguration-HO with condition HO_NR cell 1 | Table 8.1.4.4.1.3.3-8 | Step 9 |
| } |  |  |  |
| CondReconfigToAddMod-r16[2] SEQUENCE { |  | entry 2 |  |
| condReconfigId-r16 | 2 |  |  |
| condExecutionCond-r16 SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO with condition HO_NR cell 4 | Table 8.1.4.4.1.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.1.3.3-8: RRCReconfiguration-HO (Tables 8.1.4.4.1.3.3-7 and 8.1.4.4.1.3.3-10)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig with condition PCI_NR cell 1 | Table 8.1.4.4.1.3.3-9 | HO_NR cell 1 |
|  | CellGroupConfig with condition PCI_NR cell 2 | Table 8.1.4.4.1.3.3-9 | HO_NR cell 2 |
|  | CellGroupConfig with condition PCI_NR cell 4 | Table 8.1.4.4.1.3.3-9 | HO_NR cell 4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.1.3.3-9: CellGroupConfig (Table 8.1.4.4.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 1 |  | PCI_NR cell 1 |
|  | Physical Cell Identity of NR Cell 2 |  | PCI_NR Cell 2 |
|  | Physical Cell Identity of NR Cell 4 |  | PCI_NR Cell 4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.4.1.3.3-10: CondReconfigToAddModList-r16 (Step 16, Table 8.1.4.4.1.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 2 entries |  |  |
| CondReconfigToAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| MeasId [2] | 2 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO with condition HO_NR cell 1 | Table 8.1.4.4.1.3.3-8 |  |
| } |  |  |  |
| CondReconfigToAddMod-r16[2] SEQUENCE { |  | entry 2 |  |
| condReconfigId-r16 | 2 |  |  |
| condExecutionCond-r16 SEQUENCE { |  |  |  |
| MeasId [1] | 1 |  |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO with condition HO_NR cell 2 | Table 8.1.4.4.1.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |

[End of change]