3GPP TSG-RAN5 Meeting #105	R5-246434

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0079 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTN A-Spurious test - corrections for NS 02N |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As captured in table 6.2.3.3.1-1, for NS_02N no A-MPR requirements apply but only A-Spurious requirements. In A-Spurious test cases, for NS_02N, test configuration table refer to a test configuration table in A-MPR test case 6.2.3, but that table does not exist for the reasons explained above. Therefore, test configuration table from MPR test case 6.2.2 should be referred instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In A-Spurious test case 6.5.3.3, in test configuration table for NS_02N, used configuration table 6.2.2.4.1-1 as reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.5.3.3	Additional Spurious emissions

##### 6.5.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

##### 6.5.3.3.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

##### 6.5.3.3.3	Minimum conformance requirements

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The normative reference for this requirement is TS 38.101-5 [11] subclause 6.5.3.3.

6.5.3.3.3.1	Minimum conformance requirements (network signalling value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3.1-1: Additional requirements for "NS_02N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1,559≤ f < 1,605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1,605≤ f ≤ 1,610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1,559 ≤ f < 1,605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1,605≤ f ≤ 1,610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

##### 6.5.3.3.4	Test description

6.5.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All these configurations shall be tested with applicable test parameters for each channel bandwidth and sub-carrier spacing, are shown in Tables 6.5.3.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.3.4.1-1: Test Configuration Table (network signalling value “NS_02N”)

Same test configuration as listed in Table 6.2.2.4.1-1 shall be used with the following exceptions:

- 	Test SCS shall be: No exception for UE mean power testing (step 3) and only Lowest for additional spurious emission testing (step 4)

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.5.3.3.4.3.

6.5.3.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.3.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration in Tables 6.5.3.3.4.1-1, which shall meet the requirements in clause 6.5.3.3.5 with allowed A-MPR values specified in 6.2.3.5. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to clauses 6.5.3.3.3.1. The centre frequency of the filter shall be stepped in contiguous steps according to the same table. During measurement the spectrum analyser shall be set to 'Detector' = RMS. For NS_02N the additional spurious emissions requirement shall be verified with UE transmission power obtained by sending uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level 15 dBm for at least the duration of the additional spurious emissions measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in 3GPP TS 38.521-1 [2] Table 6.3.4.3.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty for 3GPP TS 38.521-1 [2] test case 6.3.4.3 is specified in 3GPP TS 38.521-1 [2] Table F.1.2-1.

6.5.3.3.4.3	Message contents

Message contents for SIB19 are according to TS 38.508-1 [12] clause 5.6.2.1 with following exceptions:

6.5.3.3.4.3.1	Message contents exceptions (network signalling value "NS_02N")

Message contents are according to TS 38.508-1 [12] subclause 4.6, with the following exceptions:

1.	Information element additionalSpectrumEmission is set to NS_02N. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.5.3.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_02N"

| Derivation Path: TS 38.508-1 [12] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_02N) |  |  |

##### 6.5.3.3.5	Test requirement

This clause specifies the requirements for the specified NR band for an additional spectrum emission requirement with protected bands as indicated from Table 6.5.3.3.5.1-1.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5.3.3.5.1	Test requirement (network signalling value "NS_02N")

When "NS_02N" is indicated in the cell,

The power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.5.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.5.1-1: Additional requirements for "NS_02N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1,559≤ f < 1,605 | -50 | 700 Hz | Averaged over any 2-millisecond active transmission interval |
| 1,605≤ f ≤ 1,610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1,559 ≤ f < 1,605 | -40 | 1MHz | Averaged over any 2-millisecond active transmission interval |
| 1,605≤ f ≤ 1,610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

## <<< END OF CHANGES >>>