3GPP TSG-RAN5 Meeting #105	R5-246445

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

Agenda item:	5.3.21.9

Source:	Keysight Technologies

Title:	Discussion on MU and TT for ATG RF testing

Document for:	Discussion and Endorsement

# Introduction

Air-To-Ground devices testing is being introduced in RAN5 via the WI NR_ATG-UEConTest. Progress has been made in certain test cases and main open topic has been the definition of the MU and TT or even potential relaxations. This contribution aims to compile some background on the associated discussions and implicit agreements so far and to analyse in more detail some of the RF test cases already in progress in the associated work plan in order to propose specific values for MUs, TTs and relaxations whenever needed.

# Discussion

## Background

#### Maximum UE power

Before ATG, the highest DUT power class defined in FR1 was PC1. In that case maximum nominal power is 31dBm, that considering DUT power tolerance of +/-2dB and considering certain testing tolerance, let’s assume at least 1dB, that means the tester must be able to receive DUT power up to 34dBm to be safe, that is approximately 2.5W.

Other power classes like PC3 (23dBm nominal power) and PC2 (26dBm nominal) power are more popular. That means that certain testers could have been designed natively to support input power up to 29dBm to support up to PC2. However, considering the small difference with PC1, just 5dB, those testers can easily be extended to support PC1.

However, regarding an ATG device maximum power, as defined in TS 38.101-1[1] , clause 6.2J.1, ATG DUT nominal power can be up to 40dBm. That is 9dB more than PC1 or 14dB more than PC2.  In addition, repeating the power tolerance analysis made above, in order to be safe, the test system must be able to receive up to 43dBm, that are 20W.

Observation 1. To support testing an ATG device, a test system must be able to receive up to 43dBm, that are 20W from the DUT UL connector.

In the best scenario, considering a test system natively designed to support the highest power class so far, PC1, in order to allow that test system support an ATG device without getting damaged, a ≥9dB attenuator supporting ≥20W of input power would be required to be connected between the test system and the DUT UL antenna connector.

Observation 2. A test system natively designed to support up to the highest NR FR1 power class so far, PC1, to support testing an ATG device without getting damaged, a ≥9dB attenuator supporting ≥20W of input power would be required to be connected at the test system input.

Discrete attenuators are not available for any attenuation value at required input powers while at the same time, they present certain attenuation tolerance. So, for the situation presented above, if required attenuation to avoid damage is ≥9dB, most likely the best attenuator to use will need to have at least 10dB of nominal attenuation and considering the attenuator tolerance in its frequency response, easily the extra attenuation to be considered in the test system dynamic range analysis is about 11dB.

Proposal 1. In the dynamic range analysis of the test systems to support ATG, the discrete value of the nominal attenuation and the tolerance in the frequency response of the extra attenuators need to be considered.

One key feature of a test system is the ability to automate testing. The analysis above is oriented to protect the test system when the ATG DUT is transmitting at is maximum power or close to. That is the situation in most of the test cases, including RX test cases. However, in some of them where UE is set to min power or OFF power in UL, such an attenuation would not be needed. Nevertheless, to allow fully test cases automation and also to protect the test system from a misfunction during the test, whenever an ATG device is connected to the test system, the required attenuation should always be present, disregarding the test in progress.

Observation 3. Not all the test cases require to set the DUT to its maximum power, however, any misfunction of/during the test could make the DUT to transmit at maximum power anyway, damaging the test system if not correctly protected.

Observation 4. A key feature of a test system is the ability to automate testing.

Proposal 2. If a test system requires extra attenuation to run ATG test cases, RAN5 shall consider the highest required attenuation to be added to all ATG test cases. That is, the same level of extra attenuation will be used for the MU and TT analysis of all the ATG test cases.

At the same time, it must be noted that test requirements for an ATG device are in general different to requirements to other NR devices. That is, there are significant differences not only in the maximum output power but also in the maximum input level or the minimum output power, for example. As a result, in general the needed attenuators to support ATG testing should be removed when testing other type of devices.

Observation 5. Attenuators required for ATG test cases should not be present for executions of other type of test cases.

#### Maximum UE UL connectors

- ATG UL power requirements defined in TS 38.101-1[1] , starting with clause 6.2J.1, assume that an ATG UE can have either multiple omni-directional antennas or an antenna array with multiple connectors at the transceiver array boundary (TAB). The power requirements apply at the transmit antenna connectors or transceiver array boundary (TAB) connectors depending on the ATG DUT type and in most of the cases the requirements apply to the sum of power at each of UE antenna/TAB connector. However, in TS 38.101-1[1], it is not defined the specific number of UL connectors an ATG DUT may have.

Observation 6. It is unclear how many transmit antenna connectors/ transceiver array boundary (TAB) connectors an ATG DUT may have.

- Looking at RAN4 ATG study TR 38.876 [5], section 6.2.3.2 about ATG UE antenna model, in case of antenna array, the following antenna array configurations have been considered: (8x2x2) and (16x1x2) (Row x Column x Polarization). Depending on how ATG UEs will implement beamforming, analogic, hybrid or digital, the number of TAB connectors can vary from a very small number up to, in worst case, 32 connectors.  In this potential worse case of 32 TAB connectors, apart from the impact on the MU analysis for UL power requirements applicable to the sum power of all connectors, that is the main analysis in this subsection, it may have in general a serious impact on the connectivity to the test equipment. For that reason, it is important that ATG UE manufacturers clarify the expected implementation in terms of number of omni-directional antennas or, in case of using an antenna array, the number of TAB connectors.

- Figure 2.1.2-1. Radiated and conducted reference points (Figure 4.3.2-1 in [6]).

Observation 7. In worst case, number of TAB connectors could be up to 32. Such a large number can have serious connectivity issues at test equipment to support ATG UE testing. Even more, MU and TT analysis so far, that is assuming a small number of connectors, may need to be revisited.

- Proposal 3. Create an AP to ATG UE manufacturers to clarify the expected implementation in terms of number of omni-directional antennas or, in case of using an antenna array, the number of TAB connectors.

- Back to the UL power requirements analysis, for the cases where the requirements apply to the sum of power at all connectors, as well analyzed in [3], for a given power level requirement, the more UL connectors to combine, the less available SNR to assume. SNR assumption is reduced by 10*log10(N); with  N = Number of antenna/TAB connectors, compared to the single connector case.

- The MU analysis in [3] and all the test cases with MU analyzed so far in TS 38.521-1 [2] assume at maximum 2 UL connectors, that means at maximum 3dB SNR degradation compared to the single connector case.

Observation 8. In case an UL power requirement applies to the sum of power at all UE connectors, all MTSU defined so far in TS 38.521-1 [2] assume a maximum of 2 UL connectors.

- Based on two previous observations, while no specific number of UL connector is defined, for requirements that apply to the sum of the mean power from all antenna connectors or all TAB connectors, it is proposed to limit MTSU definition for ATG for no more than 2 transmit antenna connectors / transceiver array boundary (TAB) connectors.

Proposal 4. While it is unclear the number of UL connectors for ATG, for requirements that apply to the sum of the mean power from all antenna connectors or all TAB connectors, limit MTSU definition for ATG assuming no more than 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, otherwise MTSU will be FFS.

Observation 9. Proposal 4 is already an implicit agreement from offline discussions in previous meeting. MTSU defined so far for ATG in TS 38.521-1 [2] already follows Proposal 3.

## Test cases specific analysis

As commented above, MTSU were tentatively defined during last meeting for certain test cases, following Proposal 4, letting values in square brackets [ ] and, in cases of doubt but in order to progress, limiting definition of MU for ATG UEs with maxOutputPowerATG-r18 up to 31dBm (equal to PC1).

After further analysis in our test system, MTSU values already defined for the following test cases can be accepted, even removing any limitation of the ATG UE maximum output power, but still assuming a maximum of 2 UL connectors (in case of requirements that apply to the sum of the mean power from all antenna connectors or all TAB connectors):

- 6.2J.1	UE maximum output power for ATG

- 6.2J.2	Configured transmitted power for ATG

- 6.3J.1	Minimum output power for ATG

- 6.5J.1 	Occupied bandwidth for ATG

- 7.4J		Maximum input level for ATG

- 7.5J		Adjacent channel selectivity for ATG

- 7.6J.2	In-band blocking for ATG

- 7.7J		Spurious response for ATG

- 7.8J.2	Wide band intermodulation for ATG

Proposal 5. For ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, in case of combined power requirement, accept, without any restriction in the DUT maximum output power, MU and TT already defined for the following test cases: 6.2J.1, 6.2J.2, 6.3J.1, 6.5J.1, 7.4J, 7.5J, 7.6J.2, 7.7J, 7.8J.2.

In addition, in case of In-band blocking test case, it was captured in the editor’s note that MU is FFS for Case 3. However, after further analysis, it can be observed that only Case 1 and Case 2 apply for the ATG bands defined in TS 38.101-1[1], clause 5.2J. Therefore, such an editor’s note does not apply and can be removed.

Proposal 6. For ATG IBB test case 7.6J.2, remove any mention to Case 3 and Case 4 in editor’s notes or test definition while they don’t apply to ATG bands.

Finally, it must be observed that core requirements in TS 38.101-1[1] have been updated by R4-2414321 [4]. As a result, from the test cases listed above, occupied BW requirements are defined now at each transmit antenna connector or each TAB connector. However, current test case definition TS 38.521-1 [2] is assuming requirement applies to the sum of power at all UL connectors. Therefore, test case needs to be generally updated to be aligned with core specs.

Proposal 7. For ATG occupied BW test case 6.5J.1, update definition according to core specs to apply requirement at each transmit antenna connector or each TAB connector.

#### OFF power test case

- During previous meeting, it was unclear, even at core specs, whether OFF power requirement for ATG apply per connector or for the sum of power at all connectors and so was captured in the editor’s notes while current test definition assumes the requirements applies to the sum of power.  However, the mentioned update in TS 38.101-1[1] by R4-2414321 [4] has resolved the ambiguity clarifying the requirement is per connector. Hence, test case editor’s note and test definition needs to be updated accordingly.

Proposal 8. For ATG OFF power test case 6.3J.2, update editor’s notes and definition according to core specs to apply requirement at each transmit antenna connector or each TAB connector.

- With that clear, we have analyzed the test requirement in our test system and concluded that, for ATG UEs with maximum nominal power in the range {31dBm (PC1) < maxOutputPowerATG-r18 ≤ 40dBm}, MTSU and TT should be increased, and even certain relaxation would be required depending on the CBW to test compared to regular OFF power test case 6.3.2, otherwise, values from test case 6.3.2 can apply, assuming required attenuators remain connected to enable testing automation and test system protection of ATG devices, as per proposals and observations made in section 2.1.1.

| - MTSU proposal | - TT Proposal | - Relaxation Proposal |
| --- | --- | --- |
| For ATG UEs with maxOutputPowerATG-r18 up to 31dBm (up to PC1): Same as 6.2.3 for each UE antenna/TAB connector Otherwise, for each UE antenna/TAB connector: f ≤ 3.0GHz ±2.15 dB, BW ≤ 40MHz ±2.35 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±2.45 dB, BW ≤ 40MHz ±2.55 dB, 40MHz < BW ≤ 80MHz ±2.85 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.65 dB, BW ≤ 20MHz ±2.75 dB, 20MHz < BW ≤ 80MHz ±2.85 dB, 80MHz < BW ≤ 100MHz | For ATG UEs with maxOutputPowerATG-r18 up to 31dBm (up to PC1): Same as 6.2.3 for each UE antenna/TAB connector Otherwise, for each UE antenna/TAB connector: f ≤ 3.0GHz 2.15 dB, BW ≤ 40MHz 2.35 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 2.45 dB, BW ≤ 100MHz | For ATG UEs with maxOutputPowerATG-r18 up to 31dBm (up to PC1): 0dB Otherwise: 0dB, BW ≤ 25MHz 3dB, 25MHz < BW ≤ 50MHz 6dB, 50MHz < BW ≤ 100MHz |

Table 2.2.1-1. MTSU, TT and relaxations proposal for ATG OFF power test case 6.3J.2.

- Proposal 9. For ATG OFF power test case 6.3J.2, define MTSU, TT and relaxation according to proposal in Table 2.2.1-1.

#### Out-of-band blocking test case

This is the test case requiring the highest power in DL, in particular, for CW signal, required power in Range 3 is -15dBm at DUT plane operating at frequencies from 1MHz up to 12.75GHz. This is quite remarkable high DL power compared to other test cases. For example, it is 15dB higher than the highest DL power required for ATG Maximum input level test 7.4J. Therefore, the presence of any extra attenuation would have a significant impact on the maximum power a test system can transmit at DUT plane.

After evaluating our test system under all premises presented in section 2.1, we conclude that, for ATG UEs with maximum nominal power in the range {31dBm (PC1) < maxOutputPowerATG-r18 ≤ 40dBm}, the need of certain relaxation to apply to the CW power. In particular 7dB for f ≤ 7.5GHz and 8dB for 7.5GHz < f ≤ 12.75GHz.

In this type of test cases, the most important is the power difference between the interferer and the wanted signal so if CW signal power level is decreased by certain relaxation, the wanted signal power level should be decreased accordingly. However, the required wanted signal power level, for CBW ≤ 15MHz is P ≤ REFSENS +7dB. In no case, wanted signal should be set below REFSENS. Therefore, in those cases, CBW ≤ 15MHz, either should not be tested for Range 3 or tested with less power reduction in the wanted signal. Given the small deviation in the delta power in those cases between the interferer and the wanted signal vs the delta required, we propose to test them in the way captured in Table 2.2.2-1.

| - Relaxation Proposal - (CW) | - Relaxation Proposal - (Wanted Signal) |
| --- | --- |
| For Case 3 for ATG UEs with maxOutputPowerATG-r18 greater than 31dBm (greater than PC1): f ≤ 7.5GHz 7dB 7.5GHz < f ≤ 12.75GHz 8dB Otherwise: 0dB | For Range 3 for ATG UEs with maxOutputPowerATG-r18 greater than 31dBm (greater than PC1): f ≤ 7.5GHz 6dB, BW ≤ 10MHz 7dB, 10MHz < BW ≤ 100MHz 7.5GHz < f ≤ 12.75GHz 6dB, BW ≤ 10MHz 7dB, BW = 15MHz 8dB, 15MHz < BW ≤ 100MHz Otherwise: 0dB |

Table 2.2.2-1. Relaxations proposal for ATG Out-of-band blocking test case 7.6J.3, Range 3.

- Proposal 10. For ATG Out-of-band blocking test case 7.6J.3, Range 3, apply relaxations according to proposal in Table 2.2.2-1.

#### Spurious Emissions test cases

More time is required to evaluate MTSU and TT for spurious emissions test cases for ATG noting that in these test cases the analysis must be extended up to 26GHz so in case extra attenuators are required, they must support such a frequency range.

Observation 10. For spurious emissions test cases, extra attenuators, if needed, must support up to 26GHz and so the MTSU analysis must be extended up to that frequency range.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. To support testing an ATG device, a test system must be able to receive up to 43dBm, that are 20W from the DUT UL connector.

Observation 2. A test system natively designed to support up to the highest NR FR1 power class so far, PC1, to support testing an ATG device without getting damaged, a ≥9dB attenuator supporting ≥20W of input power would be required to be connected at the test system input.

Observation 3. Not all the test cases require to set the DUT to its maximum power, however, any misfunction of/during the test could make the DUT to transmit at maximum power anyway, damaging the test system if not correctly protected.

Observation 4. A key feature of a test system is the ability to automate testing.

Observation 5. Attenuators required for ATG test cases should not be present for executions of other type of test cases.

Observation 6. It is unclear how many transmit antenna connectors/ transceiver array boundary (TAB) connectors an ATG DUT may have.

Observation 7. In worst case, number of TAB connectors could be up to 32. Such a large number can have serious connectivity issues at test equipment to support ATG UE testing. Even more, MU and TT analysis so far, that is assuming a small number of connectors, may need to be revisited.

Observation 8. In case an UL power requirement applies to the sum of power at all UE connectors, all MTSU defined so far in TS 38.521-1 [2] assume a maximum of 2 UL connectors.

Observation 9. Proposal 4 is already an implicit agreement from offline discussions in previous meeting. MTSU defined so far for ATG in TS 38.521-1 [2] already follows Proposal 3.

Observation 10. For spurious emissions test cases, extra attenuators, if needed, must support up to 26GHz and so the MTSU analysis must be extended up to that frequency range.

Proposal 1. In the dynamic range analysis of the test systems to support ATG, the discrete value of the nominal attenuation and the tolerance in the frequency response of the extra attenuators need to be considered.

Proposal 2. If a test system requires extra attenuation to run ATG test cases, RAN5 shall consider the highest required attenuation to be added to all ATG test cases. That is, the same level of extra attenuation will be used for the MU and TT analysis of all the ATG test cases.

Proposal 3. Create an AP to ATG UE manufacturers to clarify the expected implementation in terms of number of omni-directional antennas or, in case of using an antenna array, the number of TAB connectors.

Proposal 4. While it is unclear the number of UL connectors for ATG, for requirements that apply to the sum of the mean power from all antenna connectors or all TAB connectors, limit MTSU definition for ATG assuming no more than 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, otherwise MTSU will be FFS.

Proposal 5. For ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, in case of combined power requirement, accept, without any restriction in the DUT maximum output power, MU and TT already defined for the following test cases: 6.2J.1, 6.2J.2, 6.3J.1, 6.5J.1, 7.4J, 7.5J, 7.6J.2, 7.7J, 7.8J.2.

Proposal 6. For ATG IBB test case 7.6J.2, remove any mention to Case 3 and Case 4 in editor’s notes or test definition while they don’t apply to ATG bands.

Proposal 7. For ATG occupied BW test case 6.5J.1, update definition according to core specs to apply requirement at each transmit antenna connector or each TAB connector.

Proposal 8. For ATG OFF power test case 6.3J.2, update editor’s notes and definition according to core specs to apply requirement at each transmit antenna connector or each TAB connector.

Proposal 9. For ATG OFF power test case 6.3J.2, define MTSU, TT and relaxation according to proposal in Table 2.2.1-1.

Proposal 10. For ATG Out-of-band blocking test case 7.6J.3, Range 3, apply relaxations according to proposal in Table 2.2.2-1.

# References

- TS 38.101-1, “User Equipment (UE) Radio transmission and reception; Part 1: Range 1 Standalone”, v18.7.0 (2024-09)

- TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v18.4.0 (2024-09)

- R5-195141, “MU for sum of power”, Anritsu, RAN5#83, May’19.

- R4-2414321, “(NR_ATG-Core) CR for TS 38.101-1 to clarify the Tx requirements definition for ATG UE”, Huawei, RAN4#112, Aug’24.

- TR 38.876, “Air-to-ground network for NR Standalone (Release 18)”, v18.2.0 (2024-07)

- TS 38.101-4, “NR; Base Station (BS) radio transmission and reception”, v18.7.0 (2024-09)