3GPP TSG-RAN5 Meeting #105	R5-246450

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3022 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction in FR1 Spurious emissions for UE co-existence for 2 UL CA test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When test case 6.5A.3.2.1 was updated for inter-band by R5-241880, only one test point was left for most of the combos. However, the test IDs left were not renumbered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Renumbered the test IDs in Table 6.5A.3.2.1.4.1-1, considering the total number of test IDs per combo. Table notes updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

##### 6.5A.3.2.1	Spurious emissions for UE co-existence for CA (2UL CA)

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions for 2UL CA.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL Allocation |  |  |  |  |
|  |  | CA Configuration |  |  |  |  |  |  |  | PCC NRB @SCS | SCC NRB@SCS | CC MOD | PCC & SCC RB allocation |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |
|  |  | PCC |  |  |  | SCC |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |
|  |  | Band |  | Range |  | Band |  | Range |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n1 |  | Low |  | n28 |  | Low |  | 25@15kHz | 25@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n2A-n14A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 |  | Mid |  | n14 |  | High |  | 106@15kHz |  | 52@15kHz | CP-OFDM QPSK | NA |  |  | CP-OFDM QPSK | 1@0 |  | 1@51 |
| 2 | n2 |  | Mid |  | n14 |  | Low |  | 106@15kHz |  | 52@15kHz | CP-OFDM QPSK | NA |  |  | CP-OFDM QPSK | 1@0 |  | 1@0 |
| Test Settings for CA_n3A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n3 |  | Low |  | n8 |  | Low |  | 100@15kHz | 100@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@99 | 1@0 |  |
| 2 |  | n3 |  | High |  | n8 |  | High |  | 100@15kHz | 100@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@99 | 1@100 |  |
| 3 |  | n3 |  | Low |  | n8 |  | High |  | 100@15kHz | 100@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@100 |  |
| Test Settings for CA_n3A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n3 |  | Low |  | n77 |  | 3660.15MHz (NOTE 19) |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | - CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n3 |  | Low |  | n78 |  | High |  | 160@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@159 | 1@272 |  |
| Test Settings for CA_n5A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n5 |  | Low |  | n48 |  | Low |  | 106@15kHz | 106@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n5 |  | Low |  | n77 |  | 3538,29 MHz (NOTE 11) |  | 106@15kHz | 52@15kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n8 |  | High |  | n78 |  | High |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@105 | 1@35 |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n28 |  | High |  | n41 |  | High |  | 79@15kHz | 217@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@78 | 1@115 |  |
| Test Settings for CA_n28A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n28 |  | Low |  | n77 |  | Low |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n28A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n28 |  | Low |  | n78 |  | Low |  | 106@15kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| Test Settings for CA_n41A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n41 |  | Mid |  | n77 |  | Low |  | 273@30kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@272 | 1@272 |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n41 |  | High |  | n79 |  | Low |  | 273@30kHz | 273@30kHz | CP-OFDM QPSK | NA |  | CP-OFDM QPSK |  | 1@0 | 1@0 |  |
| NOTE 1: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. NOTE 2: Void NOTE 3: Void NOTE 4: Void NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE 10: Void NOTE 11: Test Point for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3538.29MHz (NR ARFCN=635867). NOTE 12: Void NOTE 13: Void NOTE 14: Void NOTE 15: Void NOTE 16: Void NOTE 17: Void NOTE 18: Void NOTE 19: Test point for CA_n3A-n77A and Band n77 has the centre frequency at 3660.15MHz (NR ARFCN=644010). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>