3GPP TSG-RAN5 Meeting #105	R5-246578

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5367 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EPS P-CSCF restoration test case 10.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conditions Additional_P-CSCF_IPv6 and Additional_P-CSCF_IPv4 in Table 10.3.2.3.3-1 are not needed as it is not clear when SS needs to configure them. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected conditions in Table 10.3.2.3.3-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 10.3.2	EPS Bearer context modification / new P-CSCF address / Initial IMS registration

10.3.2.1	Test Purpose (TP)

(1)

with { the UE is in BEARER CONTEXT ACTIVE STATE and in EMM-CONNECTED mode and having performed IMS registration }

ensure that {

when { the UE receives a MODIFY EPS BEARER CONTEXT REQUEST message containing P-CSCF addresses different from the currently used P-CSCF }

then { UE transmits a MODIFY EPS BEARER CONTEXT ACCEPT message and performs initial IMS registration using the first P-CSCF address received }

}

10.3.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.229, clauses L.2.2.1 and L.2.2.1C. Unless otherwise stated these are Rel-9 requirements.

[TS 24.229, clause L.2.2.1]

Prior to communication with the IM CN subsystem, the UE shall:

…

c)	acquire a P-CSCF address(es).

The methods for P-CSCF discovery are:

I.	When using IPv4, employ the Dynamic Host Configuration Protocol (DHCP) RFC 2132 [20F], the DHCPv4 options for SIP servers RFC 3361 [35A], and RFC 3263 [27A] as described in subclause 9.2.1. When using IPv6, employ Dynamic Host Configuration Protocol for IPv6 (DHCPv6) RFC 3315 [40], the DHCPv6 options for SIP servers RFC 3319 [41] and DHCPv6 options for Domain Name Servers (DNS) RFC 3646 [56C] as described in subclause 9.2.1.

II.	Transfer P-CSCF address(es) within the EPS bearer context activation procedure.

The UE shall indicate the request for a P-CSCF address to the network within the Protocol Configuration Options information element of the PDN CONNECTIVITY REQUEST message or BEARER RESOURCE ALLOCATION REQUEST message.

If the network provides the UE with a list of P-CSCF IPv4 or IPv6 addresses in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message or ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message, the UE shall assume that the list is prioritised with the first address within the Protocol Configuration Options information element as the P-CSCF address with the highest priority.

…

The UE can freely select method I or II for P-CSCF discovery, if:

-	the UE is in the home network; or

-	the UE is roaming and the P-CSCF is to be discovered in the visited network.

…

[TS 24.229, clause L.2.2.1C]

A UE supporting the P-CSCF restoration procedure performs one of the following procedures:

A	if the UE used method II for P-CSCF discovery and if the UE receives one or more P-CSCF address(es) in the Protocol Configuration Options information element of a Modify EPS Bearer Context Request message the one or more P-CSCF addresse(s) do not include the address of the currently used P-CSCF, then the UE shall acquire a different P-CSCF address from the one or more P-CSCF addresse(s) in the Modify EPS Bearer Context Request message. If more than one P-CSCF address with the same container identifier (i.e. "P-CSCF IPv6 Address" or "P-CSCF IPv4 Address") are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority.

If the UE used method II for P-CSCF discovery and if the UE has previously sent the "P-CSCF Re-selection support" PCO indicator at PDN creation and if the UE receives one or more P-CSCF address(es) in the Protocol Configuration Options information element of a Modify EPS Bearer Context Request message, then the UE shall acquire a P-CSCF address from the one or more P-CSCF addresse(s) in the Modify EPS Bearer Context Request message. If more than one P-CSCF address with the same container identifier (i.e. "P-CSCF IPv6 Address" or "P-CSCF IPv4 Address") are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority;

B	if the UE uses RFC 6223 [143] as part of P-CSCF restoration procedures, and if the P-CSCF fails to respond to a keep-alive request, then the UE shall acquire a different P-CSCF address using one of the methods I, III and IV for P-CSCF discovery described in the subclause L.2.2.1.

When the UE has acquired the P-CSCF address, the UE shall perform an initial registration as specified in subclause 5.1.

NOTE:	For UEs using procedure A described above, the network ensures that P-CSCF address(es) in the Protocol Configuration Options information element of a Modify EPS Bearer Context Request message is sent only during P-CSCF restoration procedures as defined in subclause 5 of 3GPP TS 23.380 [7D].

10.3.2.3	Test description

10.3.2.3.1	Pre-test conditions

System Simulator:

-	Cell A is configured according to Table 6.3.2.2-1 in [18].

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

10.3.2.3.2	Test procedure sequence

Table 10.3.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a MODIFY EPS BEARER CONTEXT REQUEST message containing new P-CSCF addresses. | <-- | MODIFY EPS BEARER CONTEXT REQUEST | - | - |
| 2 | Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? | --> | MODIFY EPS BEARER CONTEXT ACCEPT | 1 | P |
| - | The following messages are sent to the first P-CSCF address received at step 1. | - | - | - | - |
| 3-10 | Check: Does the UE perform IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11? | - | - | 1 | P |
| 11 | The SS releases the RRC connection | - | - | - | - |

10.3.2.3.3	Specific message contents

Table 10.3.2.3.3-1: MODIFY EPS BEARER CONTEXT REQUEST (Step 1, Table 10.3.2.3.2-1)

| Derivation path: TS 36.508, Table 4.7.3-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol configuration options |  |  |  |
| Container ID 1 | ‘0001’H |  | P-CSCF IPv6 |
| Length of container ID 1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 1 contents | IPv6 address | P-CSCF IPv6 Address different to that allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |
| Container ID 2 | ‘000C’H |  | P-CSCF IPv4 |
| Length of container ID 2 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 2 contents | IPv4 address | P-CSCF IPv4 Address different to that allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |
| Container ID 3 | ‘0001’H |  | P-CSCF_IPv6 |
| Length of container ID 3 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 3 contents | IPv6 address | Additional P-CSCF IPv6 Address different to that allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |
| Container ID 4 | ‘000C’H |  | P-CSCF_IPv4 |
| Length of container ID 4 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 4 contents | IPv4 address | Additional P-CSCF IPv4 Address different to that allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |

| Condition | Explanation |
| --- | --- |
| P-CSCF_IPv6 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message for IMS, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv6 Address Request”. |
| P-CSCF_IPv4 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message for IMS, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv4 Address Request”. |

<End of modified section>