3GPP TSG-RAN5 Meeting #105	R5-246591

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3359 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PDSCH-ConfigBroadcast-r17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the last meeting pdsch-TimeDomainAllocationList-r17 is added to enhance test coverage. However, it cause the inconsistency with the field Time domain resource assignment (TDRA) in DCI format 4_0 for MCCH because PDSCH-CONFIGMCCH is not always configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct and simply the configuration, the PDSCH-CONFIGMCCH always not include (TDRA), and PDSCH-CONFIGMTCH always include (TDRA). MCCH always use the first entry in the PDSCH-TDRA list (list is configured in Table 4.6.3-103). The TDRA field in DCI 4-0 is 0000 MTCH always use the second entry in the PDSCH-TDRA list (list is configured in Table 4.6.7-8). The TDRA field in DCI 4-0 is 0001 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	PDSCH-ConfigBroadcast

Table 4.6.7-8: PDSCH-ConfigBroadcast

| Derivation Path: TS 38.331 [6], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigBroadcast-r17 ::= SEQUENCE { |  |  |  |
| pdschConfigList-r17 SEQUENCE (SIZE (1..maxNrofPDSCH-ConfigPTM-r17) ) OF PDSCH-ConfigPTM-r17 { | 1 entry |  | MCCH |
| PDSCH-ConfigPTM-r17[1] SEQUENCE { |  | entry 1 |  |
| dataScramblingIdentityPDSCH-r17 | 0 |  |  |
| dmrs-ScramblingID0-r17 | Not present |  |  |
| pdsch-AggregationFactor-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| pdschConfigList-r17 SEQUENCE (SIZE (1..maxNrofPDSCH-ConfigPTM-r17) ) OF PDSCH-ConfigPTM-r17 { | 2 entris |  | MTCH |
| PDSCH-ConfigPTM-r17[1] SEQUENCE { |  | entry 1 |  |
| dataScramblingIdentityPDSCH-r17 | 0 |  |  |
| dmrs-ScramblingID0-r17 | Not present |  |  |
| pdsch-AggregationFactor-r17 | Not present |  |  |
| } |  |  |  |
| PDSCH-ConfigPTM-r17[1] SEQUENCE { |  | entry 2 |  |
| dataScramblingIdentityPDSCH-r17 | 1023 |  |  |
| dmrs-ScramblingID0-r17 | Not present |  |  |
| pdsch-AggregationFactor-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| pdsch-TimeDomainAllocationList-r17 | Not present |  | MCCH |
| pdsch-TimeDomainAllocationList-r17 SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation-r16 { | 2 entries |  | MTCH |
| PDSCH-TimeDomainResourceAllocation-r16[1] SEQUENCE { |  | entry 1 |  |
| k0-r16 | Not present | K0=0 |  |
| mappingType-r16 | typeA |  |  |
| startSymbolAndLength-r16 | 72 | S=2, L=6 |  |
| repetitionNumber-r16 | Not present |  |  |
| k0-v1710 | Not present |  |  |
| repetitionNumber-v1730 | Not present |  |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation-r16[2] SEQUENCE { |  | entry 2 |  |
| k0-r16 | Not present | K0=0 |  |
| mappingType-r16 | typeA |  |  |
| startSymbolAndLength-r16 | 53 | S=2, L=12 |  |
| repetitionNumber-r16 | Not present |  |  |
| k0-v1710 | Not present |  |  |
| repetitionNumber-v1730 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| rateMatchPatternToAddModList-r17 | Not present |  |  |
| lte-CRS-ToMatchAround-r17 | Not present |  |  |
| mcs-Table-r17 | Not present |  |  |
| xOverhead-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCCH | PDSCH-ConfigBroadcast-r17 is configured in pdsch-ConfigMCCH-r17 |
| MTCH | PDSCH-ConfigBroadcast-r17 is configured in pdsch-ConfigMTCH-r17 |

<End of modified section 1>