3GPP TSG-RAN5 Meeting #105	R5-246592

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3360 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of DCI format 4-0 for MBS broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the last meeting pdsch-TimeDomainAllocationList-r17 is added to enhance test coverage. However, it cause the inconsistency with the field Time domain resource assignment (TDRA) in DCI format 4_0 for MCCH because PDSCH-CONFIGMCCH is not always configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct and simply the configuration, the PDSCH-CONFIGMCCH always not include (TDRA), and PDSCH-CONFIGMTCH always include (TDRA). MCCH always use the first entry in the PDSCH-TDRA list (list is configured in Table 4.6.3-103). The TDRA field in DCI 4-0 is 0000 MTCH always use the second entry in the PDSCH-TDRA list (list is configured in Table 4.6.7-8). The TDRA field in DCI 4-0 is 0001 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.6.1.5	Physical layer parameters for scheduling of MBS

4.3.6.1.5.1	Physical layer parameters for DCI format 4_0

DCI format 4_0 is used for the scheduling of PDSCH for broadcast in DL cell.

Default physical layer parameters for DCI format 4_0 are specified in Table 4.3.6.1.5.1-1.

Table 4.3.6.1.5.1-1: Physical layer parameters for DCI format 4_0

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the second entry of PDSCH-TimeDomainResourceAllocationList to be used. PDSCH-TimeDomainResourceAllocationList is provided in the pdsch-ConfigMTCH in Table 4.6.7-8. | “0001” | G-RNTI |
|  | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used in Table 4.6.3-103 | “0000” | MCCH-RNTI |
| VRB-to-PRB mapping | Non-interleaved | “0” |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| MCCH change notification | MSB indicates no new MBS service(s) start. LSB indicates no modification of MCCH information other than the change caused by start of new MBS service(s). | “00” | MCCH-RNTI |
|  | Reserved 2 bits | - | G-RNTI |
| Reserved bits | Reserved 14 bits |  |  |

| Condition | Explanation |
| --- | --- |
| MCCH-RNTI | UE is configured to monitor DCI format 4_0 with CRC scrambled by MCCH-RNTI |
| G-RNTI | UE is configured to monitor DCI format 4_0 with CRC scrambled by G-RNTI for MTCH |

<End of modified section 1>