3GPP TSG-RAN5 Meeting #105	R5-246599

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3363 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SIB4 for eRedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Set the default configuration of InterFreqCarrierFreqList-v1800 for the eRedCap test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If pc_supportOfERedCap_r18 = False, InterFreqCarrierFreqList-v1800 set to absent. If pc_supportOfERedCap_r18 = True, InterFreqCarrierFreqList-v1800 set to present and eRedCapAccessAllowed-r18 set to true. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No default configuration of InterFreqCarrierFreqList-v1800 for the eRedCap test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	SIB4

Table 4.6.2-3: SIB4

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | The same number of entries as the configured inter-freq carriers defined in table 6.3.1.2-1 | n denotes the index of the entry |  |
| InterFreqCarrierFreqInfo[n] SEQUENCE { |  | entry n |  |
| dl-CarrierFreq | Downlink NR SSB ARFCN. See table 6.3.1.2-1 |  |  |
| frequencyBandList | MultiFrequencyBandListNR-SIB |  |  |
| frequencyBandListSUL | Not present |  |  |
| nrofSS-BlocksToAverage | 2 |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | RSRP-Range | Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| smtc | SSB-MTC | Table 4.6.3-185 |  |
| ssbSubcarrierSpacing | SubcarrierSpacing | Table 4.6.3-188 |  |
| ssb-ToMeasure | SSB-ToMeasure |  |  |
| deriveSSB-IndexFromCell | false |  | FDD |
|  | true |  | TDD |
| ss-RSSI-Measurement | Not present |  |  |
| q-RxLevMin | -70 | -140dBm |  |
|  | -55 | -110dBm | SIG and FR1 |
|  | ROUND((-110+Delta(NRfn))/2) | NOTE1 and NOTE2. | SIG and FR2 |
| q-RxLevMinSUL | -70 | -140dBm, For RF/RRM test cases | SUL |
|  | Not present |  |  |
|  | -55 | -110dBm | SUL and SIG |
| q-QualMin | Not present |  |  |
|  | -20 | -20dB | QBASED |
| p-Max | Not present |  |  |
| t-ReselectionNR | 0 |  |  |
| t-ReselectionNR-SF | Not present | Not required unless speed-dependent cell re-selection is tested. |  |
| threshX-HighP | 2 | 4dB, this value should be higher than threshServingLow of the serving cell to avoid ping-pong with lower priority cells. |  |
| threshX-LowP | 1 | 2dB |  |
| threshX-Q | Not present |  |  |
| threshX-Q SEQUENCE { |  |  | QBASED |
| threshX-HighQ | 5 | 5dB |  |
| threshX-LowQ | 5 | 5dB |  |
| } |  |  |  |
| cellReselectionPriority | 4 | The same priority as the one used for serving cell in SIB 2. |  |
| cellReselectionSubPriority | Not present | The same subpriority as the one used for serving cell in SIB 2. |  |
| q-OffsetFreq | dBXY with XY = (FLOOR((Delta(NRfn) – Delta(NRfs))/2))*2 (NOTE 3) | This value is type of Q-OffsetRange in TS 38.331 [6] which must be even value when its absolute value is larger than dB5. | FR2 AND NOT AbsoluteValue_Within_dB5 |
|  | dBXY with XY = Delta(NRfn) – Delta(NRfs) |  | FR2 AND AbsoluteValue_Within_dB5 |
|  | dB0 |  |  |
| interFreqNeighCellList | Not present | Not required unless Qoffset configuration is tested. |  |
| interFreqExcludedCellList | Not present | Not required unless Excluded cell list configuration is tested. |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| InterFreqCarrierFreqList-v1700 | Not present |  |  |  |
| InterFreqCarrierFreqList-v1700 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1700 { | The same number of entries as the configured inter-freq carriers defined in table 6.3.1.2-1 | n denotes the index of the entry | pc_supportOfRedCap_r17 |  |
| InterFreqCarrierFreqInfo-v1700[n] SEQUENCE { |  | entry n |  |  |
| interFreqNeighHSDN-CellList-r17 | Not present |  |  |  |
| highSpeedMeasInterFreq-r17 | Not present |  |  |  |
| redCapAccessAllowed-r17 | true |  |  |  |
| ssb-PositionQCL-Common-r17 | Not present |  |  |  |
| interFreqNeighCellList-v1710 | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| InterFreqCarrierFreqList-v1800 | Not present |  |  |  |
| InterFreqCarrierFreqList-v1800 ::= SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1800 { | The same number of entries as the configured inter-freq carriers defined in table 6.3.1.2-1 | n denotes the index of the entry | pc_supportOfERedCap_r18 |  |
| InterFreqCarrierFreqInfo-v1800[n] SEQUENCE { |  | entry n |  |  |
| dl-CarrierFreq-r18 | Not present |  |  |  |
| frequencyBandList-r18 | Not present |  |  |  |
| frequencyBandListAerial-r18 | Not present |  |  |  |
| mobileIAB-CellList-r18 | Not present |  |  |  |
| mobileIAB-Freq-r18 | Not present |  |  |  |
| eRedCapAccessAllowed-r18 | true |  |  |  |
| tn-AreaIdList-r18 | Not present |  |  |  |
| accessAllowed2RxXR-r18 | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |
| NOTE 1: Delta(NRfn) and Delta(NRfs) is derived based on calibration procedure defined in the clause 6.1.3.3. NRfn and NRfs are NR frequencies in dl-CarrierFreq[n] and serving cell frequency on which SIB4 is broadcasted. NOTE 2: ROUND is rounded off to the nearest integer. As an example, ‘1 to 1.49’ set to ‘1’ while ‘1.5 to 2’ to ‘2’ and ‘-2.0 to 1.5’ set to ‘-2’ while ‘-1.49 to -1’ set to ‘-1’. NOTE 3: FLOOR is rounded off to the smaller integer. As an example, '1.0 to 1.99' set to 1, '-1.01 to -2.00' set to -2. |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMin-sul. |
| QBASED | This condition applies to Quality based signalling test cases. |
| AbsoluteValue_Within_dB5 | -5dB <= (Delta(NRfn) – Delta(NRfs)) <= 5dB. |

<End of modified section 1>