3GPP TSG-RAN5 Meeting #105	R5-246600

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3364 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update common test environment for eRedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is not general excution principle for eRedCap test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update TS 38.508-1 clause 4.5B for eRedCap UE to capture general excution principle for eRedCap test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eRedcap testing is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.5B	Common test environment for Vertical UEs

4.5B.1	SNPN-only UEs

This clause defines the test environment which applies to all test cases listed in TS 38.523-2 [19], clause 4.3 and executed for SNPN-only UEs (TS 38.508-2 [10] A.4.1-5/3), unless otherwise specified.

-	Each configured NR Cell is an SNPN cell, i.e., condition ‘SNPN’ (Standalone NPN cell) applies in the default message contents.

-	The SIB1 of each NR Cell includes the NPN-Identity-r16 set to snpn-r16 with the PLMN of the cell and the nid-List-r16 IE configured with only one entry of NID with Assignment mode = 1 and NID value = 0.

-	UE’s "list of subscriber data" is pre-configured with the list of all PLMNs and NID value = 0.

NOTE: PLMNs are specified in Table 4.4.2-3 or clause 6.3.2 unless specified otherwise in the test case.

4.5B.2	RedCap UEs

This clause defines the test environment which applies to all test cases executed against RedCap UEs, unless otherwise specified.

-	The test channel bandwidth is reduced, to a maximum of 20MHz for FR1 and 100MHz for FR2.

-	The following ASN.1 types are configured with condition pc_supportOfRedCap_r17 = TRUE:

-	SIB1 in Table 4.6.1-28,

-	SIB4 in Table 4.6.2-3,

-	RLC-Config in Table 4.6.3-149, and

-	PDCP-Config in Table 4.6.3-99.

-	In case of HD-FDD mode, the following ASN.1 type is configured with condition pc_halfDuplexFDD_TypeA_RedCap_r17 = TRUE:

-	SIB1 in Table 4.6.1-28.

4.5B.3	NR NTN UEs

This clause defines the test environment which applies to all test cases executed against NR NTN UEs operating in NTN specific bands, unless otherwise specified.

-	Each configured NR Cell is an NTN cell.

-	Test uses system information Combination NR-28 and NR-29 for single cell and multicell scenarios respectively.

-	The following RRC message is configured with condition NTN.

-SIB1 in Table 4.6.1-28.

-	The following ASN.1 type is configured with conditions GSO or NGSO depending on the NTN scenario under test.

-SIB19 in Table 4.6.2-18C.

4.5B.4	eRedCap UEs

This clause defines the test environment which applies to all test cases executed against eRedCap UEs, unless otherwise specified.

-	The test channel bandwidth is reduced, to a maximum of 20MHz for FR1.

-	The following ASN.1 types are configured with condition pc_supportOfERedCap_r18 = TRUE:

-	SIB1 in Table 4.6.1-28,

-	SIB4 in Table 4.6.2-3,

-	RLC-Config in Table 4.6.3-149, and

-	PDCP-Config in Table 4.6.3-99.

-	In case of HD-FDD mode, the following ASN.1 type is configured with condition pc_halfDuplexFDD_TypeA_RedCap_r17 = TRUE:

-	SIB1 in Table 4.6.1-28.

<End of modified section 1>