3GPP TSG-RAN5 Meeting # 105	R5-246609

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5368 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of WLAN P-CSCF Reselection test case 20.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 20.6 (“Initial Registration on new P-CSCF / Reactivation requested by network / WLAN”) needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 20.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage does not exist for WLAN P-CSCF restoration procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 20.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1472 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 20.6	Initial Registration on new P-CSCF / Reactivation requested by network / WLAN

20.6.1	Test Purpose (TP)

(1)

with { UE having an established tunnel and performed IMS registration }

ensure that {

when { UE receives an INFORMATIONAL Request message including DELETE payload with REACTIVATION_REQUESTED_CAUSE Notify payload }

then { UE transmits an INFORMATIONAL Response message and initiates new tunnel establishment procedure for IMS PDN }

}

20.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.302, clause 7.2.4.2. Unless otherwise stated these are Rel-13 requirements.

[TS 24.302, clause 7.2.4.2]

On receipt of the INFORMATIONAL request message including "DELETE" payload, indicating that the ePDG is attempting tunnel disconnection, the UE shall:

i)	Close all security associations identified within the DELETE payload (these security associations correspond to outgoing security associations from the UE perspective). If no security associations were present in the DELETE payload, and the protocol ID was set to "1", the UE shall close the IKE security association, and all IPsec ESP security associations that were negotiated within it towards the ePDG; and

ii)	The UE shall delete the incoming security associations corresponding to the outgoing security associations identified in the "DELETE" payload.

The UE shall send an INFORMATIONAL response message. If the INFORMATIONAL request message contained a list of security associations, the INFORMATIONAL response message shall contain a list of security associations deleted in step (ii) above.

If the UE is unable to comply with the INFORMATIONAL request message, the UE shall send INFORMATION response message with either:

i)	A NOTIFY payload of type "INVALID_SPI", for the case that it could not identify one or more of the Security Parameters Indexes in the message from the ePDG; or

ii)	A more general NOTIFY payload type. This payload type is implementation dependent.

If the INFORMATIONAL request message including the DELETE payload contains the REACTIVATION_REQUESTED_CAUSE Notify payload, the UE shall re-establish the IPsec Tunnel for the corresponding PDN connection after its release. The coding of the P-CSCF_RESELECTION_SUPPORT Notify payload is described in subclause 8.2.9.6.

NOTE:	For an IMS PDN connection, the re-establishment of the IPSec tunnel is part of the "Re-establishment of the IP-CAN used for SIP signalling procedure" specified in 3GPP TS 24 229 [67] subclause R.2.2.1B.

20.6.3	                         Test description

20.6.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 in TS 36.508 [18].

UE:

-	None

Preamble:

-	The UE has an established tunnel according to table 4.5A.23.3-1 in TS 36.508[18].

20.6.3.2	Test procedure sequence

Table 20.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an INFORMATIONAL Request message with “DELETE” payload containing REACTIVATION_REQUESTED_CAUSE in Notify payload, indicating that the ePDG is attempting tunnel disconnection. | <-- | INFORMATIONAL Request | - | - |
| 2 | Check: Does the UE transmit an INFORMATIONAL Response message? | --> | INFORMATIONAL Response | 1 | P |
| 3-13 | Check: Does the UE establish IPsec tunnel using IKEv2 protocol and perform IMS registration using new P-CSCF address by executing steps 3-13 of Table 4.5A.23.3-1 in TS 36.508[18]? | - | - | 1 | - |

20.6.3.3	Specific message contents

Table 20.6.3.3-1: INFORMATIONAL request (step 1, Table 20.6.3.2-1)

| Derivation path: IETF RFC 7296 [66] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IKE Header |  |  |  |
| Next Payload | ‘00101010’B | D |  |
| Exchange Type | ‘00100101’B | INFORMATIONAL |  |
| Delete Payload |  |  |  |
| Next Payload | ‘00101001’B | N |  |
| Protocol ID | ‘00000001’B | For IKE_SA |  |
| SPI Size | ‘00000000’B |  |  |
| Notify Payload |  |  |  |
| Next Payload | ‘00000000’B | No Next Payload |  |
| Protocol ID | ‘00000000’B |  |  |
| SPI Size | ‘00000000’B |  |  |
| Notify Message Type | ‘1010 0000 0000 0001’B | REACTIVATION_REQUESTED_CAUSE |  |

Table 20.6.3.3-2: INFORMATIONAL response (step 2, Table 20.6.3.2-1)

| Derivation path: IETF RFC 7296 [66] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IKE Header |  |  |  |
| Next Payload | Not checked |  |  |
| Exchange Type | ‘00100101’B | INFORMATIONAL |  |
| NOTE 1: The order of Payloads/fields is not checked, unless explicitly specified. Additional Payloads/fields are ignored. |  |  |  |

Table 20.6.3.3-3: IKE_AUTH response (step 4, Table 20.6.3.2-1)

| Derivation path: 36.508[18], Table 4.7G-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IKE Header |  |  |  |
| Next Payload | ‘00101111’B | CP |  |
| Exchange Type | ‘00100011’B | IKE_AUTH |  |
| Configuration Payload |  |  |  |
| Next Payload | Set by the SS |  |  |
| CFG Type | ‘00000010’B | CFG_REPLY |  |
| Attribute Type | ‘00010100’B | P_CSCF_IP4_ADDRESS | IPv4 |
| IPv4 Address | P-CSCF IPv4 Address different than what was allocated in preamble |  |  |
| Attribute Type | ‘00010101’B | P_CSCF_IP6_ADDRESS | IPv6 |
| IPv6 Address | P-CSCF IPv6 Address different than what was allocated in preamble |  |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | If the UE requested an IPv4 address |
| IPv6 | If the UE requested an IPv6 address |

<End of modified section>