3GPP TSG-RAN5 Meeting #105	R5-246643

# Orlando, United States, 18th - 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0382 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and clarification of UserUri in clauses 5.5.4.10.6, 5.5.4.10.8 and 5.5.4.10.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The context of a "MC ID" in 33.180 is used only in regards to user authentication (33.180 clause 5.1.2.1). In addition 23.280 clauses 7.4.2.2.5, 7.4.2.2.6 and 8.1.1 also define the "MC ID" solely for identity management (user authentication) purposes. The UserUri of the SignedKmsResponses in Table 5.5.4.10.6-1 and Table 5.5.4.10.8-1 shall be an MC service ID (MCPTT ID, MCVideo ID or MCData ID). - The comments for the SignatureValue in Tables 5.5.4.10.6-1, 5.5.4.10.8-1 and 5.5.4.10.9-1 clarify which values shall be used for FC and P1 of 33.180 F.1.4 but not which MC Service user ID to be used as P0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - UserUri of the SignedKmsResponses in Table 5.5.4.10.6-1 and Table 5.5.4.10.8-1 corrected. - Clarification added to comments for the SignatureValue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.4.10.6, 5.5.4.10.8, 5.5.4.10.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR .... |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

##### 5.5.4.10.6	KMS Certificate

Table 5.5.4.10.6-1: KMS Certificate

| Derivation Path: TS 33.180 [94], clause D.3.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SignedKmsResponse |  |  |  |  |
| Id | “kmsResponse” | arbitrarily selected id which the Signature’s Reference URI refers to |  |  |
| KmsUri | tsc_MCX_KMS_Hostname | The URI of the KMS which issued the key set |  |  |
| UserUri | px_MCPTT_ID_User_A | The user's MCPTT ID |  | MCPTT |
|  | px_MCVideo_ID_User_A | The user's MCVideo ID |  | MCVIDEO |
|  | px_MCData_ID_User_A | The user's MCData ID |  | MCDATA |
| Time | Current system time of the SS | Time stamp of KMS message |  |  |
| ClientReqUrl | same URI as used by the client as Request URI in the HTTP POST requesting the KMS Certificate (KMS Initialize request) |  |  |  |
| KmsMessage |  |  |  |  |
| KmsInit |  |  |  |  |
| Version | "1.0.0" |  |  |  |
| KmsCertificate |  |  |  |  |
| Version | "1.1.0" | The version number of the certificate type |  |  |
| Role | "Root" | This shall indicate whether the certificate is a "Root" or "External" certificate |  |  |
| CertUri | tsc_MCX_KMS_CertUri | The URI of the Certificate (this object) |  |  |
| KmsUri | tsc_MCX_KMS_Hostname | The URI of the KMS which issued the Certificate |  |  |
| Issuer | Not present | (Optional) String describing the issuing entity |  |  |
| ValidFrom | Not present | (Optional) Date from which the Certificate may be used |  |  |
| ValidTo | Not present | (Optional) Date at which the Certificate expires |  |  |
| Revoked | false | (Optional) A Boolean value defining whether a Certificate has been revoked |  |  |
| UserIDFormat | "2" | Shall contain the value '2' |  |  |
| UserKeyPeriod | "2592000" | The number of seconds that each user key issued by this KMS should be used (2592000 seconds are 30 days) |  |  |
| UserKeyOffset | CurrentTimestamp MODULO UserKeyPeriod | UserKeyOffset so that KeyPeriod starts at current system time; CurrentTimestamp is the current system time in seconds since 0h on 1st Jan 1900 |  |  |
| PubEncKey | SAKKE Public Key Z_T derived from master secret z_T according to RFC 6508 | The SAKKE Public Key, "Z_T". This is an OCTET STRING encoding of an elliptic curve point | RFC 6508 [99] |  |
| PubAuthKey | ECCSI Public Key KPAK derived from private key KSAK according to RFC 6507 | The ECCSI Public Key, "KPAK". This is an OCTET STRING encoding of an elliptic curve point | RFC 6507 [98] |  |
| ParameterSet | Not present | (Optional) The choice of parameter set used for SAKKE and ECCSI |  |  |
| KmsDomainList | Not present | (Optional) List of domains associated with the certificate |  |  |
| SignedInfo |  |  |  |  |
| CanonicalizationAlgorithm | "xml-c14n" | XML Signature processing |  |  |
| SignatureAlgorithm | "HMAC-SHA-256" | Hashing algorithm to be applied to sign the SignedInfo with the key given in the KeyInfo |  |  |
| Reference |  |  |  |  |
| URI | “#kmsResponse” | referring to the data object for which the hash is generatet (KMS response element in this case) |  |  |
| DigestAlgorithm | "SHA-256" | Hashing algorithm to be applied to sign the data object |  |  |
| DigestValue | Hash signing the data object (referred to by the URI) |  |  |  |
| SignatureValue | Hash signing the SignedInfo | The signing key is derived from the InK (px_MCX_InK) according to TS 33.180 [94] Annex F.1.4 with FC = 0x52. XPK-ID = InK-ID (px_MCX_InK_ID) and MC Service user ID being the same as used as UserUri in the SignedKmsResponse |  |  |
| KeyInfo |  |  |  |  |
| KeyName | base64 encoded InK-ID (px_MCX_InK_ID) |  |  |  |

<End of modification>

<Start of modification>

##### 5.5.4.10.8	KMS Key Set

Table 5.5.4.10.8-1: KMS Key Set

| Derivation Path: TS 33.180 [94], clause D.3.3.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Signed KmsResponse |  |  |  |  |
| Id | “kmsResponse” | arbitrarily selected id which the Signature’s Reference URI refers to |  |  |
| KmsUri | tsc_MCX_KMS_Hostname | The URI of the KMS which issued the key set |  |  |
| UserUri | px_MCPTT_ID_User_A | The user's MCPTT ID |  | MCPTT |
|  | px_MCVideo_ID_User_A | The user's MCVideo ID |  | MCVIDEO |
|  | px_MCData_ID_User_A | The user's MCData ID |  | MCDATA |
| Time | Current system time of the SS | Time stamp of KMS message |  |  |
| ClientReqUrl | same URI as used by the client as Request URI in the HTTP POST requesting the KMS Key Set (KMS KeyProvision request) |  |  |  |
| KmsMessage |  |  |  |  |
| KmsKeyProv |  |  |  |  |
| Version | "1.0.0" | The version number of the key provision XML |  |  |
| KmsKeySet[1] |  |  |  |  |
| Version | "1.1.0" | The version number of the key set XML |  |  |
| KmsUri | tsc_MCX_KMS_Hostname | The URI of the KMS which issued the key set |  |  |
| CertUri | Not present | (Optional) The URI of the Certificate which may be used to validate the key set |  |  |
| Issuer | Not present | (Optional) String describing the issuing entity |  |  |
| UserUri | px_MCPTT_ID_User_A | The user's MCPTT ID |  | MCPTT |
|  | px_MCVideo_ID_User_A | The user's MCVideo ID |  | MCVIDEO |
|  | px_MCData_ID_User_A | The user's MCData ID |  | MCDATA |
| UserID | Base64 encoded UID generated according to annex F.2.1 of TS 33.180 [94] with MCX-Id as identifier | UID corresponding to the key set | TS 33.180 [94] |  |
| ValidFrom | Not present | (Optional) Date and time from which the key set may be used |  |  |
| ValidTo | Not present | (Optional) Date and time at which the key set expires |  |  |
| KeyPeriodNo | FLOOR((CurrentTimestamp - UserKeyOffset) / UserKeyPeriod) | Current Key Period: CurrentTimestamp is the current system time in seconds since 0h on 1st Jan 1900; UserKeyOffset and UserKeyPeriod are given in the KMS Certificate (Table 5.5.4.10.6-1) in seconds | TS 33.180 [94] |  |
| Revoked | "false" | (Optional) A Boolean value defining whether the key set has been revoked |  |  |
| UserDecryptKey |  | The SAKKE "Receiver Secret Key" (RSK). This is an OCTET STRING encoding of an elliptic curve point | RFC 6508 [99] |  |
| EncryptionAlgorithm | "AES256" | Encryption algorithm to use |  |  |
| KeyInfo |  |  |  |  |
| KeyName | base64 encoded TrK-ID (px_MCX_TrK_ID) |  |  |  |
| CipherData |  |  |  |  |
| CipherValue | encrypted RSK | The encryption key is derived from the TrK (px_MCX_TrK) according to TS 33.180 [94] Annex F.1.4 with FC = 0x51 XPK-ID = TrK-ID (px_MCX_TrK_ID) |  |  |
| UserSigningKeySSK |  | The ECCSI private Key, "SSK". This is an OCTET STRING encoding of an integer; the PVT is generated using the UID as contained in the UserID of the KSM message | RFC 6507 [98] |  |
| EncryptionAlgorithm | "AES256" | Encryption algorithm to use |  |  |
| KeyInfo |  |  |  |  |
| KeyName | base64 encoded TrK-ID (px_MCX_TrK_ID) |  |  |  |
| CipherData |  |  |  |  |
| CipherValue | encrypted SSK | The encryption key is derived from the TrK (px_MCX_TrK) according to TS 33.180 [94] Annex F.1.4 with FC = 0x51 XPK-ID = TrK-ID (px_MCX_TrK_ID) |  |  |
| UserPubTokenPVT |  | The ECCSI public validation token, "PVT". This is an OCTET STRING encoding of an elliptic curve point; the PVT is generated using the UID as contained in the UserID of the KSM message | RFC 6507 [98] |  |
| EncryptionAlgorithm | "AES256" | Encryption algorithm to use |  |  |
| KeyInfo |  |  |  |  |
| KeyName | base64 encoded TrK-ID (px_MCX_TrK_ID) |  |  |  |
| CipherData |  |  |  |  |
| CipherValue | Encrypted PVT | The encryption key is derived from the TrK (px_MCX_TrK) according to TS 33.180 [94] Annex F.1.4 with FC = 0x51 XPK-ID = TrK-ID (px_MCX_TrK_ID) |  |  |
| Signature |  |  |  |  |
| SignedInfo |  |  |  |  |
| CanonicalizationAlgorithm | "xml-c14n" | XML Signature processing |  |  |
| SignatureAlgorithm | "HMAC-SHA-256" | Hashing algorithm to be applied to sign the SignedInfo with the key given in the KeyInfo |  |  |
| Reference |  |  |  |  |
| URI | “#kmsResponse” | referring to the data object for which the hash is generatet (KMS response element in this case) |  |  |
| DigestAlgorithm | "SHA-256" | Hashing algorithm to be applied to sign the data object |  |  |
| DigestValue | Hash signing the data object (referred to by the URI) |  |  |  |
| SignatureValue | Hash signing the SignedInfo | The signing key is derived from the InK (px_MCX_InK) according to TS 33.180 [94] Annex F.1.4 with FC = 0x52, XPK-ID = InK-ID (px_MCX_InK_ID) and MC Service user ID being the same as used as UserUri in the SignedKmsResponse |  |  |
| KeyInfo |  |  |  |  |
| KeyName | base64 encoded InK-ID (px_MCX_InK_ID) |  |  |  |

<End of modification>

<Start of modification>

##### 5.5.4.10.9	Signed KMS Request

Table 5.5.4.10.9-1: Signed KMS Request

| Derivation Path: TS 33.180 [94], clause D.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SignedKmsRequest |  |  |  |  |
| KmsRequest |  |  |  |  |
| Id attribute | any value | value as used as reference in the signature |  |  |
| Version attribute | "1.1.0" |  |  |  |
| UserUri | px_MCPTT_ID_User_A | The user's MCPTT ID |  | MCPTT |
|  | px_MCVideo_ID_User_A | The user's MCVideo ID |  | MCVIDEO |
|  | px_MCData_ID_User_A | The user's MCData ID |  | MCDATA |
| KmsUri | tsc_MCX_KMS_Hostname | The URI of the KMS to which the request is sent |  |  |
| Time | any value | Date/time that the request is made by the client |  |  |
| ClientId | any value if present | A string representing the client |  |  |
| DeviceId | any value if present | A string representing the device |  |  |
| ClientReqUrl | URI with same path as in the request URI of the HTTP request | The resource URI to which the HTTP POST request is sent |  |  |
| KrrList | not present |  |  |  |
| ClientError | not present |  |  |  |
| Signature |  |  |  |  |
| SignedInfo |  |  |  |  |
| CanonicalizationAlgorithm | "http://www.w3.org/TR/2001/REC-xml-c14n-20010315" | XML Signature processing |  |  |
| SignatureAlgorithm | "http://www.w3.org/2001/04/xmldsig-more#hmac-sha256" | Hashing algorithm to be applied to sign the SignedInfo with the key given in the KeyInfo |  |  |
| Reference |  |  |  |  |
| URI | URI referring to the Id of the request | same value as the Id attribute of the request with leading "#" |  |  |
| DigestAlgorithm | "http://www.w3.org/2001/04/xmlenc#sha256" | Hashing algorithm applied to sign the data object |  |  |
| DigestValue | Hash signing the data object (referred to by the URI) |  |  |  |
| SignatureValue | Hash signing the SignedInfo; shall be validated by the SS | The signing key is derived from the InK (px_MCX_InK) according to TS 33.180 [94] Annex F.1.4 with FC = 0x52, XPK-ID = InK-ID (px_MCX_InK_ID) and MC Service user ID being the same as used as UserUri |  |  |
| KeyInfo |  |  |  |  |
| KeyName | base64 encoded InK-ID (px_MCX_InK_ID) |  |  |  |

<End of modification>