3GPP TSG-RAN5 Meeting #105	R5-246652

# Orlando, The United States of America, 18th – 22th Nov 2023

Source:	NTTDOCOMO, INC

Title:	Proposal for new test case related to PLMN operator specific range on ETWS

Agenda item:	6.5.6

Document for:	Endorsement

Background

As discussion paper(R5-242949) mentioned, Japan will start domestic roaming project using 3GPP features. The Image of intercarrier roaming in an emergency is as follows.

000883579.pdf

Figure 1: Image of intercarrier roaming in an emergency

In the case of domertic roaming, Japan has a plan to use “1104” as a common message ID of ETWS service on SIB11.

TS 23.041 9.4.1.2.2	Message Identifier

==

- 1104

| 4352 | 1100 | ETWS CBS Message Identifier for earthquake warning message. |
| --- | --- | --- |
| 4353 | 1101 | ETWS CBS Message Identifier for tsunami warning message. |
| 4354 | 1102 | ETWS CBS Message Identifier for earthquake and tsunami combined warning message. |
| 4355 | 1103 | ETWS CBS Message Identifier for test message. The UE silently discards this message. A UE specially designed for testing purposes may display its contents. |
| 4356 | 1104 | ETWS CBS Message Identifier for messages related to other emergency types. |
| 4357 - 4359 | 1105 - 1107 | ETWS CBS Message Identifier for future extension. |

==

Discussion

We checked current TS36.523-1 test cases in terms of whether 1104 can be checked or not. However, there was no test cases related to it.

TS36.523-1

==

14	ETWS

14.1	ETWS reception in RRC_IDLE state / Duplicate detection

14.2	ETWS reception in RRC_CONNECTED state / Duplicate detection

==

Proposal

I have two proposal to check UE behavior after receiving message ID ”1104” in SIB11.

Proposal 1: RAN5 make a new test case to check whether UE successfully activate the “User Alerting” popup display after receiving “1104” on SIB11. This test case is applied to UEs supporting Rel.8.

If this proposal is agreed, following new test case has already been proposed.

# test case:

R5-246711:

14.1a	ETWS reception in RRC_IDLE state / Duplicate detection / 1104 / Messages related to other emergency types

R5-246712:

14.2a	ETWS reception in RRC_CONNECTED state / Duplicate detection / 1104 / Messages related to other emergency types

# Test applicability:

R5-246713:

Addition of applicability for new test case message ID related to ETWS in IDLE state 14.1a

R5-246714:

Addition of applicability for new test case for message ID related to ETWS in IDLE state 14.2a

Proposal 2: RAN5 updates current test case(14.1/TS36.523-1, 14.2/TS36.523-1)‘s check points to check UE successfully activate the “User Alerting” popup display after receiving “1104” on SIB11.

If this proposal is agreed, following new test case has already been proposed.

# test case:

R5-246709:

Correction of test case 14.1 for message ID related to ETWS

R5-246710:

Correction of test case 14.2 for message ID related to ETWS

References

[1]	3GPP TS 36.523-1: "3rd Generation Partnership Project;Technical Specification Group Radio Access Network;Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC);User Equipment (UE) conformance specification;Part 1: Protocol conformance specification ".

[2]	3GPP TS 23.401: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Technical realization of Cell Broadcast Service (CBS)