3GPP TSG-RAN5 Meeting #105	R5-246753

# Orlando, US, Nov.18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4747 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SL SIG test case 12.2.10.3.1 - Concurrent SL BSR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add NR SL SIG test case 12.2.10.3.1 - Inter-concurrent operation / MAC / Uu procedures related to sidelink / Sidelink buffer status reporting |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR SL SIG test case 12.2.10.3.1 - Inter-concurrent operation / MAC / Uu procedures related to sidelink / Sidelink buffer status reporting is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.10.3.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0528 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.2.10.3	Inter-concurrent operation / MAC / Uu procedures related to sidelink

12.2.10.3.1	Inter-concurrent operation / MAC / Uu procedures related to sidelink / Sidelink buffer status reporting and scheduling request

12.2.10.3.1.1	Test Purpose (TP)

(1)

with { UE is in connected state and configured to perform mode 1 sidelink transmission }

ensure that {

when { Data arrives at one STCH }

then { UE triggers a regular SL-BSR }

}

(2)

with { UE is in connected state and configured to perform mode 1 sidelink transmission }

ensure that {

when { Regular SL-BSR is triggered. UE dosen't has UL grant to send SL-BSR MAC-CE and is configured with SR resources }

then { UE triggers a scheduling request }

}

(3)

with { UE is in connected state and configured to perform mode 1 sidelink transmission }

ensure that {

when { sl-retxBSR-Timer is expired and there still is data to transmit }

then { UE triggers a regular SL-BSR }

}

(4)

with { UE is in connected state and configured to perform mode 1 sidelink transmission }

ensure that {

when { sl-periodicBSR-Timer is expired and there still is data to transmit }

then { UE triggers a periodic SL-BSR }

}

(5)

with { UE is in connected state and configured to perform mode 1 sidelink transmission }

ensure that {

when { SL-CSI reporting has been triggered by SCI from peer UE. UE doesn't has SL grant to send SL CSI report MAC-CE and is configured with SR resource }

then { UE triggers a scheduling request }

}

12.2.10.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, subclause 5.22.1.1, 5.22.1.2, 5.22.1.3.1 and TS 38.213, subclause 16.3.1. Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, subclause 5.22.1.5]

In addition to clause 5.4.4, the Scheduling Request (SR) is also used for requesting SL-SCH resources for new transmission when triggered by the Sidelink BSR (clause 5.22.1.6) or the SL-CSI reporting (clause 5.22.1.7). If configured, the MAC entity performs the SR procedure as specified in this clause unless otherwise specified in clause 5.4.4. For a sidelink logical channel or for SL-CSI reporting, at most one PUCCH resource for SR is configured per UL BWP.

The SR configuration of the logical channel that triggered the Sidelink BSR (clause 5.22.1.6) (if such a configuration exists) is also considered as corresponding SR configuration for the triggered SR (clause 5.4.4). The value of the priority of the triggered SR corresponds to the value of priority of the logical channel that triggered the SR.

Each sidelink logical channel may be mapped to zero or one SR configuration, which is configured by RRC. If the SL-CSI reporting procedure is enabled by RRC, the SL-CSI reporting is mapped to one SR configuration for all PC5-RRC connections. The SR configuration of the SL-CSI reporting triggered according to 5.22.1.7 is considered as corresponding SR configuration for the triggered SR (clause 5.4.4). The value of the priority of the triggered SR corresponds to the value of the priority of the Sidelink CSI Reporting MAC CE.

All pending SR(s) triggered according to the Sidelink BSR procedure (clause 5.22.1.6) prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes an SL-BSR MAC CE which contains buffer status up to (and including) the last event that triggered a Sidelink BSR (see clause 5.22.1.4) prior to the MAC PDU assembly.

All pending SR(s) triggered according to the Sidelink BSR procedure (clause 5.22.1.6) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the SL grant(s) can accommodate all pending data available for transmission in sidelink.

The pending SR triggered according to the SL-CSI reporting for a destination shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the SL grant(s) can accommodate the Sidelink CSI Reporting MAC CE when the SL-CSI reporting that has been triggered but not cancelled or when the triggered SL-CSI reporting is cancelled due to latency non-fulfilment as specified in 5.22.1.7. All pending SR(s) triggered by either Sidelink BSR or Sidelink CSI report shall be cancelled, when RRC configures Sidelink resource allocation mode 2.

[TS 38.323, subclause 5.22.1.6]

The Sidelink Buffer Status reporting (SL-BSR) procedure is used to provide the serving gNB with information about SL data volume in the MAC entity.

RRC configures the following parameters to control the SL-BSR:

-	sl-periodicBSR-Timer, configured by periodicBSR-Timer in sl-BSR-Config;

-	sl-retxBSR-Timer, configured by retxBSR-Timer in sl-BSR-Config;

-	sl-logicalChannelSR-DelayTimerApplied;

-	sl-logicalChannelSR-DelayTimer, configured by logicalChannelSR-DelayTimer in sl-BSR-Config;

-	sl-logicalChannelGroup.

Each logical channel which belongs to a Destination is allocated to an LCG as specified in TS 38.331 [5]. The maximum number of LCGs is eight.

The MAC entity determines the amount of SL data available for a logical channel according to the data volume calculation procedure in TSs 38.322 [3] and 38.323 [4].

An SL-BSR shall be triggered if any of the following events occur:

1>	if the MAC entity has been configured with Sidelink resource allocation mode 1:

2>	SL data, for a logical channel which belongs to an LCG of a Destination, becomes available to the MAC entity; and either

3>	this SL data belongs to a logical channel with higher priority than the priorities of the logical channels containing available SL data which belong to any LCG belonging to the same Destination; or

3>	none of the logical channels which belong to an LCG belonging to the same Destination contains any available SL data.

in which case the SL-BSR is referred below to as 'Regular SL-BSR';

2>	UL resources are allocated and number of padding bits remaining after a Padding BSR has been triggered is equal to or larger than the size of the SL-BSR MAC CE plus its subheader, in which case the SL-BSR is referred below to as 'Padding SL-BSR';

2>	sl-retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains SL data, in which case the SL-BSR is referred below to as 'Regular SL-BSR';

2>	sl-periodicBSR-Timer expires, in which case the SL-BSR is referred below to as 'Periodic SL-BSR'.

1>	else:

2>	Sidelink resource allocation mode 1 is configured by RRC and SL data is available for transmission in the RLC entity or in the PDCP entity, in which case the Sidelink BSR is referred below to as 'Regular SL-BSR'.

For Regular SL-BSR, the MAC entity shall:

1>	if the SL-BSR is triggered for a logical channel for which sl-logicalChannelSR-DelayTimerApplied with value true is configured by RRC:

2>	start or restart the sl-logicalChannelSR-DelayTimer.

1>	else:

2>	if running, stop the sl-logicalChannelSR-DelayTimer.

For Regular and Periodic SL-BSR, the MAC entity shall:

1>	if sl-PrioritizationThres is configured and the value of the highest priority of the logical channels that belong to any LCG and contain SL data for any Destination is lower than sl-PrioritizationThres; and

1>	if ul-PrioritizationThres is configured and the value of the highest priority of the logical channels that belong to any LCG and contain UL data is equal to or higher than ul-PrioritizationThres according to clause 5.4.5:

2>	prioritize the LCG(s) for the Destination(s).

1>	if the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled according to clause 5.4.5 and the UL grant cannot accommodate an SL-BSR MAC CE containing buffer status only for all prioritized LCGs having data available for transmission plus the subheader of the SL-BSR according to clause 5.4.3.1.3, in case the SL-BSR is considered as not prioritized:

2>	prioritize the SL-BSR for logical channel prioritization specified in clause 5.4.3.1;

2>	report Truncated SL-BSR containing buffer status for as many prioritized LCGs having data available for transmission as possible, taking the number of bits in the UL grant into consideration.

1>	else if the number of bits in the UL grant is expected to be equal to or larger than the size of an SL-BSR containing buffer status for all LCGs having data available for transmission plus the subheader of the SL-BSR according to clause 5.4.3.1.3:

2>	report SL-BSR containing buffer status for all LCGs having data available for transmission.

1>	else:

2>	report Truncated SL-BSR containing buffer status for as many LCGs having data available for transmission as possible, taking the number of bits in the UL grant into consideration.

For Padding SL-BSR:

1>	if the number of padding bits remaining after a Padding BSR has been triggered is equal to or larger than the size of an SL-BSR containing buffer status for all LCGs having data available for transmission plus its subheader:

2>	report SL-BSR containing buffer status for all LCGs having data available for transmission;

1>	else:

2>	report Truncated SL-BSR containing buffer status for as many LCGs having data available for transmission as possible, taking the number of bits in the UL grant into consideration.

For SL-BSR triggered by sl-retxBSR-Timer expiry, the MAC entity considers that the logical channel that triggered the SL-BSR is the highest priority logical channel that has data available for transmission at the time the SL-BSR is triggered.

The MAC entity shall:

1>	if the sidelink Buffer Status reporting procedure determines that at least one SL-BSR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the SL-BSR MAC CE plus its subheader as a result of logical channel prioritization according to clause 5.4.3.1:

3>	instruct the Multiplexing and Assembly procedure in clause 5.4.3 to generate the SL-BSR MAC CE(s);

3>	start or restart sl-periodicBSR-Timer except when all the generated SL-BSRs are Truncated SL-BSRs;

3>	start or restart sl-retxBSR-Timer.

2>	if a Regular SL-BSR has been triggered and sl-logicalChannelSR-DelayTimer is not running:

3>	if there is no UL-SCH resource available for a new transmission; or

3>	if UL-SCH resources are available for a new transmission and the UL-SCH resources cannot accommodate the SL-BSR MAC CE plus its subheader as a result of logical channel prioritization according to clause 5.4.3.1; or

3>	if the set of Subcarrier Spacing index values in sl-AllowedSCS-List, if configured for the logical channel that triggered the SL-BSR, does not include the Subcarrier Spacing index associated to the UL-SCH resources available for a new transmission; or

3>	if sl-MaxPUSCH-Duration, if configured for the logical channel that triggered the SL-BSR, is smaller than the PUSCH transmission duration associated to the UL-SCH resources available for a new transmission:

4>	trigger a Scheduling Request.

NOTE 1:	UL-SCH resources are considered available if the MAC entity has been configured with, receives, or determines an uplink grant. If the MAC entity has determined at a given point in time that UL-SCH resources are available, this need not imply that UL-SCH resources are available for use at that point in time.

A MAC PDU shall contain at most one SL-BSR MAC CE, even when multiple events have triggered an SL-BSR. The Regular SL-BSR and the Periodic SL-BSR shall have precedence over the padding SL-BSR.

The MAC entity shall restart sl-retxBSR-Timer upon reception of an SL grant for transmission of new data on any SL-SCH.

All triggered SL-BSRs may be cancelled when the SL grant(s) can accommodate all pending data available for transmission. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes an SL-BSR MAC CE which contains buffer status up to (and including) the last event that triggered an SL-BSR prior to the MAC PDU assembly. All triggered SL-BSRs shall be cancelled, and sl-retx-BSR-Timer and sl-periodic-BSR-Timer shall be stopped, when RRC configures Sidelink resource allocation mode 2.

NOTE 2:	MAC PDU assembly can happen at any point in time between uplink grant reception and actual transmission of the corresponding MAC PDU. SL-BSR and SR can be triggered after the assembly of a MAC PDU which contains an SL-BSR MAC CE, but before the transmission of this MAC PDU. In addition, SL-BSR and SR can be triggered during MAC PDU assembly.

[TS 38.323, subclause 5.22.1.7]

The MAC entity shall for each pair of the Source Layer-2 ID and the Destination Layer-2 ID corresponding to a PC5-RRC connection which has been established by upper layers:

1>	if the SL-CSI reporting has been triggered by an SCI and not cancelled:

…

2>	else if the MAC entity has been configured with Sidelink resource allocation mode 1:

3>	trigger a Scheduling Request.

NOTE:	The MAC entity configured with Sidelink resource allocation mode 1 may trigger a Scheduling Request if transmission of a pending SL-CSI reporting with the sidelink grant(s) cannot fulfil the latency requirement associated to the SL-CSI reporting.

[TS 38.321, subclause 6.1.3.33]

Sidelink Buffer Status Report (SL-BSR) MAC CEs consist of either:

-	SL-BSR format (variable size); or

-	Truncated SL-BSR format (variable size).

SL-BSR and Truncated SL-BSR MAC control elements consist of one Destination Index field, one LCG ID field and one corresponding Buffer Size field per reported target group.

The SL-BSR formats are identified by MAC subheaders with LCIDs as specified in in Table 6.2.1-2.

The fields in the SL-BSR MAC CE are defined as follows:

-	Destination Index: The Destination Index field identifies the destination. The length of this field is 5 bits. The value is set to one index corresponding to SL-DestinationIdentity associated to same destination reported in SL-TxResourceReqList. The value is indexed sequentially from 0 in the same ascending order of SL-DestinationIdentity in SL-TxResourceReqList as specified in TS 38.331 [5];

-	LCG ID: The Logical Channel Group ID field identifies the group of logical channel(s) whose SL buffer status is being reported. The length of the field is 3 bits;

-	Buffer Size: The Buffer Size field identifies the total amount of data available according to the data volume calculation procedure in TSs 38.322 [3] and 38.323 [4] across all logical channels of a logical channel group of a destination after the MAC PDU has been built (i.e. after the logical channel prioritization procedure, which may result the value of the Buffer Size field to zero). The amount of data is indicated in number of bytes. The size of the RLC headers and MAC subheaders are not considered in the buffer size computation. The length of this field is 8 bits. The values for the Buffer Size field are shown in Table 6.1.3.1-2, respectively. For the Truncated SL-BSR format the number of Buffer Size fields included is maximised, while not exceeding the number of padding bits.

Buffer Sizes of LCGs are included in decreasing order of the highest priority of the sidelink logical channel having data available for transmission in each of the LCGs irrespective of the value of the Destination Index field.

NOTE:	Void.

Figure 6.1.3.33-1: SL-BSR and Truncated SL-BSR MAC control element

12.2.10.3.1.3	Test description

12.2.10.3.1.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1

-	System information combination NR-14 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used in NR Cell 1.

-	NR-SS-UE

-	NR-SS-UE1 as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.2.10.3.1.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.2.10.3.1.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 |  |

Preamble:

-	The UE is in state 3N-B as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22 with NR-SS-UE1.

12.2.10.3.1.3.2	Test procedure sequence

Table 12.2.10.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an RRCReconfiguration to provide dedicated SL configuration and configure UE to perform mode 1 sidelink transmission. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE sends an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 4 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 5 | The SS ignores scheduling requests and does not allocate any uplink grant nor sidelink grant. | - | - | - | - |
| 6 | Check: Does the UE transmit a scheduling request? | --> | (SR) | 2 | P |
| 7 | The SS responds to the scheduling request in step 6 by an UL Grant which is enough for SL BSR | <-- | (UL Grant) | - | - |
| 8 | Check: Does the UE transmit a SL BSR? | --> | MAC CE (SL BSR) | 1 | P |
| 9 | The SS waits for retxBSR-Timer expiry on UE side. | - | - | - | - |
| 10 | Check: Does the UE transmit a scheduling request? | --> | (SR) | 3 | P |
| 11 | The SS responds to the scheduling request in step 10 by an UL Grant which is enough for SL BSR | <-- | (UL Grant) | - | - |
| 12 | Check: Does the UE transmit a SL BSR? | --> | MAC CE (SL BSR) | 3 | P |
| 13 | The SS resumes responsing to scheduling requests and allocates uplink grants/sidelink grants. | - | - | - | - |
| 13 | The SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP | - | - |
| 14 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 15 | The SS sends an RRCReconfiguration to reconfigure SL BSR ReTx timer and periodic SL BSR timer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 16 | The UE sends an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 17 | The SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 18 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 19 | The SS ignores scheduling requests and does not allocate any uplink grant nor sidelink grant. | - | - | - | - |
| 20 | The UE transmit a scheduling request. | --> | - | - | - |
| 21 | The SS responds to the scheduling request in step 20 by an UL Grant which is enough for SL BSR | <-- | (UL Grant) | - | - |
| 22 | The UE transmit a SL BSR. | --> | MAC CE (SL BSR) | - | - |
| 23 | The SS waits for sl-periodicBSR-Timer expiry on UE side. | - | - | - | - |
| 24 | Check: Does the UE transmit a scheduling request? | --> | (SR) | 4 | P |
| 25 | The SS responds to the scheduling request in step 24 by an UL Grant which is enough for SL BSR | <-- | (UL Grant) | - | - |
| 26 | Check: Does the UE transmit a SL BSR? | --> | MAC CE (SL BSR) | 4 | P |
| 27 | The SS resumes responsing to scheduling requests and allocates uplink grants/sidelink grants. | - | - | - | - |
| 28 | The SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP | - | - |
| 29 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 30 | The NR-SS-UE1 sends an RRCReconfigurationSidelink message to trigger the UE the perform a SL CSI report. | <-- | NR PC5 RRC: RRCReconfigurationSidelink | - | - |
| 31 | The UE sends an RRCReconfigurationCompleteSidelink message. | --> | NR PC5 RRC: RRCReconfigurationCompleteSidelink | - | - |
| 32 | The NR-SS-UE1 starts to transmite SL CSI-RS | - | - | - | - |
| 33 | The SS ignores scheduling requests and does not allocate any uplink grant nor sidelink grant. | - | - | - | - |
| 34 | Check: Does the UE transmit a scheduling request? | --> | (SR) | 5 | P |
| 35 | The SS responds to the scheduling request in step 33 by an SL Grant which is enough for SL CSI report | <-- | (SL Grant) | - | - |
| 36 | Check: Does the UE transmit a SL CSI report? |  | MAC CE (SL CSI report) | 5 | P |
| 37 | The SS resumes responsing to scheduling requests and allocates uplink grants/sidelink grants. | - | - | - | - |

12.2.10.3.1.3.3	Specific message contents

Table 12.2.10.3.1.3.3-1: RRCReconfiguration (Step 1 and 15, Table 12.2.10.3.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SIDELINK |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 12.2.10.3.1.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  |  |
| setup |  | SL-ConfigDedicatedNR | Table 12.2.10.3.1.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.10.3.1.3.3-2: SL-ConfigDedicatedNR (Table 12.2.10.3.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-7 with condition SCHEDULING and SL_DRB |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |  |
| sl-ScheduledConfig-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| mac-MainConfigSL-r16 SEQUENCE { |  |  |  |  |
| sl-BSR-Config-r16 SEQUENCE { |  |  |  |  |
| periodicBSR-Timer |  | infinity |  | Step 1 |
|  |  | sf64 |  | Step 15 |
| retxBSR-Timer |  | sf80 |  | Step 1 |
|  |  | sf10240 |  | Step 15 |
| logicalChannelSR-DelayTimer |  | sf64 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-RLC-BearerToAddModList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { |  | 1 entry |  |  |
| SL-RLC-BearerConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-MAC-LogicalChannelConfig-r16 SEQUENCE { |  |  |  |  |
| sl-LogicalChannelSR-DelayTimerApplied-r16 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-CSI-Acquisition-r16 |  | Not present |  | Step 1 |
|  |  | enabled |  | Step 15 |
| sl-CSI-SchedulingRequestId-r16 |  | Not present |  | Step 1 |
| sl-CSI-SchedulingRequestId-r16 CHOICE { |  |  |  | Step 15 |
| setup |  | SchedulingRequestId |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.10.3.1.3.3-3: RRCReconfigurationSidelink (Step 30, Table 12.2.10.3.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1A-3 with condition RX and SL_CSI

<End of Change 1>