3GPP TSG-RAN5 Meeting #105	R5-246754

# Orlando, US, Nov.18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4748 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR SL SIG test case 12.2.1.1 - Concurrent idle Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To update test procedure based on MCC TF160 comments: - Change the check in step 31 from “sends SDAP SDUs on odd slot with F verdict” to “sends SDAP SDUs on even slot with P verdict”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Wording of the check in step 31 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion is caused |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.2.1.1	Inter-carrier concurrent operation / Sidelink communication / RRC_IDLE / Transmission

12.2.1.1.1	Test Purpose (TP)

(1)

with { UE is in RRC_IDLE state and its serving cell broadcasts SIB12 including no sl-FreqInfoList but anchor carrier }

ensure that {

when { UE is configured by upper layer to perform NR sidelink transmission }

then { UE considers the anchor carrier to be the highest priority during cell reselection }

}

(2)

with { UE is in RRC_IDLE state and its serving cell broadcasts SIB12}

ensure that {

when { UE reselects to a neighbour cell which also broadcasts SIB12 }

then { UE discards stored SIB12 and performs NR sidelink transmission using sl-TxPoolSelectedNormal in SIB12 of the new serving cell when it is configured by upper layer to perform sidelink transmission }

}

(3)

with { UE is in RRC_IDLE state and its serving cell broadcasts SIB12}

ensure that {

when { valueTag of SIB12 is changed }

then { UE discards stored SIB12 and performs NR sidelink transmission using sl-TxPoolSelectedNormal in SIB12 with new valueTag when it is configured by upper layer to perform sidelink transmission }

}

(4)

with { UE is in RRC_IDLE state and its serving cell broadcasts SIB12 which including sl-FreqInfoList but no sl-TxPoolSelectedNormal }

ensure that {

when { UE is configured by upper layer to perform NR sidelink transmission }

then { UE sends an RRCSetupRequest message to trigger RRC connection establishment procedure }

}

(5)

with { UE is in RRC_IDLE state and its serving cell broadcasts SIB12 including sl-TxPoolExceptional }

ensure that {

when { UE is performing RRC connection establishment procedure and UE is configured by upper layer to perform NR sidelink transmission }

then { UE perform NR sidelink transmission using sl-TxPoolExceptional from the moment the UE initiates RRC connection establishment until receiving an RRCReconfiguration including sl-ConfigDedicatedNR }

}

12.2.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.304, clause 5.2.4.1, TS 38.331, clause 5.2.2.4.13, 5.3.3.1a and 5.8.8. Unless otherwise stated these are Rel-16 requirements.

[TS 38.304, clause 5.2.4.1]

…If the UE is configured to perform both NR sidelink communication and V2X sidelink communication, the UE may consider the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform NR sidelink communication and not perform V2X communication, the UE may consider the frequency providing NR sidelink communication configuration to be the highest priority. If the UE is configured to perform V2X sidelink communication and not perform NR sidelink communication, the UE may consider the frequency providing V2X sidelink communication configuration to be the highest priority.

…

[TS 38.331, clause 5.2.2.4.13]

Upon receiving SIB12, the UE shall:

1>	if the UE has stored at least one segment of SIB12 and the value tag of SIB12 has changed since a previous segment was stored:

2>	discard all stored segments;

1>	store the segment;

1>	if all segments have been received:

2>	assemble SIB12-IEs from the received segments;

2>	if sl-FreqInfoList is included in sl-ConfigCommonNR:

…

3>	if configured to transmit NR sidelink communication:

4>	use the resource pool(s) indicated by sl-TxPoolSelectedNormal, or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.8.8;

…

The UE should discard any stored segments for SIB12 if the complete SIB12 has not been assembled within a period of 3 hours. The UE shall discard any stored segments for SIB12 upon cell (re-)selection.

[TS 38.331, clause 5.3.3.1a]

For NR sidelink communication, an RRC connection establishment is initiated only in the following cases:

1>	if configured by upper layers to transmit NR sidelink communication and related data is available for transmission:

2>	if the frequency on which the UE is configured to transmit NR sidelink communication is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the concerned frequency;

[TS 38.331, clause 5.8.8]

A UE capable of NR sidelink communication that is configured by upper layers to transmit NR sidelink communication and has related data to be transmitted shall:

1>	if the conditions for NR sidelink communication operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

…

3>	else:

4>	if the cell chosen for NR sidelink communication transmission provides SIB12:

5>	if SIB12 includes sl-TxPoolSelectedNormal for the concerned frequency, and a result of sensing on the resources configured in the sl-TxPoolSelectedNormal is available in accordance with TS 38.214 [19]

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on sensing using the pools of resources indicated by sl-TxPoolSelectedNormal for the concerned frequency as defined in TS 38.321 [3];

5>	else if SIB12 includes sl-TxPoolExceptional for the concerned frequency:

6>	from the moment the UE initiates RRC connection establishment or RRC connection resume, until receiving an RRCReconfiguration including sl-ConfigDedicatedNR, or receiving an RRCRelease or an RRCReject; or

…

7>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection (as defined in TS 38.321 [3]) using the pool of resources indicated by sl-TxPoolExceptional for the concerned frequency;

…

12.2.1.1.3	Test description

12.2.1.1.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1, NR Cell 3 and NR Cell 12.

-	TAI of NR Cell 12 is set to “TAI-1”.

-	System information combination NR-14 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used in NR cells.

-	NR-SS-UE

-	NR-SS-UE1 operates as NR sidelink communication device on the resources that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3.

- The UE uses GNSS as the synchronization reference source.

Preamble:

-	The UE is in state 1N-B as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), test mode (On) and GNSS Sync (On) as defined in TS 38.508-1 [4], subclause 4.5.1 using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.2.1.1.3.2	Test procedure sequence

Table 12.2.1.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T0" is applied for Preamble. Configurations marked "T1" are applied at the points indicated in the Main behaviour description in Table 12.2.1.1.3.2-2.

Table 12.2.1.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -77 | OFF | OFF |  |
|  | QRxLevMin | dBm | -140 | N/A | N/A |  |
|  | Srxlev | dB | 63 | N/A | N/A |  |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -77 | -83 | OFF | The power level is such that Rn,Cell 3 < Rs,Cell1 but SrxlevCell 3 > ThreshX, HighP |
|  | QRxLevMin | dBm | -140 | -140 | N/A |  |
|  | Srxlev | dB | 63 | 57 | N/A |  |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | OFF | -83 | -77 | The power level is such that Rn,Cell 3 < Rs,Cell12 |
|  | QRxLevMin | dBm | N/A | -140 | -140 |  |
|  | Srxlev | dB | N/A | 57 | 63 |  |
| T3 | SS/PBCH SSS EPRE | dBm/ SCS | OFF | OFF | -77 | The power level is such that SrxlevCell 12 > 0 |
|  | QRxLevMin | dBm | N/A | N/A | -140 |  |
|  | Srxlev | dB | N/A | N/A | 63 |  |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. |  |  |  |  |  |  |

Table 12.2.1.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS-NW re-adjusts the cell power level according to row "T1" in Table 12.2.1.1.3.2-1. | - | - | - | - |
| 2 | The SS-NW triggers UE to close UE test loop mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 3 | Wait 5s to let the UE to reselect NR Cell 3 and acquire system information. | - | - | - | - |
| 4 | Check: Does UE continuously send SDAP SDUs in the next 5 seconds on the even slot of every radio frame? | - | - | 1 | P |
| 5 | The SS-NW triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 6 | The SS-NW re-adjusts the cell power level according to row "T2" in Table 12.2.1.1.3.2-1. | - | - | - | - |
| 7 | Wait 5s to let the UE to reselect NR Cell 12 and acquire system information. | - | - | - | - |
| 8 | The SS-NW triggers UE to close UE test loop mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 9 | Check: Does UE continuously send SDAP SDUs in the next 5 seconds on the odd slots of every radio frame? | - | - | 2 | P |
| 10 | The SS-NW triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 11 | The SS-NW re-adjusts the cell power level according to row "T3" in Table 12.2.1.1.3.2-1. | - | - | - | - |
| 12 | The SS-NW transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. Bit 1 of Short Message field is set to 1 to indicate the SI modification. | <-- | Short Message | - | - |
| 13 | The SS-NW increases valueTag for SIB12 in the SIB1 message and start to broadcast SIB12 with updated sl-TxPoolSelectedNormal-r16 | - | - | - | - |
| 14 | Wait 5s to let the UE to acquire system information. | - | - | - | - |
| 15 | The SS-NW triggers UE to close UE test loop mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 16 | Check: Does UE continuously send SDAP SDUs in the next 5 seconds on the even slots of every radio frame? | - | - | 3 | P |
| 17 | The SS-NW triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 18 | The SS-NW transmits Short Message on PDCCH addressed to P-RNTI using Short Message field in DCI format 1_0. Bit 1 of Short Message field is set to 1 to indicate the SI modification. | <-- | Short Message | - | - |
| 19 | The SS-NW increase the valueTag for SIB12 in the SIB1 message and starts to broadcast SIB12 without sl-TxPoolSelectedNormal-r16 but with sl-TxPoolExceptional-r16?. | - | - | - | - |
| 20 | Wait 5s to let the UE to acquire system information. | - | - | - | - |
| 21 | The SS triggers UE to close UE test loop mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 22 | Check: Does the UE send an RRCSetupRequest message on NR Cell 12? | --> | NR RRC: RRCSetupRequest | 4 | P |
| - | EXCEPTION: In parallel with step 23 to 30, parallel behaviour defined in table 12.2.1.2.3.2-3 is executed. | - | - | - | - |
| 23-28 | Steps 3 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 12. | - | - | - | - |
| - | EXCEPTION: In parallel with step 29, parallel behaviour defined in table 12.2.1.1.3.2-4 may happen. | - | - | - | - |
| 29 | The SS-NW sends an RRCReconfiguration message with sl-ConfigDedicatedNR including sl-FreqInfoToAddModList and sl-UE-SelectedConfig | <-- | NR RRC: RRCReconfiguration | - | - |
| 30 | The UE sends an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 31 | Check: Does UE continuously send SDAP SDUs in the next 5 seconds on the even slots of every radio frame? | - | - | 5 | P |
| 32 | The SS-NW transmits an OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 33 | The UE transmits an OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |

Table 12.2.1.1.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does UE continuously send SDAP SDUs on the odd slots of every radio frame? | - | - | 5 | P |

Table 12.2.1.1.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE sends a SidelinkUEInformationNR message. | --> | NR RRC: SidelinkUEInformationNR | - | - |

12.2.1.1.3.3	Specific message contents

Table 12.2.1.1.3.3-1: SIB12-IEs (Preamble, NR Cell 1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoList-r16 | Not present |  |  |
| sl-UE-SelectedConfig-r16 | Not present |  |  |
| sl-NR-AnchorCarrierFreqList-r16 SEQUENCE (SIZE (1..maxFreqSL-NR-r16)) OF ARFCN-ValueNR { | 1 entry |  |  |
| ARFCN-ValueNR[1] | absoluteFrequencySSB of Cell 3 |  |  |
| } |  |  |  |
| sl-RadioBearerConfigList-r16 | Not present |  |  |
| sl-RLC-BearerConfigList-r16 | Not present |  |  |
| sl-MeasConfigCommon-r16 | Not present |  |  |
| t400-r16 | Not present |  |  |
| sl-SSB-PriorityNR-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.1.3.3-2: SIB12-IEs (Preamble, NR Cell 3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-BWP-List-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-ConfigCommon-r16 { | 1 entry |  |  |
| SL-BWP-ConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-BWP-PoolConfigCommon-r16 SEUQENECE { |  |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { | 1 entry |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-ResourcePool-r16 SEQUENCE { |  |  |  |
| sl-TimeResource-r16 | 1010101010 | every even slot |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.1.3.3-3: SIB12-IEs (Preamble, step 13 & 19, Table 12.2.1.1.3.2-2, NR Cell 12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-BWP-List-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-ConfigCommon-r16 { | 1 entry |  |  |
| SL-BWP-ConfigCommon-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-BWP-PoolConfigCommon-r16 SEUQENECE { |  |  |  |
| sl-TxPoolSelectedNormal-r16 | Not present |  | Step 19 |
| sl-TxPoolSelectedNormal-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { | 1 entry |  | Preamble Step 13 |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-ResourcePool-r16 SEQUENCE { |  |  |  |
| sl-TimeResource-r16 | 0101010101 | every odd slot | Preamble |
|  | 1010101010 | every even slot | Step 13 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-TxPoolExceptional-r16 | Not present |  | Preamble Step 13 |
| sl-TxPoolExceptional-r16 SEQUENCE { |  |  | Step 19 |
| sl-ResourcePoolID-r16 | 2 |  |  |
| sl-ResourcePool-r16 | SL-ResourcePool | Table 12.2.1.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.1.3.3-4: SL-ResourcePool (Table 12.2.1.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-25 with condition SL_HARQ |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-TimeResource-r16 |  | 0101010101 | every odd slot |  |
| } |  |  |  |  |

Table 12.2.1.1.3.3-5: RRCReconfiguration (Step 29, Table 12.2.1.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SIDELINK |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  |  |
| setup |  | SL-ConfigDedicatedNR | Table 12.2.1.1.3.3-6 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.1.1.3.3-6: SL-ConfigDedicatedNR (Table 12.2.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [6], Table 4.6.6-7 with condition SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { |  | 1 entry |  |  |
| SL-FreqConfig-r16[1] SEQUENCE { |  | SL-FreqConfig | entry 1 |  |
| sl-BWP-ToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-Config-r16 { |  | 1 entry |  |  |
| SL-BWP-Config-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-BWP-PoolConfig-r16 |  | SL-BWP-PoolConfig | Table 12.2.1.1.3.3-7 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.1.1.3.3-7: SL-BWP-PoolConfig (Table 12.2.1.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-3 with condition SELECTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| SL-BWP-PoolConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE { |  |  |  |  |
| sl-PoolToAddModList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { | 1 entry |  |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  | entry 1 |  |  |
| sl-ResourcePool-r16 SEQUENCE { |  |  |  |  |
| sl-TimeResource-r16 | 1010101010 | every even slot |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.1.1.3.3-8: SIB1 (Preamble and step 13 & 19, Table 12.2.1.1.3.2-2, NR Cell 12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| si-SchedulingInfo SEQUENCE { |  |  |  |  |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo { |  | 3 entries |  |  |
| SchedulingInfo[3] SEQUENCE { |  | Set accoding to 38.508-1 [4] Table 4.4.3.1.3-13 | entry 3 entry 1 and entry 2 are kept unchanged |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { |  | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  |  |  |  |
| valueTag |  | 0 |  | Preamble |
|  |  | 1 |  | step 13 |
|  |  | 2 |  | step 19 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of Change 1>