3GPP TSG-RAN5 Meeting #105	R5-246915

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3073 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases 6.4L.3.x for inter-band CA with TxD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test cases 6.4L.3.x Transmit signal quality for inter-band UL CA with TxD are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test cases 6.4L.3.x for inter-band CA with TxD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned test cases will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4L.3.1(new), 6.4L.3.2.1(new), 6.4L.3.2.2(new), 6.4L.3.2.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.4L.2

6.4L.3	Transmit signal quality for inter-band UL CA with Tx Diversity

6.4L.3.1	Frequency error for inter-band UL CA with Tx Diversity

6.4L.3.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency for inter-band UL CA correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.4L.3.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.4L.3.1.3	Minimum conformance requirements

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the frequency error requirement defined in clause 6.4.1 shall apply on the component carrier without Tx Diversity and the frequency error requirement defined in clause 6.4G.1 shall apply on the component carrier configured with Tx Diversity with all component carriers active.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4L.3.1.

6.4L.3.1.4	Test description

This test is covered by clauses 6.4.1 Frequency error and 6.4G.1 Frequency error for Tx Diversity.

6.4L.3.1.5	Test requirement

Same test requirement as 6.4G.1 for the band supporting Tx Diversity and 6.4.1 for the other band.

6.4L.3.2	Transmit modulation quality for inter-band UL CA with Tx Diversity

6.4L.3.2.1	Error Vector Magnitude for inter-band UL CA with Tx Diversity

6.4L.3.2.1.1	Test purpose

For inter-band UL CA with Tx Diversity, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in clause 6.4.2.1.

6.4L.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.4L.3.2.1.3	Minimum conformance requirements

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the transmit modulation quality requirements defined in clause 6.4.2 shall apply on the component carrier without Tx Diversity and the transmit modulation quality requirements defined in clause 6.4G.2 shall apply on the component carrier configured with Tx Diversity with all component carriers active: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4L.3.2.

6.4L.3.2.1.4	Test description

This test is covered by clauses 6.4.2.1 Error Vector Magnitude and 6.4G.2.1 Error Vector Magnitude for Tx Diversity.

6.4L.3.2.1.5	Test requirement

Same test requirement as 6.4G.2.1 for the band supporting Tx Diversity and 6.4.2.1 for the other band.

6.4L.3.2.2	Carrier leakage for inter-band UL CA with Tx Diversity

6.4L.3.2.2.1	Test purpose

Carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement for inter-band UL CA with Tx Diversity is defined for each component carrier and is measured on the component carrier with PRBs allocated.

6.4L.3.2.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.4L.3.2.2.3	Minimum conformance requirements

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the transmit modulation quality requirements defined in clause 6.4.2 shall apply on the component carrier without Tx Diversity and the transmit modulation quality requirements defined in clause 6.4G.2 shall apply on the component carrier configured with Tx Diversity with all component carriers active: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4L.3.2.

6.4L.3.2.2.4	Test description

This test is covered by clauses 6.4.2.2 Carrier leakage and 6.4G.2.2 Carrier leakage for Tx Diversity.

6.4L.3.2.2.5	Test requirement

Same test requirement as 6.4G.2.2 for the band supporting Tx Diversity and 6.4.2.2 for the other band.

6.4L.3.2.3	In-band emissions for inter-band UL CA with Tx Diversity

6.4L.3.2.3.1	Test purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks. The in-band emission requirement for inter-band UL CA with Tx Diversity is defined for each component carrier and is measured on the component carrier with PRBs allocated.

6.4L.3.2.3.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with Tx Diversity.

6.4L.3.2.3.3	Minimum conformance requirements

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the transmit modulation quality requirements defined in clause 6.4.2 shall apply on the component carrier without Tx Diversity and the transmit modulation quality requirements defined in clause 6.4G.2 shall apply on the component carrier configured with Tx Diversity with all component carriers active: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4L.3.2.

6.4L.3.2.3.4	Test description

This test is covered by clauses 6.4.2.3 In-band emissions and 6.4G.2.3 In-band emissions for Tx Diversity.

6.4L.3.2.3.5	Test requirement

Same test requirement as 6.4G.2.3 for the band supporting Tx Diversity and 6.4.2.3 for the other band.

<End of Changes>