3GPP TSG-RAN5 Meeting #105	R5-246918

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1860 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases 6.4H.x for inter-band EN-DC with UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test cases 6.4H.x Transmit signal quality for inter-band EN-DC with UL MIMO are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test cases 6.4H.x for inter-band EN-DC with UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned test cases will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4H.1.3(new), 6.4H.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.4E.2.3.1	In-band emissions for intra-band V2X

6.4E.2.3.1.1	Test purpose

Same test purpose as in clause 6.4E.2.4.1 in TS 38.521-1 [8].

6.4E.2.3.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.2E.1.0. The requirements in this test case can be well covered in clause 6.5.2.3G of TS 36.521-1 [10] and clause 6.4E.2.4 of TS 38.521-1 [8] and don’t need to be tested again.

6.4H	Transmit signal quality for DC with UL MIMO

6.4H.1	Frequency error for DC with UL MIMO

6.4H.1.1

6.4H.1.2

6.4H.1.3	Frequency error for inter-band EN-DC with UL MIMO within FR1

6.4H.1.3.1	Test purpose

Same test purpose as in clause 6.4D.1 in TS 38.521-1 [8] for the NR carrier.

6.4H.1.3.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band EN-DC with UL MIMO.

6.4H.1.3.3	Minimum conformance requirements

For inter-band EN-DC with UL MIMO and uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [5] and in clause 6.4D.1 in TS 38.101-1 [2], respectively, with all component carriers active.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4H.1.3.

6.4H.1.3.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.4D.1 Frequency Error for UL MIMO in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.4D.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.4D.1.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.4D.1.4 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4H.1.3.5	Test requirement

Same test requirement as 6.4D.1.5 in TS 38.521-1 [8].

6.4H.2	Transmit signal quality for DC with UL MIMO

6.4H.2.1

6.4H.2.2

6.4H.2.3	Transmit modulation quality for inter-band EN-DC with UL MIMO

6.4H.2.3.1	Error Vector Magnitude for inter-band EN-DC with UL MIMO within FR1

6.4H.2.3.1.1	Test purpose

Same test purpose as in clause 6.4D.2.1 in TS 38.521-1 [8] for the NR carrier.

6.4H.2.3.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band EN-DC with UL MIMO.

6.4H.2.3.1.3	Minimum conformance requirements

For inter-band EN-DC with UL MIMO and uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.2 in TS 36.101 [5 and in clause 6.4D.2 in TS 38.101-1 [2], respectively, with all component carriers active.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4H.2.3.

6.4H.2.3.1.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.4D.2.1 Error Vector Magnitude for UL MIMO in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.4D.2.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.4D.2.1.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.4D.2.1.4 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4H.2.3.1.5	Test requirement

Same test requirement as 6.4D.2.1 for the band supporting UL MIMO and 6.4.2.1 for the other band.

6.4H.2.3.2	Carrier leakage for inter-band EN-DC with UL MIMO within FR1

6.4H.2.3.2.1	Test purpose

Same test purpose as in clause 6.4D.2.2 in TS 38.521-1 [8] for the NR carrier.

6.4H.2.3.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band EN-DC with UL MIMO.

6.4H.2.3.2.3	Minimum conformance requirements

For inter-band EN-DC with UL MIMO and uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.2 in TS 36.101 [5 and in clause 6.4D.2 in TS 38.101-1 [2], respectively, with all component carriers active.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4H.2.3.

6.4H.2.3.2.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.4D.2.2 Carrier leakage for UL MIMO in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.4D.2.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.4D.2.2.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.4D.2.2.4 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4H.2.3.2.5	Test requirement

Same test requirement as 6.4D.2.2 for the band supporting UL MIMO and 6.4.2.2 for the other band.

6.4H.2.3.3	In-band emissions for inter-band EN-DC with UL MIMO within FR1

6.4H.2.3.3.1	Test purpose

Same test purpose as in clause 6.4D.2.3 in TS 38.521-1 [8] for the NR carrier.

6.4H.2.3.3.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band EN-DC with UL MIMO.

6.4H.2.3.3.3	Minimum conformance requirements

For inter-band EN-DC with UL MIMO and uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.2 in TS 36.101 [5 and in clause 6.4D.2 in TS 38.101-1 [2], respectively, with all component carriers active.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4H.2.3.

6.4H.2.3.3.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.4D.2.3 In-band emissions for UL MIMO in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.4D.2.3.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.4D2.3.4 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.4D.2.3.4 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.4H.2.3.3.5	Test requirement

Same test requirement as 6.4D.2.3 for the band supporting UL MIMO and 6.4.2.3 for the other band.

<End of Changes>