3GPP TSG-RAN5 Meeting #105	R5-247165

# Orlando, USA, 18th Nov. – 22nd Nov. 2024

Title:							MU discussion on OBW for FR2 UL MIMO

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In the previous meeting, MU discussion was started for FR2 UL MIMO test cases [1]. This paper provides our analysis on OBW MU for FR2 UL MIMO. Unless otherwise commented, observations and proposals in this paper are for PC3, NTC, IFF, max device size ≤ 40 cm.

# 2.	Discussion

As observed in [1] and [2], CP-OFDM QPSK Outer_Full is used in MIMO test cases instead of DFT-s-OFDM used in SISO test cases. As a result, UL level for MIMO is degraded from that for SISO due to the difference of MPR, and then OBW MU for MIMO could be larger than that for SISO. The following table shows the summary of UL level and MU for each test point. Note that MBW = 1.5-times of CBW is assumed in MU analysis.

Table 1: UL level and MU for each test point of OBW test case for MIMO

| Frequency sub-range | CBW | UL level | OBW MU |
| --- | --- | --- | --- |
| FR2a | 50 MHz | 15.4 dBm | 0.4%CBW |
|  | 100 MHz |  | 0.4%CBW |
|  | 200 MHz |  | 1.2%CBW |
|  | 400 MHz | 13.4 dBm | TBD |
| FR2b | 50 MHz | 13.6 dBm | 0.4%CBW |
|  | 100 MHz |  | 0.4%CBW |
|  | 200 MHz |  | 1.3%CBW |
|  | 400 MHz | 11.6 dBm | TBD |
| FR2c | 50 MHz | 11.7 dBm | 0.65%CBW |
|  | 100 MHz |  | 0.65%CBW |
|  | 200 MHz |  | TBD |
|  | 400 MHz | 9.7 dBm | TBD |

For CBW = 400 MHz of all frequency sub-ranges and CBW = 200 MHz of FR2c, OBW MU for MIMO could exceed MTSU for SISO. Further analysis is needed to solve this testability issue. For other lower CBWs, MTSU values for SISO can be reused for MIMO.

Proposal 1: Adopt the OBW MU values in Table 1 for MIMO.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals

Proposal 1: Adopt the OBW MU values in Table 1 for MIMO.

# 4.	References

[1] R5-244895, “On the MU and TT for MIMO test cases”, Rohde & Schwarz, RAN5#104

[2] TR 38.905, “Derivation of test points for radio transmission and reception; User Equipment (UE) conformance test cases”, V18.4.0 (2024-09)