3GPP TSG-RAN5 Meeting #105	R5-247168

# Orlando, USA, 18th Nov. – 22nd Nov. 2024

Title:							Update of FR1 MU for configured output power and absolute power tolerance

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

Most of MTSU for FR1 test cases were discussed and defined at the early stages of Release 15. However, UL power assumed in MTSU derivation is currently contradict with the actual test conditions in configured transmitted power and absolute power tolerance. This paper proposes to update MTSU and TT for these test cases by assuming correct UL power.

# 2.	Discussion

In TS 38.521-1 [1], MTSU for Tx test cases is defined in Table F.1.2-1. The following shows extracts about configured transmitted power and absolute power tolerance.

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.4 Configured transmitted power | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.3.4.2 Absolute power tolerance | f ≤ 3.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.4 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 40MHz ±2.2 dB, 80MHz < BW ≤ 100MHz | Test System uncertainty = SQRT (UL Meas Uncer2 + DL Meas Uncer2) |

These values were derived from the MTSU factors of UL absolute power measurement uncertainty (UL MU) and DL absolute power measurement uncertainty (DL MU). The following Table 1 and Table 2 summarize the UL MU values and DL MU values agreed in [2]. As shown in the tables, both UL MU and DL MU depend on frequency range, and UL MU further depends on BW and UL power range.

Table 1: FR1 UL absolute power measurement uncertainty

| UL power range | Frequency range | BW | Uncertainty |
| --- | --- | --- | --- |
| P ≥ 0 dBm | f ≤ 3.0 GHz | BW ≤ 40 MHz | ±0.7 dB |
|  |  | 40 MHz < BW ≤ 100 MHz | ±1.4 dB |
|  | 3.0 GHz < f ≤ 4.2 GHz | BW ≤ 40 MHz | ±1.0 dB |
|  |  | 40 MHz < BW ≤ 100 MHz | ±1.6 dB |
|  | 4.2 GHz < f ≤ 6.0 GHz | BW ≤ 20 MHz | ±1.3 dB |
|  |  | 20 MHz < BW ≤ 40 MHz | ±1.5 dB |
|  |  | 40 MHz < BW ≤ 100 MHz | ±1.6 dB |
| P ≥ -40 dBm | f ≤ 3.0 GHz | BW ≤ 40 MHz | ±1.0 dB |
|  |  | 40 MHz < BW ≤ 100 MHz | ±1.4 dB |
|  | 3.0 GHz < f ≤ 4.2 GHz | BW ≤ 40 MHz | ±1.3 dB |
|  |  | 40 MHz < BW ≤ 100 MHz | ±1.6 dB |
|  | 4.2 GHz < f ≤ 6.0 GHz | BW ≤ 40 MHz | ±1.5 dB |
|  |  | 40 MHz < BW ≤ 100 MHz | ±1.8 dB |
| P ≥ -50 dBm | f ≤ 3.0 GHz | BW ≤ 40 MHz | ±1.5 dB |
|  |  | 40 MHz < BW ≤ 100 MHz | ±1.7 dB |
|  | 3.0 GHz < f ≤ 4.2 GHz | BW ≤ 40 MHz | ±1.8 dB |
|  |  | 40 MHz < BW ≤ 80 MHz | ±1.9 dB |
|  |  | 80 MHz < BW ≤ 100 MHz | ±2.2 dB |
|  | 4.2 GHz < f ≤ 6.0 GHz | BW ≤ 20 MHz | ±2.0 dB |
|  |  | 20 MHz < BW ≤ 80 MHz | ±2.1 dB |
|  |  | 80 MHz < BW ≤ 100 MHz | ±2.2 dB |

Table 2: FR1 DL absolute power measurement uncertainty

| Frequency range | Uncertainty |
| --- | --- |
| f ≤ 3.0 GHz | ±0.7 dB |
| 3.0 GHz < f ≤ 4.2 GHz | ±1.0 dB |
| 4.2 GHz < f ≤ 6.0 GHz | ±1.5 dB |

In [2], it was also agreed that MTSU for configured output power follows the above UL MU. From comparison of Table F.1.2-1 and Table 1, it is clear that the current MTSU for configured output power assumes UL power range of P ≥ 0 dBm.

On the other hand, MTSU for absolute power tolerance was discussed in [3] and agreed to be calculated by RSS of UL MU and DL MU. UL MU for absolute power tolerance can be calculated by SQRT (MTSU2 – DL MU2) using values in Table F.1.2-1 and Table 2, and the calculation result matches the UL MU values for UL power range of P ≥ 0 dBm in Table 1.

In summary, the current MTSU for configured output power and absolute power tolerance is based on an assumption of UL power ≥ 0 dBm. However, as shown in the following test requirement tables in [1], the minimum expected UL power is less than 0 dBm in these test cases. Therefore, there is a contradiction of UL power between the actual test conditions and assumption in MTSU derivation.

Observation 1: The current MTSU for configured output power and absolute power tolerance assumes UL power ≥ 0 dBm, but these test cases actually have test points with UL power < 0 dBm.

Table 6.2.4.5-1: PCMAX configured UE output power

|  | Maximum output power |  |
| --- | --- | --- |
|  | Test ID 1,2 | Test ID 3 |
| Measured UE output power test point 1 | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) |
| Measured UE output power test point 2 | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) |
| Measured UE output power test point 3 | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) |
| Measured UE output power test point 4 | Note 3 | Note 4 |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. Note 2: Power class 3 is default power class unless otherwise stated. Note 3: The maximum output power shall be within the range in Table 6.2.4.5-1a. Note 4: The maximum output power shall be within the range in Table 6.2.4.5-1b. |  |  |

Table 6.3.4.2.5-1: Absolute power tolerance: test point 1

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -17.6 | -14.4 | -12.6 | -11.3 | -10.4 | -9.6 | -8.9 | -8.3 | -7.8 | -7.3 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -18.2 | -14.8 | -12.8 | -11.5 | -10.5 | -9.7 | -9 | -8.3 | -7.8 | -7.4 | -6.5 | -5.8 | -5.2 | -4.7 | -4.2 |
|  | SCS60 | N/A | -15.2 | -13 | -11.8 | -10.7 | -9.8 | -9.1 | -8.5 | -7.9 | -7.5 | -6.6 | -5.9 | -5.3 | -4.8 | -4.3 |
| Power tolerance |  |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3 Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3.4.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

In order to solve the contradiction, we propose to update MTSU values for test points with UL power < 0 dBm based on the correct UL power assumption. Considering UL power at all test points above are higher than -40 dBm, it is appropriate to adopt the UL MU values for UL power range of P ≥ -40 dBm in Table 1 as MTSU for configured output power and use it to calculate MTSU for absolute power tolerance as below.

Table 3: Proposed MTSU for absolute power tolerance with UL power < 0 dBm

| Frequency range | BW | Uncertainty |
| --- | --- | --- |
| f ≤ 3.0 GHz | BW ≤ 40 MHz | ±1.2 dB |
|  | 40 MHz < BW ≤ 100 MHz | ±1.6 dB |
| 3.0 GHz < f ≤ 4.2 GHz | BW ≤ 40 MHz | ±1.6 dB |
|  | 40 MHz < BW ≤ 100 MHz | ±1.9 dB |
| 4.2 GHz < f ≤ 6.0 GHz | BW ≤ 40 MHz | ±2.1 dB |
|  | 40 MHz < BW ≤ 100 MHz | ±2.3 dB |

Proposal 1: Adopt the UL absolute power measurement uncertainty values for UL power range of P ≥ -40 dBm in Table 1 as MTSU for configured output power with UL power < 0 dBm.

Proposal 2: Adopt Table 3 as MTSU for absolute power tolerance with UL power < 0 dBm.

Based on the above updated MTSU, TT should be also updated. Considering FR1 TT values were defined in [4] to not exceed 1.0 dB and 1.4 dB for configured output power and absolute power tolerance, respectively, we propose the following TT values for test points with UL power < 0 dBm.

Table 4: Proposed TT for configured output power and absolute power tolerance with UL power 0 dBm

| Frequency range | BW | TT for configured output power | TT for absolute power tolerance |
| --- | --- | --- | --- |
| f ≤ 3.0 GHz | BW ≤ 40 MHz | 1.0 dB | 1.2 dB |
|  | 40 MHz < BW ≤ 100 MHz | 1.0 dB | 1.4 dB |
| 3.0 GHz < f ≤ 4.2 GHz | BW ≤ 40 MHz | 1.0 dB | 1.4 dB |
|  | 40 MHz < BW ≤ 100 MHz | 1.0 dB | 1.4 dB |
| 4.2 GHz < f ≤ 6.0 GHz | BW ≤ 40 MHz | 1.0 dB | 1.4 dB |
|  | 40 MHz < BW ≤ 100 MHz | 1.0 dB | 1.4 dB |

Proposal 3: Adopt Table 4 as TT for configured output power and absolute power tolerance with UL power < 0 dBm.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals.

Proposal 1: Adopt the UL absolute power measurement uncertainty values for UL power range of P ≥ -40 dBm in Table 1 as MTSU for configured output power with UL power < 0 dBm.

Proposal 2: Adopt Table 3 as MTSU for absolute power tolerance with UL power < 0 dBm.

Proposal 3: Adopt Table 4 as TT for configured output power and absolute power tolerance with UL power < 0 dBm.

# 4.	References

[1] TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, V18.4.0 (2024-09)

[2] R5-183951, “Proposal for Maximum Test System Uncertainty for NR FR1 TRx tests”, Anritsu, Rohde & Schwarz, RAN5#79

[3] R5-184484, “Proposal for remaining Maximum Test System Uncertainty for NR FR1 TRx tests”, Anritsu, Rohde & Schwarz, RAN5#80

[4] R5-183952, “Wayforward on TT & MU values in FR1”, Orange, Telecom Italia, Vodafone, CMCC, China Unicom, China Telecom, AT&T, Dish, RAN5#79