3GPP TSG-RAN5 Meeting #105 	R5-247191

# Orlando, USA, 18th Nov. 2024 - 22nd Nov. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5528 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of test condition in 3DL CA Rx test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test, LTE_CA_Rel12_3DL-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are two types of intra-band contiguous + inter-band 3DL CA depending on whether intra-band contiguous band is configured in PCC-SCC or SCC-SCC. In test configuration tables of 3DL CA Rx test cases, the existing notes, e.g., Note 9 in Table 7.4A.5.4.1-1, clarify how to determine CC combination of intra-band contiguous band configured in PCC-SCC when it supports multiple CC combinations with the same NRB_agg. However, it is not clarified for intra-band contiguous band configured in SCC-SCC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New notes were added to 3DL CA Rx test cases in order to clarify how to determine CC combination of intra-band contiguous band configured in SCC-SCC when it supports multiple CC combinations with the same NRB_agg. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration will be unclear for intra-band contiguous band configured in SCC-SCC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4A.5, 7.5A.5, 7.6.1A.5, 7.6.2A.5, 7.6.3A.5, 7.8.1A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Unchanged sections skipped>>

### 7.4A.5	Maximum input level for 3DL CA

#### 7.4A.5.1	Test purpose

Maximum input level for CA test verifies the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB.

#### 7.4A.5.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 3DL with CA configurations in Table 7.1-2a.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

#### 7.4A.5.3	Minimum conformance requirements

The Minimum conformance requirements are defined in clause 7.4A.0.

#### 7.4A.5.4	Test description

##### 7.4A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in tables of 5.4.2A.1 as applicable. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 7.4A.5.4.1-1 to 7.4A.5.4.1-2. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for a CA_XD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | 64-QAM |  | Lowest NRB_agg | N/A | 64-QAM |  | Lowest NRB_agg | N/A | 64-QAM |  | Lowest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB_agg | N/A | 64-QAM |  | Highest NRB_agg | N/A | 64-QAM |  | Highest NRB_agg |
| Default Test Settings for a CA_XA-YA-ZA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | All RBs | Y | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 3 | Z | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| Default Test Settings for CA_XB-YA and CA_XC-YA Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB_agg | N/A | 64-QAM |  | Highest NRB_agg | N/A | 64-QAM |  | Highest NRB |
| 29 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | Lowest NRB_agg | N/A | 64-QAM |  | Lowest NRB_agg | N/A | 64-QAM |  | Highest NRB |
| 311 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB | N/A | 64-QAM |  | Highest NRB_agg | N/A | 64-QAM |  | Highest NRB_agg |
| 411 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB | N/A | 64-QAM |  | Lowest NRB_agg | N/A | 64-QAM |  | Lowest NRB_agg |
| Default Test Settings for CA_XA-YB and CA_XA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB_agg | N/A | 64-QAM |  | Highest NRB_agg | N/A | 64-QAM |  | Highest NRB |
| 29 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | Lowest NRB_agg | N/A | 64-QAM |  | Lowest NRB_agg | N/A | 64-QAM |  | Highest NRB |
| 311 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB | N/A | 64-QAM |  | Highest NRB_agg | N/A | 64-QAM |  | Highest NRB_agg |
| 411 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB | N/A | 64-QAM |  | Lowest NRB_agg | N/A | 64-QAM |  | Lowest NRB_agg |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration, The applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band & Intra-band contiguous + Inter-band: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: Intra-band contiguous: X corresponds to the band in the CA Configuration. E.g. for CA_41D, X=41 Note 5: Inter-band: X,Y,Z correspond to the 1st, 2nd and 3rd band in the CA Configuration. E.g. for CA_1A-3A-19A, X=1, Y=3, Z=19 Note 6: Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1C-3A, X=1 and Y=3 Note 7: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 9: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_SCCx = NRB_SCCx+1 for testing. If no such combination is supported, choose Combination with maximum NRB_SCCx for testing. |  |  |  |  |  |  |  |  |  |  |  |  |

## <<Unchanged sections skipped>>

### 7.5A.5	Adjacent Channel Selectivity (ACS) for 3DL CA

#### 7.5A.5.1	Test purpose

Adjacent channel selectivity for CA test verifies the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when other e-NodeB transmitters exist in the adjacent channel.

#### 7.5A.5.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 3DL with CA configurations in Table 7.1-2a.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

#### 7.5A.5.3	Minimum conformance requirements

The Minimum conformance requirements are defined in clause 7.5A.0.

#### 7.5A.5.4	Test description

##### 7.5A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in tables of 5.4.2A.1 as applicable. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in tables 7.5A.5.4.1-1 to 7.5A.5.4.1-2. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.5A.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3) | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for a CA_XD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XA-YA-ZA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | All RBs | Y | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 3 | Z | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| Test Settings for CA_1A-18A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 50@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 2 | 1 | Low | 75@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 3 | 1 | Low | 100@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 4 | 1 | Low | 100@0 | All RBs | 18 | Mid | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 5 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 6 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 7 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 8 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 25 |
| 9 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 10 | 28 | High | 25@0 | All RBs | 1 | Low | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| 11 | 28 | Mid | 25@25 | All RBs | 1 | Low | N/A | All RBs | 18 | Low | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 12 | 28 | High | 25@25 | All RBs | 1 | High | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| Default Test Settings for CA_XB-YA and CA_XC-YA Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 29 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 312 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 412 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Default Test Settings for CA_XA-YB and CA_XA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 29 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 312 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 412 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration, The applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band & Intra-band contiguous + Inter-band: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: Intra-band contiguous: X corresponds to the band in the CA Configuration. E.g. for CA_41D, X=41 Note 5: Inter-band: X,Y,Z correspond to the 1st, 2nd and 3rd band in the CA Configuration. E.g. for CA_1A-3A-19A, X=1, Y=3, Z=19 Note 6: Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1C-3A, X=1 and Y=3 Note 7: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 9: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. Note 12: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_SCCx = NRB_SCCx+1 for testing. If no such combination is supported, choose Combination with maximum NRB_SCCx for testing. |  |  |  |  |  |  |  |  |  |  |  |  |

## <<Unchanged sections skipped>>

#### 7.6.1A.5	In-band blocking for 3DL CA

##### 7.6.1A.5.1	Test Purpose

Same test purpose as in clause 7.6.1A.1.

##### 7.6.1A.5.2	Test Applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support 3DL with Intra-band contiguous CA or 3DL with Inter-band CA or 3DL with Intra-band contiguous and Inter-band CA.

This test case also applies to all types of E-UTRA UE release 11 and forward that support 3DL with Intra-band non-contiguous and Inter-band CA or 3DL with Intra-band non-contiguous and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 3DL with FDD-TDD CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.7.6.1A.5.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.1A.0.

##### 7.6.1A.5.4	Test Description

###### 7.6.1A.5.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.6.1A.5.4.1-1 through table 7.6.1A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6.1A.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3) | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for a CA_XD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XA-YA-ZA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | All RBs | Y | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 3 | Z | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| Test Settings for CA_1A-18A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 50@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 2 | 1 | Low | 75@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 3 | 1 | Low | 100@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 4 | 1 | Low | 100@0 | All RBs | 18 | Mid | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 5 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 6 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 7 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 8 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 25 |
| 9 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 10 | 28 | High | 25@0 | All RBs | 1 | Low | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| 11 | 28 | Mid | 25@25 | All RBs | 1 | Low | N/A | All RBs | 18 | Low | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 12 | 28 | High | 25@25 | All RBs | 1 | High | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| Default Test Settings for CA_XB-YA and CA_XC-YA Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 29 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 312 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 412 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Default Test Settings for CA_XA-YB and CA_XA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 29 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 312 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 412 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration, The applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band & Intra-band contiguous + Inter-band: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: Intra-band contiguous: X corresponds to the band in the CA Configuration. E.g. for CA_41D, X=41 Note 5: Inter-band: X,Y,Z correspond to the 1st, 2nd and 3rd band in the CA Configuration. E.g. for CA_1A-3A-19A, X=1, Y=3, Z=19 Note 6: Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1C-3A, X=1 and Y=3 Note 7: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 9: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. Note 12: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_SCCx = NRB_SCCx+1 for testing. If no such combination is supported, choose Combination with maximum NRB_SCCx for testing. |  |  |  |  |  |  |  |  |  |  |  |  |

## <<Unchanged sections skipped>>

#### 7.6.2A.5	Out-of-band blocking for 3DL CA

##### 7.6.2A.5.1	Test Purpose

Same test purpose as in clause 7.6.2A.3.1.

##### 7.6.2A.5.2	Test Applicability

This test case applies to all types of E-UTRA UE and forward that support 3DL with CA configurations in Table 7.1-2a.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

##### 7.6.2A.5.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.2A.0.

##### 7.6.2A.5.4	Test Description

###### 7.6.2A.5.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.6.2A.5.4.1-1 through table 7.6.2A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6.2A.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB ” columns |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3) | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for a CA_XD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XA-YA-ZA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | All RBs | Y | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 3 | Z | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| Test Settings for CA_1A-18A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 50@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 2 | 1 | Low | 75@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 3 | 1 | Low | 100@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 4 | 1 | Low | 100@0 | All RBs | 18 | Mid | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 5 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 6 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 7 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 8 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 25 |
| 9 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 10 | 28 | High | 25@0 | All RBs | 1 | Low | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| 11 | 28 | Mid | 25@25 | All RBs | 1 | Low | N/A | All RBs | 18 | Low | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 12 | 28 | High | 25@25 | All RBs | 1 | High | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| Default Test Settings for CA_XB-YA and CA_XC-YA Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 212 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for CA_XA-YB and CA_XA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 212 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band & Intra-band contiguous + Inter-band: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: Intra-band contiguous: X corresponds to the band in the CA Configuration. E.g. for CA_41D, X=41 Note 5: Inter-band: X,Y,Z correspond to the 1st, 2nd and 3rd band in the CA Configuration. E.g. for CA_1A-3A-19A, X=1, Y=3, Z=19 Note 6: Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1C-3A, X=1 and Y=3 Note 7: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 9: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. Note 12: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_SCCx = NRB_SCCx+1 for testing. If no such combination is supported, choose Combination with maximum NRB_SCCx for testing. |  |  |  |  |  |  |  |  |  |  |  |  |

## <<Unchanged sections skipped>>

#### 7.6.3A.5	Narrow band blocking for 3DL CA

##### 7.6.3A.5.1	Test Purpose

Same test purpose as 7.6.3A.1.

##### 7.6.3A.5.2	Test Applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support 3DL with Intra-band contiguous CA or 3DL with Inter-band CA or 3DL with Intra-band contiguous and  Inter-band CA.

This test case also applies to all types of E-UTRA UE release 11 and forward that support 3DL with Intra-band non-contiguous and  Inter-band CA or 3DL with Intra-band non-contiguous and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 3DL with FDD-TDD CA.

##### 7.6.3A.5.3	Minimum Conformance Requirements

Same as in clause 7.6.3A.1.

##### 7.6.3A.5.4	Test Description

###### 7.6.3A.5.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.6.3A.5.4.1-1 through table 7.6.3A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6.3A.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3) | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for a CA_XD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | X | Mid/CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XA-YA-ZA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | All RBs | Y | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 3 | Z | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| Test Settings for CA_1A-18A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 50@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 2 | 1 | Low | 75@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 3 | 1 | Low | 100@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 4 | 1 | Low | 100@0 | All RBs | 18 | Mid | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 5 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 6 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 7 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 8 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 25 |
| 9 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 10 | 28 | High | 25@0 | All RBs | 1 | Low | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| 11 | 28 | Mid | 25@25 | All RBs | 1 | Low | N/A | All RBs | 18 | Low | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 12 | 28 | High | 25@25 | All RBs | 1 | High | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| Default Test Settings for CA_XB-YA and CA_XC-YA Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 29 | X | Mid/CC1 | REFSENS | All RBs | X | Mid/CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 312 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 412 | Y | Mid | REFSENS | All RBs | X | Mid/CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Default Test Settings for CA_XA-YB and CA_XA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 29 | Y | Mid/CC1 | REFSENS | All RBs | Y | Mid/CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 312 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 412 | X | Mid | REFSENS | All RBs | Y | Mid/CC1 | N/A | All RBs | Y | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band & Intra-band contiguous + Inter-band: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: Intra-band contiguous: X corresponds to the band in the CA Configuration. E.g. for CA_41D, X=41 Note 5: Inter-band: X,Y,Z correspond to the 1st, 2nd and 3rd band in the CA Configuration. E.g. for CA_1A-3A-19A, X=1, Y=3, Z=19 Note 6: Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1C-3A, X=1 and Y=3 Note 7: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 9: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. Note 12: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_SCCx = NRB_SCCx+1 for testing. If no such combination is supported, choose Combination with maximum NRB_SCCx for testing. |  |  |  |  |  |  |  |  |  |  |  |  |

## <<Unchanged sections skipped>>

#### 7.8.1A.5	Wideband intermodulation for 3DL CA

##### 7.8.1A.5.1	Test purpose

Same as 7.8.1A.1

##### 7.8.1A.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support 3DL with Intra-band contiguous CA or 3DL with Inter-band CA, or 3DL with Inter-band and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 11 and forward that support 3DL with Intra-band non-contiguous CA and Inter-band CA, or 3DL with Intra-band non-contiguous and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 3DL  with FDD-TDD CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

##### 7.8.1A.5.3	Minimum conformance requirements

Same as 7.8.1A.1.

##### 7.8.1A.5.4	Test description

7.8.1A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.8.1A.5.4.1-1 through table 7.8.1A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.8.1A.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3) | DL alloc | UL MOD | DL MOD | UL alloc | DL alloc | UL MOD | DL MOD | UL alloc | DL alloc |
| Default Test Settings for a CA_XD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/ CC1 | REFSENS | All RBs | X | Mid/ CC2 | N/A | All RBs | X | Mid/ CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | LowHighest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 2 | X | Mid/ CC1 | REFSENS | All RBs | X | Mid/ CC2 | N/A | All RBs | X | Mid/ CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | HighLowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Default Test Settings for a CA_XA-YA-ZA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | All RBs | Y | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Z | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| 3 | Z | Mid | REFSENS | All RBs | X | Mid | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB |
| Test Settings for CA_1A-18A-28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 50@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 2 | 1 | Low | 75@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 3 | 1 | Low | 100@0 | All RBs | 18 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 4 | 1 | Low | 100@0 | All RBs | 18 | Mid | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 5 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 |
| 6 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 |
| 7 | 18 | Low | 25@25 | All RBs | 1 | Low | N/A | All RBs | 28 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 8 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 25 |
| 9 | 18 | Mid | 25@50 | All RBs | 1 | Low | N/A | All RBs | 28 | High | N/A | All RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 10 | 28 | High | 25@0 | All RBs | 1 | Low | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| 11 | 28 | Mid | 25@25 | All RBs | 1 | Low | N/A | All RBs | 18 | Low | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 |
| 12 | 28 | High | 25@25 | All RBs | 1 | High | N/A | All RBs | 18 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 |
| Default Test Settings for CA_XB-YA and CA_XC-YA Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | X | Mid/ CC1 | REFSENS | All RBs | X | Mid/ CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 29 | X | Mid/ CC1 | REFSENS | All RBs | X | Mid/ CC2 | N/A | All RBs | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | HLowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 312 | Y | Mid | REFSENS | All RBs | X | Mid/ CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 412 | Y | Mid | REFSENS | All RBs | X | Mid/ CC1 | N/A | All RBs | X | Mid/CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Default Test Settings for CA_XA-YB and CA_XA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Y | Mid/ CC1 | REFSENS | All RBs | Y | Mid/ CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB |
| 29 | Y | Mid/ CC1 | REFSENS | All RBs | Y | Mid/ CC2 | N/A | All RBs | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB |
| 312 | X | Mid | REFSENS | All RBs | Y | Mid/ CC1 | N/A | All RBs | Y | Mid/ CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 412 | X | Mid | REFSENS | All RBs | Y | Mid/ CC1 | N/A | All RBs | Y | Mid/ CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band & Intra-band contiguous + Inter-band: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 4: Intra-band contiguous: X corresponds to the band in the CA Configuration. E.g. for CA_41D, X=41 Note 5: Inter-band: X,Y,Z correspond to the 1st, 2nd and 3rd band in the CA Configuration. E.g. for CA_1A-3A-19A, X=1, Y=3, Z=19 Note 6: Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration. E.g. for CA_1C-3A, X=1 and Y=3 Note 7: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 9: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. Note 12: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_SCCx = NRB_SCCx+1 for testing. If no such combination is supported, choose Combination with maximum NRB_SCCx for testing. |  |  |  |  |  |  |  |  |  |  |  |  |

## <<End of change>>