# Title:	Test Tolerances for FR2 intra-frequency cell reselection test case

Source:	Huawei, HiSilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for FR2 intra-frequency Cell reselection test case.

The test cases in question, from 38.533, are:

- 7.1.1.1	NR SA FR2 cell re-selection

The applicability of the analysis is:

- UE power class 3;

- FR2a (n257, n258) and FR2b (n260, n261);

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v15.18.0 Annex A.7.1.1.1. And the changes which plans to be made in RAN4 #104 are yellow highlighted.

#### A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case

##### A.7.1.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements specified in clause 4.2.2.3.

##### A.7.1.1.1.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.7.1.1.1.2-1, A.7.1.1.1.2-2 and A.7.1.1.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.7.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.7.1.1.1.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cell |  | 1, 2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 135 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2 | 35 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.7.1.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC |  | 1 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| configuration |  | 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  |  | OP.4 |  |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  | 1, 2 | 66 |  |  | 66 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -138 |  |  | -138 |  |  |
|  |  | 2 | -135 |  |  | -135 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  |  | Rough |  |  |
| Es/Iot BB Note 5 | dB | 1 | 7.45 | -3.55 | 0.95 | -infinity | 0.95 | -3.55 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 8 | -3 | 1.5 | -infinity | 1.5 | -3 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -96 | -91.5 | -infinity | -91.5 | -96 |
|  |  | 2 | -82 | -93 | -88.5 | -infinity | -88.5 | -93 |
| Io on SSB symbols | dBm/95.04 MHz | 1 | -60.53 | -67.40 | -65.34 | -69.17 | -65.34 | -67.40 |
| of each cell |  | 2 | -57.52 | -64.39 | -62.33 | -66.16 | -62.33 | -64.39 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

##### A.7.1.1.1.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 130 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to an already detected cell shall be less than 27 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2.2.3-1 in clause 4.2.2.3

Tevaluate, NR_ intra	See Table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 129.28 s, allow 130 s for the cell re-selection delay to a newly detectable cell and 26.88 s for the cell re-selection delay to an already detected cell in the test case, which we allow 27 s.

# 3. Discussion

The test consists of 2 test configurations, each test configuration has 3 time periods: T1, T2 and T3. To analyse the test case, it is helpful to visualise these as pie-charts, showing the distribution of power, as below. The distribution of power is identical for all 2 test configurations.

During T1, Cell 2 is powered off and Cell 1 is powered on. The DUT is expected to keep camping on Cell 1.

During T2, Cell 2 is powered. The DUT is expected to search and evaluate candidate intra-frequency neighbour cells. After detecting and evaluating the DUT determines that Cell 2 is ranked better than Cell 1. Then it shall reselect Cell 2 as its camping cell.

During T3, Power of Cell 2 is weaker and power of Cell 1 is stronger. The DUT is expected to start evaluating Cell 1. After evaluation the DUT shall find that Cell 1 is ranked better than Cell 2. Then it shall reselect Cell 1 as the camping cell.

Selection criterion

To apply intra-frequency cell reselection related requirements specified in 38.133 cl.4.2.2.2 in TC 7.1.1.1. Several conditions should be satisfied.

- First, UE only perform intra-frequency measurement when measurement restriction rule specified in 38.304 are not satisfied:

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	Second, following conditions specified in 38.133 should be satisfied to apply intra-frequency reselection related requirements:

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3 dB better ranked in FR1 or 4.5 dB better ranked in FR2.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

- the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2 if the current serving cell is among them.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

It is helpful to show the power levels for each cell versus time, together with the thresholds, as shown below:

The reselection criteria mentioned above are given in following extract from TS 38.304 cl.5.2.4.6：

#### 5.2.4.6	Intra-frequency and equal priority inter-frequency Cell Reselection criteria

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst - Qoffsettemp Rn = Qmeas,n -Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2.

The cells shall be ranked according to the R criteria specified above by deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If rangeToBestCell is not configured, the UE shall perform cell reselection to the highest ranked cell. If this cell is found to be not-suitable, the UE shall behave according to clause 5.2.4.4.

And the side conditions mentioned above are defined in the following extract from TS 38.133:

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-2: Conditions for intra-frequency cell re-selection in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -107.3 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -107.3 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n260 | -104.3 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -107.3 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -99.3 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -99.3 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n260 | -96.3 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -99.3 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Band n260 has the tighest minimum SSB RP and Es/Iot requirement.

In summary,

- minimum SSB RP of Cell 1 in T1, T2 and T3 shall be no less than -105.5 dBm/120kHz for UE PC3 including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2.

- minimum SSB RP of Cell 2 in T2 and T3 shall be no less than -105.5 dBm/120kHz for UE PC3 including the UE multi-band relaxation factor ∆MBp,n specified in clause 6.2.1 of TS 38.101-2.

- Minimum Es/Iot of Cell 1 in T1, T2 and T3 shall be no less than -4 dB.

- Minimum Es/Iot of Cell 2 in T2 and T3 shall be no less than -4 dB.

- UE measured (Cell 1 S-value - SintraSearchP) shall be no larger than 0 in T2.

- UE measured (Cell 2 R-value – Cell 1 R-value) shall be no less than 4.5dB in T2

- UE measured (Cell 2 S-value - SintraSearchP) shall be no larger than 0 in T3.

- UE measured (Cell 1 R-value – Cell 2 R-value) shall be no less than 4.5dB in T3

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters need to be copied from TS 38.133 Table A.7.1.1.1.2-1, Table A.7.1.1.1.2-2 and Table A.7.1.1.1.2-3. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

c) Uncertainties

The SS provides 2 NR Cells. We propose to control the following parameters:

- AWGN absolute power, Noc averaged over BWConfig uncertainty, ±5.65 dB,

- AWGN absolute power, Noc averaged over measurement PRBs uncertainty, ±5.65 dB,

- Ratio of cell 1 signal / AWGN, Ês1 / Noc averaged over BWConfig uncertainty, ±0.3 dB

- Ratio of cell 1 signal / AWGN, Ês1 / Noc averaged over measurement PRBs uncertainty, ±0.3 dB

- Ratio of cell 2 signal / AWGN, Ês2 / Noc averaged over BWConfig uncertainty, ±0.3 dB

- Ratio of cell 2 signal / AWGN, Ês2 / Noc averaged over measurement PRBs uncertainty, ±0.3 dB

- Spherical coverage gain variation of AoA #1, Spherical coverage gain variationAoA1, ±6.4dB.

- UE gain G variation, ±15 dB.

This choice forms a minimum set, so the superposition principle can be applied if necessary. Spherical coverage gain variation is not considered, because in these TCs the signals come from Rx beam peak direction, but is greyed out and left as a placeholder.

The UE gain variation is defined in 38.133 Annex B.2.1.5:

#### B.2.1.5.1	Gain to SS-RSRP measurement point for Rx Beam Peak angle of arrival

In clause 5.1.1 of TS 38.215 [4] SS-RSRP is defined to be measured based on the combined signal from antenna elements corresponding to a given receiver branch. The reference point for requirement parameters from the UE perspective is the input of the UE antenna array. The gain “G” relates the combined signal from antenna elements corresponding to a given receiver branch to the reference point for requirement parameters.

The gain “G” affects absolute signal level values reported by the UE.

Figure B.2.1.5.1-1: Gain and Reference point for requirement parameters

The gain range for each power class is specified in Table B.2.1.5.1-1.

Table B.2.1.5.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| Minimum, dBi | 0 | FFS | -10 | FFS |
| Maximum, dBi | +50 | FFS | +20 | FFS |

Gain range in spherical coverage directions may be lower than in Rx beam peak direction, according to the difference between the EIS spherical coverage value specified in TS 38.101-2 [19] clause 7.3.4 and the Reference sensitivity level specified in TS 38.101-2 [19] clause 7.3.2.

So the UE gain variation can be expressed as (maximum UE gain - minimum UE gain)/2 according to 38.903 annex A, For UE PC3, it's 15dB. For UE PC1, it's 25dB

Spherical coverage gain variation is expressed as (UE REFSENS - UE Spherical coverage)/2 according to 38.903 annex A. where UE REFSENS is specifed in 38.101-2 Table 7.3.2.3-1. For UE PC3 with 100MHz CBW, spherical coverage gain variation is 6.4dB. For UE PC1 with 100MHz CBW, spherical coverage gain variation is 4dB

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB.

Denoting critical parameter y as a function of test equipment uncertainties x1, x2, ..., i.e. y=fx1,x2,⋯. Then sensitivity factors of xi on y can be expressed as:

sensitivity factors of xi on y=∂y∂xi

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE measurement accuracies where relevant. This process follows the superposition principle. Uncertainties are calculated separately for T1, T2 and T3.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE reporting accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE reporting accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

During T1

-	The condition Cell 1 SSB Ês/Iot at baseband ≥ -4dB is satisfied.

-	The condition Cell 1 SSB_RP ≥ -105.5 dBm/120kHz (-102.5dBm/240kHz) is satisfied.

During T2

-	Nominal value of Cell 1 Ês/Iot at baseband is -3.44dB which satisfies minimum requirement by 0.55dB. However, after considering the 0.62dB uncertainty Cell 1 Ês/Iot may be lower than -4dB. So, we purpose to raise Cell 1 Ês/Iot by 0.1dB.

-	The condition Cell 1 SSB_RP ≥ -105.5 dBm/120kHz (-102.5dBm/240kHz) is satisfied.

-	The condition Cell 2 SSB Ês/Iot at baseband ≥ -4dB is satisfied.

-	The condition Cell 2 SSB_RP ≥ -105.5 dBm/120kHz (-102.5dBm/240kHz) is satisfied.

-	Nominal value of (Cell 1 S-value - SintraSearchP) is -2.67dB. However, considering the 20.66dB uncertainty Cell 1 S-value may be higher than SintraSearchP. So, we purpose to raise QrxLevMin in Cell 1 SIB1 by 18dB.

-	Nominal value of (Cell 2 R-value – Cell 1 R-value) is 4.5dB which satisfies cell reselection condition. However, considering the 0.42dB uncertainty and the 0.1dB TT added to Cell 1 Ês/Iot the reselection condition may be no longer satisfied. So, we purpose to raise Cell 2 Ês/Iot by 0.55dB.

During T3

-	Nominal value of Cell 2 Ês/Iot at baseband is -3.44dB which satisfies minimum requirement by 0.55dB. However, after considering the 0.62dB uncertainty Cell 2 Ês/Iot may be lower than -4dB. So, we purpose to raise Cell 2 Ês/Iot by 0.1dB.

-	The condition Cell 2 SSB_RP ≥ -105.5 dBm/120kHz (-102.5dBm/240kHz) is satisfied.

-	The condition Cell 2 SSB Ês/Iot at baseband ≥ -4dB is satisfied.

-	The condition Cell 2 SSB_RP ≥ -105.5 dBm/120kHz (-102.5dBm/240kHz) is satisfied.

-	The condition Cell 2 SSB_RP ≥ -105.5 dBm/120kHz (-102.5dBm/240kHz) is satisfied.

-	Nominal value of (Cell 2 S-value - SintraSearchP) is -2.67dB. However, considering the 20.66dB uncertainty Cell 2 S-value may be higher than SintraSearchP. So, we purpose to raise QrxLevMin in Cell 2 SIB1 by 18 dB.

-	Nominal value of (Cell 1 R-value – Cell 2 R-value) is 4.5dB which satisfies cell reselection condition. However, considering the 0.42dB uncertainty and the 0.1dB TT added to Cell 2 Ês/Iot the reselection condition may be no longer satisfied. So, we purpose to raise Cell 1 Ês/Iot by 0.55dB.

f) Parameters modified by Test Tolerances

To meet measurement restriction rules Qrxlevmin in Cell1 SIB1 and Cell 2 SIB1 are increased by 18dB.

To satisify minimum SSB Ês/Iot reuirements, Cell 1 Ês/Iot in T2 and Cell 2 Ês/Iot in T3 are increased by 0.1dB.

To satisfiy intra-frequency cell reselection conditions, Cell 1 Ês/Iot in T3 and Cell 2 Ês/Iot in T2 are increased by 0.55dB.

g) Check controlled parameters Min/Max

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 7.1.1.1 also contains configurations with different subcarrier spacings. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a) and associated comment.

- Qrxlevmin, section a) and associated Source/Reference.

- NR Cell 1 and NR Cell 2 SSB_RP minimum requirement, section g) and associated Source/Reference

# 6. Treatment of other UE Power Classes

The initial analysis was done for UE Power class 3, as defined in TS 38.101-2, Table 6.2.1.0-1.

As the calculations use different values for UE Power class 1, separate spreadsheet tabs are provided. The following parameters are changed, shown as underlined cells on both tabs of the spreadsheet:

- UE Refsens (Rx Beam peak direction), section a)

- UE Spherical coverage (%ile directions), section a)

- UE rough/fine beam gain ∆ Rx Beam peak, Y, section a)

- UE rough/fine beam gain ∆ Spherical Coverage, Z, section a)

- UE gain G, Rx beam peak, section a)

- Cell 1 and Cell 2 SSB_RP minimum requirement, section g) and associated Source/Reference

The following parameters are offset for UE Power Class 1

During T2

-	Nominal value of Cell 1 Ês/Iot at baseband is -3.57dB which satisfies minimum requirement by 0.43dB. However, after considering the 0.76dB uncertainty Cell 1 Ês/Iot will be lower than -4dB. So, we purpose to raise Cell 1 Ês/Iot by 0.5dB.

-	The condition Cell 1 SSB_RP ≥ -104.3 dBm/120kHz (-98.3dBm/240kHz) is satisfied.

-	The condition Cell 2 SSB Ês/Iot at baseband ≥ -4dB is satisfied.

-	The condition Cell 2 SSB_RP ≥ -104.3 dBm/120kHz (-98.3dBm/240kHz) is satisfied.

-	Nominal value of (Cell 1 S-value - SintraSearchP) is 17.03dB. However, considering the 30.66dB uncertainty, Cell 1 S-value may be higher than SintraSearchP. So, we purpose to raise QrxLevMin in Cell 1 SIB1 by 37dB and SintraSearchP by 12dB

-	Nominal value of (Cell 2 R-value – Cell 1 R-value) is 4.5dB which satisfies cell reselection condition. However, considering the 0.42dB uncertainty and the 0.5dB TT added to Cell 1 Ês/Iot the reselection, condition will not be satisfied. So, we purpose to raise Cell 2 Ês/Iot by 1dB.

During T3

-	The condition Cell 1 SSB_RP ≥ -104.3 dBm/120kHz (-98.3dBm/240kHz) is satisfied.

-	The condition Cell 1 SSB Ês/Iot at baseband ≥ -4dB is satisfied.

-	Nominal value of Cell 2 Ês/Iot at baseband is -3.57dB which satisfies minimum requirement by 0.55dB. However, after considering the 0.76dB uncertainty Cell 2 Ês/Iot will be lower than -4dB. So, we purpose to raise Cell 2 Ês/Iot by 0.5dB.

-	The condition Cell 2 SSB_RP ≥ -104.3 dBm/120kHz (-98.3dBm/240kHz)) is satisfied.

-	Nominal value of (Cell 2 S-value - SintraSearchP) is 17.03dB. However, considering the 30.66dB uncertainty, Cell 2 S-value may be higher than SintraSearchP. So, we purpose to raise QrxLevMin in Cell 2 SIB1 by 37 dB and SintraSearchP by 12dB.

-	Nominal value of (Cell 1 R-value – Cell 2 R-value) is 4.5dB which satisfies cell reselection condition. However, considering the 0.42dB uncertainty and the 0.5dB TT added to Cell 2 Ês/Iot the reselection, condition may be no longer satisfied. So, we purpose to raise Cell 1 Ês/Iot by 1dB.

f) Parameters modified by Test Tolerances

To meet measurement restriction rules S-intraSearchP in Cell1 SIB1 and Cell 2 SIB1 are increased by 12dB.

To meet measurement restriction rules Qrxlevmin in Cell1 SIB1 and Cell 2 SIB1 are increased by 37dB.

To satisify minimum SSB Ês/Iot reuirements, Cell 1 Ês/Iot in T2 and Cell 2 Ês/Iot in T3 are increased by 0.5dB.

To satisfiy intra-frequency cell reselection conditions, Cell 1 Ês/Iot in T3 and Cell 2 Ês/Iot in T2 are increased by 1dB.