3GPP TSG-RAN WG5 Meeting # 105	R5-247401

# Orlando, US,  November 18 – 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0971 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating A-MPR testing for NS_46 for band n7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_FR1_FDD_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no TP analysis for PC2 A-MPR requirements for NS_46. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the TP analysis for NS_46 to support PC2 UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TP analysis for NS_46 will not support PC2 UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 CR 3140 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Remove zip file “38.521-1_TPanalysis_6.2.3_AMPR_NS_46_v1.zip” Add zip file “38.521-1_TPanalysis_6.2.3_AMPR_NS_46_v2.zip” |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

4.1.2.1	A-MPR, A-SEM and A-SE FR1 test cases for single carrier and UL MIMO

This section contains information on test point selection for single carrier test cases 6.2.3, Additional Maximum Power Reduction (A-MPR), 6.5.2.3 Additional spectrum emission mask (A-SEM) and 6.5.3.3 Additional spurious emissions (A-SE); and for correspondent UL-MIMO test cases in 6.2D.3 and 6.5D.3.3 in [2].

Selection of test points should include some possible worst combinations based on the A-MPR characteristics specified for each NS value and these shall be selected so that they match with corresponding spectrum emission requirements test points. The number of test points should be realistic.

For pi/2 BPSK with Rel-16 DMRS, the correspondent A-MPR requirements are the same as that for Rel-15 DMRS but the PAPR is lower. Given the UE can pass the requirements using Rel-15 DMRS, it can be expected that the UE can pass the requirements using Rel-16 DMRS. Therefore there is no need to additionally test A-MPR, A-SEM and A-SE for Rel-16 DMRS.

Table 4.1.2.1-1 lists number of test points for A-MPR, A-SEM and A-SE single carrier test cases and for different NS values.

Table 4.1.2.1-1: NS value specific test points for A-MPR, A-SEM and A-SE single carrier

| NS label | Number of test points for A-MPR | Number of test points A-SEM | Number of test points A-SE | Justification | Comments |
| --- | --- | --- | --- | --- | --- |
| NS_03 NS_03U | 144 for Non-SUL testing 72 for SUL testing |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_03.zip” | RAN5#85 |
| NS_04 | 6.2.3: 120 for PC3, PC2 and PC1.5 6.2.3: 149 for PC1 6.2D.3: 112 6.5D.3.3:112 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_04_v4.zip” | RAN5#104 |
| NS_05 | 6.5.3.3: 432 |  |  | “38.521-1_TP analysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_05.zip” | RAN5#87-e |
| NS_05, NS_05U | 6.2.3: 288 for PC3 6.2.3: 396 for PC2 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_05_v3.zip” | RAN5#96-e |
| NS_06 | 6.2.3: PC1 160 for n14, 144 for n85PC3 144 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_06_v3.zip” | RAN5#104 |
| NS_07 | 162 | 162 | 162 | “38.521-1_TPanalysis_6.2.3_AMPR_NS_07.zip” | RAN5#99 |
| NS_12 | 48 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15.zip” | RAN5#90-e |
| NS_13 | 21 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15.zip” | RAN5#90-e |
| NS_14 | 6.2.3: 50 6.5.3.3: 25 |  | 25 | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15.zip” “38.521-1_TPanalysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_14.zip | RAN5#90-e RAN5#96-e |
| NS_15 | 102 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_12_13_14_15.zip” | RAN5#90-e |
| NS_17 | 6.5.3.3: 4 |  |  | “38.521-1_TPanalysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_17_v3.zip” | RAN5#104 |
| NS_18 | 88 6.2.3: 108 6.5.3.3: 54 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_18_v3.zip” | RAN5#89-e |
| NS_21 | 180 | 180 | 180 | “38.521-1_TPanalysis_6.2.3_AMPR_NS_21_v1.zip” | RAN5#98 |
| NS_24 | 6.2.3: 300 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_24.zip” | RAN5#87 |
| NS_27 | 6.2.3: 360 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_27_v5.zip” | RAN5#98 |
| NS_35 | 6.2.3: 144 6.2.3: 72 for UL-MIMO |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_35_v3.zip” | RAN5#104 |
| NS_37 | 6.2.3: 48 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_37.zip” | RAN5#86 |
| NS_38 | 6.2.3: 96 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_38.zip” | RAN5#86 |
| NS_39 | 6.2.3: 54 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_39.zip” | RAN5#86 |
| NS_40 | 6.2.3: 24 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_40.zip” | RAN5#87 |
| NS_41 | 6.2.3: 72 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_41.zip” | RAN5#87 |
| NS_42 | 6.2.3: 108 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_42.zip” | RAN5#87 |
| NS_43 | 6.2.3: PC3 28; PC2 38 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_43_v3.zip” | RAN5#103 |
|  | 6.5.3.3: 81 |  |  | “38.521-1_TP analysis_6.5.3.3_TX_Additional_Spurious_Emission_NS_43.zip” | RAN5#87-e |
| NS_43U | 6.2.3: 72 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_43U.zip” | RAN5#85 |
| NS_44 | 360 | N/A | 180 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_44.zip” | RAN5#90-e |
| NS_45 | 24 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_45.zip” | RAN5#89-e |
| NS_46 | 88 for PC3 144 for PC1 64 for PC2 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_46_v2.zip” | RAN5#105 |
| NS_47 | 70 for PC3 63 for PC272 for PC1.5 |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_47_v1.zip” | RAN5#102 |
| NS_48 | 240 for PC3 384 for PC2 | N/A | 96 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_48_v3.zip” | RAN5#96-e |
| NS_49 | 280 for PC3 392 for PC2 | N/A | 140 for PC3 191 for PC2 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_49_v3.zip” | RAN5#104 |
| NS_50 | 120 for PC3 288 for PC2 336 for PC1.5 |  | 120 for PC3 288 for PC2 336 for PC1.5 | “38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_50_v1.zip” | RAN5#103 |
| NS_56 | 175 |  | 175 | 38.521-1_TPanalysis_6.2.3_AMPR_6.5.3.3_ASE_NS_56.zip | RAN5#93-e |
| NS_100 | 144 for Non-SUL testing 72 for SUL testing |  |  | “38.521-1_TPanalysis_6.2.3_AMPR_NS_100.zip” | RAN5#85 |

==============================================================

<< End of changes >>

Remove zip file “38.521-1_TPanalysis_6.2.3_AMPR_NS_46_v1.zip”

Add zip file “38.521-1_TPanalysis_6.2.3_AMPR_NS_46_v2.zip”