3GPP TSG-RAN5 Meeting #105	R5-247474

# Orlando, USA, November 18 - 22 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3645 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Annex E for NR mobility enhancements test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR mobility enhancement test cases 6.3.5.1, 7.3.5.1 and 7.6.22.1 are missing cell configuration mapping |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cell configuration mapping has been added to tables E.4-1 and E.4-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases will remain incomplete and WP cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

# E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.8 | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.9 | NR SA FR1 cell re-selection for UE operating on a cell with less than 5MHz BW | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.2.1 | NR SA FR1 TA Validation for CG-SDT | NR Cell 1 |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.3 | NR SA FR1-FR1 Handover with unknown Target Cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.1.18 | NR SA FR1 handover with unknown target cell operating with 12 PRB SSB bandwidth | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | NR Cell 1 |  |  |  |  |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | NR Cell 1 |  |  |  |  |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | NR Cell 1 |  |  |  |  |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | NR Cell 1 |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.3.2.4.1 | PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.2.4.2 | PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.2.4.3 | PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 6.3.4.1 | RACH-based Intra-frequency PCell switch from FR1 to FR1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.4.2 | RACH based Inter-frequency LTM PCell switch from FR1 to FR1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.4.3 | RACH-less Intra-frequency PCell switch from FR1 to FR1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.5.1 | NR SA FR1 RACH-based intra-frequency LTM PSCell switch from FR1 to FR1 | NR Cell 1 | NR Cell 6 | NR Cell 2 |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.9 | SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion | NR Cell 1 |  |  |  |  |
| 6.5.1.13 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW | NR Cell 1 |  |  |  |  |
| 6.5.1.14 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW | NR Cell 1 |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.2.2 | SA FR1 interruptions at NR SRS carrier based switching | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.4 | NR SA FR1 direct SCell activation at SCell addition of known SCell | NR Cell 1 | NR Cell 6 |  |  | intra-band |
| 6.5.3.5 | NR SA FR1 direct SCell activation at handover with known SCell | NR Cell 1 | NR Cell 6 | NR Cell 2 |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.10 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.11 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.3.15 | NR SA FR1 TRS based SCell Activation of SSB-less SCell in inter-band CA in non-DRX | NR Cell 1 | NR Cell 10 |  |  | Inter-band |
| 6.5.3.16 | NR SA FR1 Inter-band SSB-less SCell Activation based on A-TRS in non-DRX | NR Cell 1 | NR Cell 10 |  |  | Inter-band |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 33 |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 6.5.5.5 | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.5.5.6 | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.5.5.7 | NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | NR Cell 1 |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |
| 6.5.6.3.1 | NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA | NR Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 6.5.6.5.1 | RRC based BWP switch on multiple CCs | NR Cell 1 | NR Cell 6 |  |  |  |
| 6.5.7.1 | NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  | inter-band |
| 6.5.7.2 | NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  | inter-band |
| 6.5.7A.1 | NR SA FR1 DL interruptions at switching between two uplink carriers in FDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  | inter-band |
| 6.5.7A.2 | NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA | NR Cell 1 | NR Cell 10 |  |  | inter-band |
| 6.5.7B.1 | NR SA FR1 DL Interruptions at switching between two uplink bands in FDD-TDD CA | NR Cell 1 | NR Cell 10 | NR Cell 31 |  | inter-band |
| 6.5.7B.2 | NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA | NR Cell 1 | NR Cell 10 | NR Cell 31 |  | inter-band |
| 6.5.7C.1 | DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA | NR Cell 1 | 10NR Cell | NR Cell 31 |  | Inter-band |
| 6.5.7C.2 | DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA | NR Cell 1 | NR Cell 10 | NR Cell 31 |  | Inter-band |
| 6.5.7D.1 | NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG | NR Cell 1 | NR Cell 10 | NR Cell 500 |  | Inter-band |
| 6.5.7D.2 | NR SA FR1 DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG | NR Cell 1 | NR Cell 10 | NR Cell 28 | NR Cell 500 | Inter-band |
| 6.5.7D.3 | NR SA FR1 DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs | NR Cell 1 | NR Cell 10 | NR Cell 500 |  | Inter-band |
| 6.5.8.1 | UE specific CBW change on PCell in FR1 in non-DRX | NR Cell 1 |  |  |  |  |
| 6.6.1.1 | SA event triggered reporting tests without gap under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.1.2 | SA event triggered reporting tests without gap under DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.1.3 | SA event triggered reporting tests with per-UE gaps under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.1.4 | SA event triggered reporting tests with per-UE gaps under DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.1.5 | SA event triggered reporting tests without gap under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.1.6 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.1.8 | NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 | NR Cell 1 | NR Cell 6 | NR Cell 13 |  | intra-band |
|  |  |  | NR Cell 10 | NR Cell 30 |  | inter-band |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.2.9 | NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.2.10 | NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.2.11 | NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.2.12 | NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 6.6.4.5 | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 |  |  |  |  |
| 6.6.4.6 | NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.6.1 | NR SA FR1 SRS-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 6.6.8.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | NR Cell 1 |  |  |  |  |
| 6.6.8.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 6.6.8.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 6.7.8.1 | NR SA FR1 SRS-RSRP measurement accuracy | NR Cell 1 |  |  |  |  |
| 6.6.9.1 | NR SA FR1 Idle mode CA/DC measurement for FR1 | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.6.17.1 | NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.17.2 | NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.18.1 | NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers | NR Cell 6 | NR Cell 3 | NR Cell 1 |  |  |
| 6.6.18.2 | NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers | NR Cell 6 | NR Cell 3 | NR Cell 1 |  |  |
| 6.6.18.4 | NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used | NR Cell 6 | NR Cell 3 | NR Cell 13 |  |  |
| 6.6.19.1 | NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1 | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.19.2 | NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.19.4 | NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX | NR Cell 1 | NR Cell 6 | NR Cell 13 |  | intra-band |
|  |  |  | NR Cell 10 | NR Cell 30 |  | inter-band |
| 6.6.20.1 | UE Rx-Tx time difference measurement with PRS for RTT-based PDC | NR Cell 1 |  |  |  |  |
| 6.6.21.1 | UE Rx-Tx time difference measurement with TRS for RTT-based PDC | NR Cell 1 |  |  |  |  |
| 6.6.26.1 | Intra-frequency SSB based L1-RSRP measurement | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.6.27.1 | Inter-frequency SSB based L1-RSRP measurement with measurement gap | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.6.28.1 | Inter-frequency SSB based L1-RSRP measurement without measurement gap | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.7.1.3.2 | NR SA FR1-FR2 SS-RSRP relative measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |
| 6.7.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |
| 6.7.4.2.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |
| 6.7.4.2.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |
| 6.7.9.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 6.7.9.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | NR Cell 1 |  |  |  |  |
| 6.7.9.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 6.7.10.1.1 | NR SA FR1 CSI-RSRP absolute measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 6.7.10.1.2 | NR SA FR1 CSI-RSRP relative measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 6.7.10.2.1 | NR SA FR1-FR1 CSI-RSRP absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.10.2.2 | NR SA FR1-FR1 CSI-RSRP relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.17.1.1 | Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.7.17.1.2 | Inter-frequency L1-RSRP relative accuracy requirements for neighbour cell | NR Cell 6 | NR Cell 3 |  |  |  |

Table E.4-2 defines the cell configuration mapping for NR/5GC FR1 – E-UTRA Inter-RAT test cases (serving cell in NR) in chapter 6 of this test specification.

Table E.4-2: Cell configuration mapping for SA FR1 – E-UTRA Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 LTE Cell2 | 38.533 LTE Cell3 | 38.533 LTE Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.2 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.3 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.4 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.5 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | LTE Cell 11 |  |  |  |
| 6.3.1.4 | NR SA FR1 – E-UTRA handover with known target cell | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.3.1.5 | NR SA FR1 – E-UTRA handover with unknown target cell | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.3.2.3.2 | NR SA FR1 – E-UTRA RRC connection release with redirection | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.1 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.2 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.3 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.15.1 | NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.19.3 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |

Table E.4-3: Cell configuration mapping for SA FR1 – UTRA FDD Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 UTRA Cell2 | 38.533 UTRA Cell3 | 38.533 UTRA Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.3.1.6 | NR SA FR1 – UTRA cell re-selection to higher priority UTRA | NR Cell 1 | UTRA Cell 1 |  |  |  |
| 6.6.5.1 | NR SA FR1 – UTRAN event-triggered reporting in non-DRX | NR Cell 1 | UTRA Cell 1 |  |  |  |

Table E.4-4 defines the multi-cell configuration mapping for NR/5GC FR1 – E-UTRA Inter-RAT test cases (serving cell in NR) in chapter 6 of this test specification.

Table E.4-4: Multi-cell configuration mapping for SA FR1 – E-UTRA Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 LTE Cell3 | 38.533 LTE Cell 4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.6.18.3 | NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1 | NR Cell 6 | NR Cell 3 | LTE Cell 1 |  |  |

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 | NR SA FR2 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | NR Cell 1 | NR Cell 23 |  |  |  |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.6 | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.7 | NR SA FR2 cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.8 | NR SA FR2 inter-frequency cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | NR Cell 1 | NR Cell 23 |  |  |  |
| 7.2.1.1 | TA validation for CG-SDT in FR2 | NR Cell 1 |  |  |  |  |
| 7.3.1.1 | NR SA FR1-FR2 Inter-frequency handover; unknown target cell | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.1.2 | NR SA FR2 Intra-frequency handover; unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.1.3 | NR SA FR2-FR2 Inter-frequency handover; unknown target cell | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.2 | NR SA FR2-FR2 re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.2.4 | NR SA FR2 2-step non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.3.1 | NR SA FR2-FR2 RRC connection release with redirection | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.3.5.1 | NR SA FR2 RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2 | NR Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 7.4.1.1 | NR SA FR2 UE transmit timing accuracy | NR Cell 1 |  |  |  |  |
| 7.4.1.3 | NR SA FR2 UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | NR Cell 1 |  |  |  |  |
| 7.4.3.1 | NR SA FR2 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |
| 7.5.1.1 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.2 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.3 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.4 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 7.5.1.9 | NR SA FR2 radio link monitoring UE scheduling restrictions | NR Cell 1 |  |  |  |  |
| 7.5.3.3 | SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.5.3.4 | NR SA FR2 direct SCell activation at SCell addition of known SCell | NR Cell 1 | NR Cell 6 |  |  |  |  |
| 7.5.3.5 | NR SA FR2 direct SCell activation at handover with known SCell | NR Cell 1 | NR Cell 6 | NR Cell 2 |  |  |  |
| 7.5.3.13 | NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX | NR Cell 1 | NR Cell 6 |  |  | intra-band |
| 7.5.3.14 | NR SA FR2 SCell Activation for known SCell in FR2 inter-band | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.5.5.1 | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.2 | NR SA FR2 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.3 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.4 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.6 | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.5.7 | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.5.8 | FFS |  |  |  |  |  |
| 7.5.5.9 | NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.5.10 | NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.6.1 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.8.1.1 | NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state | NR Cell 1 |  |  |  |  |  |
| 7.5.8.2.1 | NR SA PCell FR2 RRC based active TCI state switch for a known TCI state | NR Cell 1 |  |  |  |  |  |
| 7.5.8.3.1 | NR SA FR2 HST active TCI state switch for a known TCI state | NR Cell 1 |  |  |  |  |  |
| 7.5.8.3.2 | NR SA FR2 HST active TCI state switch for PC6 UE supporting tci-StateSwitchInd-r18 a known TCI state | NR Cell 1 |  |  |  |  |  |
| 7.5.9.1.1 | NR PCell FR2 uplink spatial relation switch associated with known DL-RS (MAC-CE based) | NR Cell 1 |  |  |  |  |  |
| 7.5.9.2.1 | NR PCell FR2 uplink spatial relation switch associated with known DL-RS (RRC based) | NR Cell 1 |  |  |  |  |  |
| 7.5.11.1 | NR SA FR2 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.5.12.1 | Addition and Release Delay of PSCell | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.5.13.1 | NR SA FR2 MAC-CE based active joint TCI state switch | NR Cell 1 |  |  |  |  |
| 7.5.13.2 | NR SA FR2 MAC-CE based active uplink TCI state switch | NR Cell 1 |  |  |  |  |
| 7.5.13.3 | NR SA FR2 MAC-CE based active downlink TCI state switch | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.13.6.1 | NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states | NR Cell 1 |  |  |  |  |
| 7.5.14 | NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.5 | NR SA FR2 event triggered reporting test without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17 | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.2.1 | NR SA FR2-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.2 | NR SA FR2-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.3 | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.4 | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.10 | SA FR2-FR2 event triggered reporting test without gap under non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.11 | SA FR2-FR2 event triggered reporting test without gap under DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.5 | NR SA FR2 SSB based L1-RSRP measurement test when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | NR Cell 1 |  |  |  |  |
| 7.6.3.6 | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used | NR Cell 10 | NR Cell 1 |  |  | Inter-band |
| 7.6.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.6.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.6.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.6.13.1 | UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2 | NR Cell 1 |  |  |  |  |  |
| 7.6.13.2 | NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS | NR Cell 1 |  |  |  |  |  |
| 7.6.14.1 | NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.6.14.2 | NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.6.15.1 | NR SA FR2 event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements | NR Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.15.2 | NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2) | NR Cell 1 | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.15.3 | NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement | NR Cell 1 | NR Cell 6 | NR Cell 2 |  |  |  |
| 7.6.16.1 | NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 7.6.16.2 | NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used(PCell in FR2) | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 7.6.16.3 | NR SA FR2 event triggered reporting test on deactivated SCell measurement via NCSG in non-DRX | NR Cell 1 | NR Cell 6 | NR Cell 13 |  | Intra-band |  |
| 7.6.22.1 | NR SA FR2 Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2 | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 7.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.7.1.3.2 | Void |  |  |  |  |  |
| 7.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.3.1 | NR SA FR2 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.7.3.2 | NR SA FR2-FR2 SS-SINR measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.7.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.7.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.7.7.1.1 | NR SA FR2 CSI-RSRP measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 7.7.7.2.1 | NR SA FR2-FR2 CSI-RSRP measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |

/// End of changes