3GPP TSG-RAN5 Meeting #105	R5-246250r2

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4666 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new R18 MUSIM test case for UE Assistance Information / MUSIM / musim-Cell-SCG-ToRelease / musim-cellToAffectList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DualTxRx_MUSIM-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adddition of a new R18 MUSIM test case for testing UE Assistance Information / MUSIM / musim-Cell-SCG-ToRelease / musim-cellToAffectList has been endorsed within the work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new R18 MUSIM test case for UE Assistance Information / MUSIM / musim-Cell-SCG-ToRelease / musim-cellToAffectList |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R18 MUSIM work plan will not be fully covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0531 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Adding RRCReconfigurationcomplete step and waiting time at step 12. r2: Remove musim-Cell-SCG-ToRelease which is not match the test scenario. |  |  |  |  |  |  |  |  |

8.1.5.10.5	UE Assistance Information / MUSIM / musim-Cell-SCG-ToRelease / musim-cellToAffectList

8.1.5.10.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE is configured to provide MUSIM assistance information for temporary capability restriction and it has temporary capability restriction on the current configuration with timer T348 is not running }

then { UE transmits a UEAssistanceInformation message including musim-CellToAffectList and starts timer T348 with the timer value set to the musim-WaitTimer }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE is configured to provide MUSIM assistance information for temporary capability restriction and it has temporary capability restriction on the current configuration with timer T348 is running }

then { UE Does NOT transmit a UEAssistanceInformation message including musim-CellToAffectList }

}

8.1.5.10.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.7.4.2, and TS 38.300, clause 20.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.7.4.2]

…

A UE capable of providing MUSIM assistance information for temporary capability restriction may initiate the procedure if it was configured to do so, upon determining it has temporary capability restriction or upon determining the removal of the capability restriction.

…

Upon initiating the procedure, the UE shall:

…

1>	if configured to provide MUSIM assistance information for temporary capability restriction:

2>	if the UE has temporary capability restriction on the current configuration and timer T348 is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-Cell-SCG-ToRelease and/or musim-CellToAffectList;

3>	start the timer T348 with the timer value set to the musim-WaitTimer.

[TS 38.300, clause 20.4]

For MUSIM operation, a MUSIM device in RRC_CONNECTED state in Network A may indicate its preference on temporary UE capability restriction or removal of restriction with Network A when the MUSIM device needs transmission or reception in Network B (e.g., including start/stop connection to Network B). Network A is NR and Network B can either be E-UTRA or NR. The MUSIM device may request a temporary capability restriction only after the Network signals via RRC that this is allowed.

When configured to do so, a MUSIM device can indicate one or more of the following temporary capability restriction or removal of restriction to the Network A:

-	A MUSIM device can explicitly request SCell(s) or SCG to be released;

-	A MUSIM device can indicate its preference on temporary maximum MIMO layers and/or supported channel bandwidth for specific serving cells for both UL/DL;

-	A MUSIM device can indicate its preference on the temporary maximum number of CCs per UL/DL;

-	A MUSIM device can indicate its preference on the concerned band(s) or band combination(s) (e.g. forbidden and/or affected band(s) or band combination(s)) based on a band-filter list configured by network. For affected band(s) and band combination(s), this preference can include temporary maximum MIMO layers and/or supported channel bandwidth for both UL/DL;

-	A MUSIM device can indicate the measurement gap requirement changes.

When it is allowed by Network A in SIB1, a MUSIM device can indicate to the Network A that its capabilities are temporarily restricted in RRCSetupComplete/RRCResumeComplete message while the MUSIM device is already in RRC_CONNECTED state in Network B.

When a MUSIM device is in RRC_CONNECTED state in both Network A and Network B, if Network B is NR, it is up to UE implementation to select which network to request temporary UE capability restriction; if Network B is E-UTRA, the request for temporary UE capability restriction can only be performed on Network A.

…

8.1.5.10.5.3	Test description

8.1.5.10.5.3.1	Pre-test conditions

System Simulator:

-   NR Cell 1(home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [18] and belong to the different frequency.

-  NR Cell 1 (belongs to TAI-1, home PLMN1, MCC=001,MNC=01) is set to ''Serving cell''.

-  NR Cell 12 (belongs to TAI-3, home PLMN2, MCC=001,MNC=02) is set to ''Serving cell ''.

UE:

- 	The UE is a Rel-18 MUSIM UE;

-    The UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] Tables 6.4.1-27 and 6.4.1-28.

Preamble:

-    The UE is in 5GS state 3N-A on NR Cell 1 and in 5GS state 1N-A on NR Cell 12 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Table 4.4A.2-2.

-    The SIB1 of NR Cell 12 and NR Cell 1 are defined in TS 38.508-1 [4], clause 4.6.1-28 with condition MUSIM_r18 for all steps.

8.1.5.10.5.3.2           Test procedure sequence

Table 8.1.5.10.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on NR Cell 12 unless explicitly stated otherwise. | - | - | - | - |
| 1-6 | The UE performs the generic procedure by executing steps 1 to 6 to enter the NR RRC_CONNECTED state as described in TS 38.508-1 [4] table 4.5.4.2-3 on NR Cell 12 | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB(s). | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | The UE transmits an UEAssistanceInformation message. | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 10 | The SS transmits an RRCReconfiguration message to trigger UE transmiting UEAssistanceInformation. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 12 | Check: Does UE transmit UEAssistanceInformation message in 1s? | --> | NR RRC: UEAssistanceInformation | 2 | F |
| NOTE: If UE support Rel-18 MUSIM feature and the UE is in connected mode on NR Cell 1, and when the UE start data transmission on NR Cell 12, UE determines itself to be in temporary capability restriction. |  |  |  |  |  |

8.1.5.10.5.3.3 Specific message contents

Table 8.1.5.10.5.3.3-1: RRCConnectionReconfiguration (step 7 and 10, Table 8.1.5.10.5.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |
| musim-CapabilityRestrictionConfig-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-WaitTimer-r18 | ms100 |  |  |
| musim-ProhibitTimer-r18 | s10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.5.3.3-2: UEAssistanceInformation (step 9, Table 8.1.5.10.5.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-Assistance-v1800 SEQUENCE { |  |  |  |
| musim-CapRestriction-r18 SEQUENCE { |  |  |  |
| musim-Cell-SCG-ToRelease-r18 | Not checked |  |  |
| musim-CellToAffectList-r18 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |