3GPP TSG-RAN5 Meeting #105	R5-246425r1

# Orlando, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0344 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NG.114 versions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An updated version of NG.114 is missing. See also related discussion paper R5-246424. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A table item for NG.114 without any specific version has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Updated NG.114 information will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, A.4.13, A.4.1-2, A.4.5, A.4.7-8. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision R5-246425r1. Wording for NG.114 item updated. Scope updated. References updated to [99] at several places. |  |  |  |  |  |  |  |  |

<Start of first modified section>

[96]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS, Version 1.0".

[97]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[98]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS, Version 2.0".

[99]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

<End of modified section>

<Start of next modified section>

# 1	Scope

The present document provides the Implementation Conformance Statement (ICS) proforma for 3rd Generation User Equipment (UE) supporting the Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP), in compliance with the relevant requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-7 [8] and ETS 300 406 [9].

The present document also specifies a recommended applicability statement for the test cases included in TS 34.229-1 [5] and TS 34.229-5 [95]. These applicability statements are based on the features implemented in the UE.

The present document is valid for UE implemented according to 3GPP releases starting from Release 5 up to the Release indicated on the cover page of the present document.

Also, it is generally assumed that an IMS capable UE is compliant to at least one of  the following GSMA PRD IR.92 [83], GSMA PRD IR.94 [75] or GSMA PRD NG.114 [99];any update of requirements in these GSMA PRD documents, which are relevant to the present document will be handled on a case by case basis, with due consideration given for grace period and/or conditions to be granted for the UE to comply to any updated requirements.

<End of modified section>

<Start of next modified section>

### A.4.13	NG.114 versions

Table A.21: NG.114 versions

| Item |  | Ref. | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Version 1.0 as of 07 August 2020 | NG.114 v1.0 [96] | o | n/a | pc_NG114_v1_0 |  |
| 2 | Version 2.0 as of 07 August 2021 | NG.114 v2.0 [98] | o | n/a | pc_NG114_v2_0 |  |
| 3 | GSMA PRD NG.114 | NG.114 [99] | o | n/a | pc_NG114 |  |

<End of modified section>

<Start of next modified section>

### A.4.1	Roles

Table A.2: Roles

| Item | UE roles | Ref. | Status | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | User agent | 24.229 [10], A.1.3 RFC 3261 [15] | m | Rel-5 |  |

Table A.3A: UE roles specific to additional capabilities

| Item | UE roles | Ref. | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 50 | Multimedia telephony service participant | 24.173 [55] | o | Rel-8 | pc_MultimediaTelephonyService |  |
|  |  | IR.92 [83], 2.2.1 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 61 | SM-over-IP sender | 24.341 [66] | o | Rel-8 | pc_SMS_IP_MO |  |
|  |  | IR.92 [83], 2.5 | m |  |  |  |
|  |  | NG.114 [99] | m NOTE 1 | Rel-15 |  |  |
| 62 | SM-over-IP receiver | 24.341 [66] | o | Rel-8 | pc_SMS_IP_MT |  |
|  |  | IR.92 [83], 2.5 | m |  |  |  |
|  |  | NG.114 [99] | m NOTE 1 | Rel-15 |  |  |
| 63 | Void |  |  |  |  |  |
| 64 | Concatenated SM-over-IP sender | 23.040 [94], 9.2.3.24.1 | o | Rel-8 | pc_Concatenated_SMS_IP_MO |  |
| 65 | Concatenated SM-over-IP receiver | 23.040 [94], 9.2.3.24.1 | o | Rel-8 | pc_Concatenated_SMS_IP_MT |  |
| NOTE 1: As an exception to NG.114 support of SM-over-IP sender/Receiver is optional for RedCap devices with form factor limitation making it impossible to support these services. |  |  |  |  |  |  |

### A.4.2	ICS related to SIP

#### A.4.2.1	Major capabilities

Prerequisite: A.2/1 - - user agent role.

Table A.4: Major capabilities

| Item | Does the implementation support | Ref. | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | Capabilities within main protocol |  |  |  |  |  |
| 2B | initiating a session? | 24.229 [10], A.2.1.2; RFC 3261 [15], 13 | o | Rel-5 | pc_InitiateSession |  |
| 2C | initiating a session which require local and/or remote resource reservation? | 24.229 [10], A.2.1.2; RFC 3262 [27] UE is capable of obtaining location information | c43 | Rel-8 |  |  |
|  | Extensions |  |  |  |  |  |
| 16 | integration of resource management and SIP? (use of preconditions) | 24.229 [10], A.2.1.2; RFC 3312 [26] | C44 | Rel-8 | pc_Preconditions |  |
|  |  | IR.92 [83], 2.4.1 | m |  |  |  |
|  |  | NG.114 [99], 2.3.5 | m | Rel-15 |  |  |
| 53 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) | 24.229 [10], A.2.1.2 | m (Note 1) | Rel-7 | pc_IMS_GRUUsInSIP |  |
|  | Conditions/Options |  |  |  |  |  |
| c43: IF A.4/2B THEN o ELSE n/a - - initiating sessions. c44: IF A.4/2C THEN m ELSE o - - initiating a session which require local and/or remote resource reservation. |  |  |  |  |  |  |
| NOTE 1: If a UE is unable to become engaged in a service that potentially requires the ability to identify and interact with a specific UE even when multiple UEs share the same single Public User Identity then the UE support can be " o" instead of " m". Examples include telemetry applications, where point-to-point communication is desired between two users. |  |  |  |  |  |  |

### A.4.3	Void

### A.4.4	Void

### A.4.5	Additional information

Table A.12: Additional information

| Item | Additional information | Reference | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Void |  |  |  |  |  |
| 2 | Void |  |  |  |  |  |
| 3 | Void |  |  |  |  |  |
| 4 | UE capable of being configured to initiate Dedicated PDP Context | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiatesDedicatedPDPContext |  |
| 5 | UE capable of being configured to initiate P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaPCO |  |
| 6 | Void |  |  |  |  |  |
| 7 | UE capable of being configured to initiate P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaDHCPv6 |  |
| 8 | UE supports P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaPCO |  |
| 9 | Void |  |  |  |  |  |
| 10 | UE supports P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaDHCPv6 |  |
| 11 | Void |  |  |  |  |  |
| 12 | UE capable of initiating a bidirectional voice session over IMS | 24.229 [10], 5.1.6 | o | Rel-5 | pc_BidirecVoiceOverIMS |  |
| 13 | Void |  |  |  |  |  |
| 14 | Void |  |  |  |  |  |
| 15 | Void |  |  |  |  |  |
| 16 | UE Supports " IPv6 address with embedded IPv4 address" in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 17 | UE Supports IPv4 address in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 18 | Void |  |  |  |  |  |
| 19 | UE supports UI capable of showing user notification for Message Waiting Indication | 24.173 [55], Annex F | o | Rel-7 |  |  |
| 20 | Void |  |  |  |  |  |
| 21 | Void |  |  |  |  |  |
| 22 | Void |  |  |  |  |  |
| 23 | Void |  |  |  |  |  |
| 24 | UE supports no reply timer setting | 24.604 [68], 4.9.1.4 | o | Rel-8 |  |  |
|  |  | IR.92 [83], 2.3.8 | m |  |  |  |
| 25 | UE supports sending DTMF events over RTP | 26.114 [56], Annex G | o | Rel-7 |  |  |
|  |  | IR.92 [83], 3.3 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 26 | Void |  |  |  |  |  |
| 27 | UE is capable of obtain ing location information | 24.229 [10] 4.7, 36.509 [79] 4.1, 5.5.2, 34.109 [78] 5.4.2 | o | Rel-9 | pc_IMS_Geolocation |  |
| 28 | Void |  |  |  |  |  |
| 29 | UE supports emergency speech call over 1xRTT | C.S0005-E [74] | o |  | pc_CS_Em_Call_in_1xRTT |  |
| 30 | Void |  |  |  |  |  |
| 31 | UE supports end-to-access-edge media security using SDES | 24.229 [10], 4.2.B.2 | o | Rel-9 |  |  |
| 32 | UE supports video media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | o | Rel-13 | pc_IMS_Video_FeatureTag |  |
|  |  | IR.94 [75], 2.2 | m |  |  |  |
|  |  | NG.114 [99] | m NOTE 1 | Rel-15 |  |  |
| 33 | UE supports Communication Hold during emergency call | 23.167 [76], 7.1.1 | o | Rel-9 | pc_CommunicationHold_DuringEmergencyCall |  |
| 34 | UE indicates g.3gpp.srvcc-alerting media feature tag in INVITE request or 180 (Ringing) response | 24.237 [77], 12.2.3B.1 | o | Rel-10 | pc_IMS_SRVCCAlert |  |
| 35 | Void |  |  |  |  |  |
| 36 | UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request | 24.237 [77] 6A.2.2.2 | o | Rel-12 | pc_BSRVCC |  |
| 37 | UE indicates OMA-TS-XDM_MO-V1_1-20080627-A.doc, section 5.2.8 "Node: /<X>/ AAUTHNAME" is configured | 24.623 [81], Annex B.2 | o | Rel-8 | pc_XCAP_UsernameIsConfiguredInUE |  |
| 38 | The UE use the default public user identity received in P-Associated -URI header in 200 OK for REGISTER as XCAP User Identity (XUI) | IR.92 [83] | m |  | pc_XCAP_XUIisDefaultPublicUserId |  |
|  |  | 24.623 [81] | o | Rel-8 |  |  |
| 39 | UE has the method that support IMS deregistration | 24.229 [10], 5.1.1.6 | o | Rel-5 | pc_IMS_Deregistration |  |
| 40 | UE supports Cs to PS SRVCC | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCC |  |
| 41 | UE supports Cs to PS SRVCC in alerting state | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCAlert |  |
| 42 | UE supports Cs to PS SRVCC and the MSC server assisted mid-call feature | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCMidCall |  |
| 43 | Void |  |  |  |  |  |
| 44 | Void |  |  |  |  |  |
| 45 | UE supports early media | 24.229 [10], 5.1.3 | o | Rel-8 | pc_EarlyMedia |  |
| 46 | UE indicates g.3gpp.accesstype media feature tag in REGISTER | 24.237 [77], 6.2.2 | c1 | Rel-11 |  |  |
| 47 | Void |  |  |  |  |  |
| 48 | Void |  |  |  |  |  |
| 49 | The UE uses XCAP without PDN connection in WLAN | IR.51 [84], 4.6 | o | Rel-11 | pc_WLAN_XCAP_without_PDN | If PICS is true, Wi-Fi access without PDN connection applies. Otherwise, EPC-integrated WLAN applies |
| 50 | The UE uses XCAP in fixed broadband access |  | o | Rel-12 | pc_fixed_broadband_XCAP | In the context of the present specification, a UE supporting fixed broadband, A.18/3, shall set this PICS to true. |
| 51 | The UE uses same public user Identity for From/To as in initial REGISTER | 24.229 [10] | o | Rel-8 | pc_Same_IMPU |  |
|  |  | IR.92 [83], 2.2.1 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 52 | UE reattempts IMS emergency service without registration after rejection of emergency registration. (For Rel-14, this is applicable only when the rejection indicates an anonymous emergency call support) | 23.167 [76], 4.1 | o | Rel-9 | pc_IMS_after_EmReg_rejection |  |
|  |  |  | m | Rel-14 |  |  |
| 53 | Void |  |  |  |  |  |
| 54 | UE supports IMS eCall type of emergency services over EPS | 24.229 [10], 5.1.6.11, L.2.2.6.2 | o | Rel-14 | pc_EPS_IMS_eCall_Capable |  |
| 55 | UE supports IMS eCall Only type of emergency services over EPS | 24.229 [10], 5.1.6.11, L.2.2.6.2 | o | Rel-14 | pc_EPS_IMS_eCall_Only |  |
| 56 | UE supports audio media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | m | Rel-13 | pc_IMS_Audio_FeatureTag |  |
|  |  | IR.92 [83], 2.2.1, 2.2.4 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 57 | UE supports Session Timer | 24.229 [10], 5.1.2A, 5.2.7.2, 5.2.7.3, 5.4.5.3 | o | Rel-6 | pc_IMS_Session_Timer |  |
|  |  | IR.92 [83], 2.2.8 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 58 | UE supports enabling/disabling SMS over IMS | 27.007 [97], 8.72 | o | Rel-15 | pc_Enable_Disable_SMSoverIMS |  |
| 59 | UE supports disabling preconditions | 24.229 [10], 5.1.5A | o | Rel-15 | pc_IMS_Disable_Preconditions |  |
| 60 | UE uses Session Timer for its benefit alone | 24.229 [10] | o | Rel-15 | pc_IMS_Session_Timer_own_benefit |  |
|  |  | NG.114 [99] | o |  |  |  |
|  |  | RFC 4028 [86] cl 7.2 | o |  |  |  |
| 61 | UE supports usage of re-INVITE for session refresh | 24.229 [10] | o | Rel-15 | pc_IMS_Session_Timer_reINVITE |  |
|  |  | NG.114 [99] | o |  |  |  |
|  |  | RFC 4028 [86] cl 7.2 | o |  |  |  |
| 62 | Void |  |  |  |  |  |
| 63 | UE supports IMS eCall type of emergency services over 5GS | 24.229 [10], 5.1.6.11, U.2.2.6.2 | o | Rel-16 | pc_5GS_IMS_eCall_Capable |  |
| 64 | UE supports IMS eCall Only type of emergency services over 5GS | 24.229 [10], 5.1.6.11, U.2.2.6.2 | o | Rel-16 | pc_5GS_IMS_eCall_Only |  |
| 65 | UE supports IMS emergency call over WLAN | 24.229 [10], 5.1.6, R.2.2.6 | o | Rel-13 | pc_WLAN_IMS_EmergencyCall |  |
| 66 | UE supports P-CSCF_RESELECTION_SUPPORT IKEv2 attribute (in untrusted non-3GPP access network) | 24.229 [10], 5.1.6, R.2.2.1C | o | Rel-13 | pc_P_CSCF_Reselection_Support_IKEv2 |  |
| Conditions/Options |  |  |  |  |  |  |
| c1: IF [73] A.4.4-1/32 THEN m ELSE o - - SC UE indicates accesstype. |  |  |  |  |  |  |
| NOTE 1: As an exception to NG.114 support of MTSI Video Call is optional. |  |  |  |  |  |  |

### A.4.6	Additional information for IPv4

Table A.13: Additional information for IPv4

| Precondition: This table is only applicable if A.7/1 IPv4 is supported |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Additional information for IPv4 | Reference | Status | Release | Mnemonic | Comments |
| 1 | UE capable of being configured to initiate P-CSCF discovery via DHCPv4 | 23.981 [18], 5.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaDHCPv4 |  |
| 2 | UE supports P-CSCF discovery via DHCPv4 | 23.981 [18], 5.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaDHCPv4 |  |

### A.4.7	MTSI media

Table A.15: MTSI media

| Item | Media | Reference | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Speech | 26.114 [56], 5.2.1 | o | Rel-7 | pc_MTSI_Speech |  |
|  |  | IR.92 [83], 3.2 | m |  |  |  |
|  |  | IR 51 [84], 3 | m |  |  |  |
|  |  | NG.108 [87] | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 2 | Speech, AMR wideband | 26.114 [56], 5.2.1 | o | Rel-7 | pc_MTSI_Speech_AMRWB |  |
|  |  | IR.51 [84], 3 | m |  |  |  |
|  |  | IR.92 [83], 3.2 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 3 | Video | 26.114 [56], 5.2.2 | o | Rel-7 | pc_MTSI_Video |  |
|  |  | IR.94 [75], 3.3 | m |  |  |  |
|  |  | IR.51 [84], 3 | m |  |  |  |
|  |  | NG.114 [99] | m NOTE 1 | Rel-15 |  |  |
| 4 | Video, H.263 Profile 3 | 26.114 [56], 5.2.2 | o | Rel-7 |  |  |
| 5 | Video, MPEG-4 | 26.114 [56], 5.2.2 | o | Rel-7 |  |  |
| 6 | Video, H.264 | 26.114 [56], 5.2.2 | o | Rel-7 |  |  |
| 7 | Void |  |  |  |  |  |
| 8 | Void |  |  |  |  |  |
| 9 | Video codec H.264 CBP Level 1.2 | 26.114 [56], 5.2.2 | o | Rel-8 | pc_VideoCodecH264CBP |  |
|  |  | IR.94 [75], 3.3 | m |  |  |  |
|  |  | IR.51 [84], 3 | m |  |  |  |
| 10 | Speech, EVS | 26.114 [56] | o | Rel-12 | pc_MTSI_Speech_EVS |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 11 | Video codec H.265 (HEVC), Main Profile, Main Tier, Level 3.1 | 26.114 [56], 5.2.2 | c1 | Rel-15 | pc_VideoCodecH265Lv31 |  |
|  |  | NG.114 [99] | m NOTE 1 | Rel-15 |  |  |
| 12 | Video codec H.264 (AVC) CHP Level 3.1 | 26.114 [56], 5.2.2 | o | Rel-15 | pc_VideoCodecH264CHPLv31 |  |
|  |  | NG.114 [99] | m NOTE 1 | Rel-15 |  |  |
| 13 | Video codec H.264 (AVC) CBP Level 3.1 | 26.114 [56], 5.2.2 | o | Rel-15 | pc_VideoCodecH264CBPLv31 |  |
|  |  | NG.114 [99] | m NOTE 1 | Rel-15 |  |  |
| NOTE 1: As an exception to NG.114 support of MTSI Video Call is optional. c1: IF NOT A.15/3 OR [93] |  |  |  |  |  |  |

### A.4.8	MTSI supplementary services

Table A.16: MTSI supplementary services

| Item | Service | Reference | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Originating Identification Presentation | 24.173 [55], Annex A | o | Rel-7 | pc_MTSI_OIP |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 2 | Originating Identification Restriction | 24.173 [55], Annex A | o | Rel-7 | pc_MTSI_OIR |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.108 [87] | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 2A | Originating Identification Restriction - Configuration | 24.173 [55], Annex A | o | Rel-7 | pc_MTSI_OIR_C |  |
|  |  | IR.92 [83], 2.3 | o |  |  |  |
|  |  | NG.114 [99] | o | Rel-15 |  |  |
| 3 | Terminating Identification Presentation | 24.173 [55], Annex B | o | Rel-7 | pc_MTSI_TIP |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.114 [99] | o | Rel-15 |  |  |
| 4 | Terminating Identification Restriction | 24.173 [55], Annex B | o | Rel-7 | pc_MTSI_TIR |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.108 [87] | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 4A | Terminating Identification Restriction - Configuration | 24.173 [55], Annex A | o | Rel-7 | pc_MTSI_TIR_C |  |
|  |  | IR.92 [83], 2.3 | o |  |  |  |
|  |  | NG.114 [99] | o | Rel-15 |  |  |
| 5 | Communication Diversion | 24.173 [55], Annex C | o | Rel-7 | pc_MTSI_CommDivert |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.108 [87] | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 6 | Communication Hold | 24.173 [55], Annex D | o | Rel-7 | pc_MTSI_CommHold |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 7 | Incoming Communication Barring | 24.173 [55], Annex E | o | Rel-7 | pc_MTSI_Incoming_CB |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 8 | Message Waiting Indication | 24.173 [55], Annex F | o | Rel-7 | pc_MTSI_MessageWait |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.108 [87] | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 9 | Conference | 24.173 [55], Annex G | o | Rel-7 | pc_MTSI_Conference |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 10 | Void |  |  |  |  |  |
| 11 | Explicit Communication Transfer - consultative transfer | 24.173 [55], Annex H | o | Rel-7 | pc_IMS_ExplicitConsultativeTransfer |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 12 | Anonymous Communication Rejection | 24.173 [55], Annex E | o | Rel-7 | pc_MTSI_ACR |  |
| 13 | Communication Waiting | 24.615 [69] | o | Rel-7 | pc_MTSI_CommWaiting |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.108 [87] | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 14 | Three way session | 24.147 [19] | o | Rel-8 | pc_IMS_TWS |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 15 | Outgoing Communication Barring | 24.173 [55], Annex E | o | Rel-7 | pc_MTSI_Outgoing_CB |  |
|  |  | IR.92 [83], 2.3 | m |  |  |  |
|  |  | IR.51 [84], 2.3 | m |  |  |  |
|  |  | NG.114 [99] | m | Rel-15 |  |  |
| 16 | Void |  |  |  |  |  |
| 17 | Void |  |  |  |  |  |
| 18 | SIP based user configuration | 24.238 [85], 432 | o | Rel-7 | pc_SIP_Based_UserConfiguration |  |
| 19 | USSI | 24.390 [90] | o | Rel-12 | pc_USSI |  |
|  |  | IR.92 [83], Annex D | o |  |  |  |
|  |  | NG.114 [99] | o | Rel-15 |  |  |

<End of modified section>